NEOGEO    OCKET

##### 8 BIT TIMER REFERENCE MANUAL

- SNK CORPORATION

- 1998.10.26  rel. 0.2

- TABLE OF CONTENTS

8 BIT TIMER OUTLINE	3

1.  PRESCALER	5

2.  UP COUNTER	6

3.  TIMER REGISTER	6

4.  COMPARATOR	8

5.  TIMER FLIP FLOP (TIMER F/F)	8

CONTROL REGISTERS	9

T01MOD (Timer 0, 1 Mode Register)	9

T23MOD (Timer 2, 3 Mode Register)	10

TFFCFR (8 BIT TIMER FLIP FLOP CONTROL REGISTER	11

TRUN (TIMER OPERATION CONTROL REGISTER)	12

TRDC (TIMER REGISTER DOUBLE BUFFER CONTROL REGISTER)	13

8 BIT TIMER SAMPLE SETTING	14

H-int Setting	14

REVISION HISTORY	15

# 8 BIT TIMER OUTLINE

The NEOGEO POCKET has four internal 8 bit timers (Timer 0, 1, 2 and 3).

The four 8 bit timers may be used separately, or two can be cascade connected to be used as two 16 bit timers.  The 8 bit timer has four different modes which are listed below.

 8 bit interval timer mode (4 total)	may be combined as

 16 bit interval timer mode (2 total)  (two 8 bit and one 16 bit)

 8 bit programmable (PRG: variable cycle and duty) square wave output mode (2 total)

 8 bit PWM (pulse wavelength modifier: fixed cycle variable duty) output mode (2 total)

Figure 1 shows a block diagram of 8 bit timers (timer 0 and 1).

Timer 2 and 3 pair has circuitry similar to timer 0 and 1 pair.  The only difference is that timer 0 has an external clock TI0 terminal.  Timer 2 does not have an external clock.  In the NEOGEO POCKET, the horizontal blanking interrupt generated by the 2D graphic controller is the external clock TIO.

Each interval timer has an 8 bit up counter, an 8 bit comparator and an 8 bit timer register.  Each pair of timers (0, 1 and 2, 3) has one timer flip flop (TFF1and TFF3 respectively).

Of the input clock source for each of the interval timers, the internal clock T1, T4, T16 and T256 is obtained from the 9 bit prescaler as shown in Figure 2.

The operation mode and timer flip flop of the 8 bit timer is controlled by the five control register (T01MOD, T23MOD, TFFCR, TRUN and TRDC).

Figure 1.  8 Bit Timer Block Figure (Timer 0,1)

## 1.  PRESCALER

The 9 bit prescaler takes a quarter of the CPU clock (fc/4, where fc = CPU clock)1 and scales the signal to be used as input clocks into the 8 bit timer, baud generator, etc.

The 8 bit timer uses 4 input clocks:  T1, T4, T16 and T256.

The prescaler can be put into count or stop mode with the use of the timer run control register TRUN <PRUN>.  <PRRUN>=1 starts the count,  <PRRUN>=0 zero clears and stops the prescaler.2  When the system resets, <PRRUN> is “0” cleared.  In other words, the prescaler is cleared and stopped.

CPU clock (fc) given to the NEOGEO POCKET is fixed at 384 kHz.

Because the serial communication may affected negatively, stopping the prescaler is prohibited.

Cycle

| fc input clock | 384 kHz |
| --- | --- |
| T1 (8/fc) T4 (32/fc) T16 (128/fc) T256 (2048/fc) | 20.83 s 83.33 s 333.3 s 5.333 ms |

0  1  2  3  4  5  6  7  8

9 bit prescaler

Generator

Circuit

1/4

1/2

2

1

T0

T1

T2

T4

T8

T16

T32

T256

TRUN<PRRUN>

run/stop & clear

fc

CLK

４/fc

T1

T4

Figure 2 Prescaler

Please refer to Figure 1 and Control Register for the following sections.

## 2.  UP COUNTER

The up counter is an 8 bit binary counter which counts up with the input clock specified by the mode register T01MOD for timer 0, 1 and mode register T23MOD for timer 2, 3.

The parameter set in T01MOD register determines the input for timer 0 from internal clock T1, T4, T16 or external clock TIO.  The parameter set in T23MOD register determines the input for timer 2 from the internal clock T1, T4 or T16.

Input clock for timer 1 and timer 3 are determined by the operation mode.  If the operation mode is set to 16 bit timer mode, the overflow output from timer 0 for timer 1 and overflow output from timer 2 for timer 3 becomes the input clock.  If the operation mode is set to other than 16 bit timer mode, the values set in register T01MOD and T23MOD determines the input clock for timer 1 and 3 from the internal clock T1, T16, T256 or output from the comparator (unison check).

Ex:	If T01MOD<T01M1,0> = 01 (i.e. if T01M1 and T01M0 in T01MOD as shown in the control register table for T01MOD is set to 0 and 1), then the input clock for timer 1 is the overflow output from the timer 0 (16 bit timer operation mode).  If T01MOD7,6 = 00 (i.e. T01MOD<T01M1,0> = 00; or T01M1 = 0 and T01M0 = 0) and T01MOD3,2 = 01 (i.e. T01MOD<T01CLK1,0> = 01; or T1CLK1 = 0 and T1CLK0 = 1); then the input clock for timer 1 is T1 (8 bit timer).

Operation mode is set in T01MOD register and T23MOD register.  After system reset, the registers are initialized to T01MOD<T01M1,0> = 00 and T23MOD<T23M1,0> = 00, 8 bit timer mode.

At each interval timer, it is possible to control the count/stop & clear of the up counter with the use of timer control register TRUN.  After system reset, all up counters are cleared and the timers are stopped.

## 3.  TIMER REGISTER

This is the 8 bit register to set the interval time.  When the values of timer registers TREG0, 1, 2, 3 match the value of the up counter, the comparator unison check becomes active.  If the value is set to 00H, unison check becomes active when the up counter is in overflow status.

This timer register TREG0/TREG2 has a double buffer structure, and each is paired with the register buffer.

<TR0DE,TR2DE> of the timer register double buffer control register TRDC sets the TREG0/TREG2 to be double buffer enabled or disabled.  If <TR0DE>/<TR2DE> = 0, it is disabled; and if <TR0DE>/<TR2DE> = 1, it is enabled.

When double buffer is enabled, the data transfer from register buffer to timer register occurs when 2n-1 mode of PWM overflows or cycle unison of PPG mode is detected.

After reset, <TR0DE>/<TR2DE> = 0 and double buffer is disabled.  If double buffer is to be used, write data to timer register, next set <TR0DE>/<TR2DE> = 1 and then write the next data to the register buffer.

Comparator

Timer Register 0/2

Register Buffer 0/2

Up Counter

Internal Bus

PRG cycle unison check

PWM 2n-1 overflow

TREG 0/2 WR

<TR0DE>/<TR2DE>

S

E

L

E

C

T

O

R

Shift Trigger

write

Figure 3  Timer Register 0/2 Structure

NOTE:	Timer register and register buffer has the same memory address block.

When <TR0DE>/<TR2DE> = 0, same value is written into register buffer and timer register.  When <TR0DE>/<TR2DE> = 1, value is written only into register buffer.

Memory address for each time register is shown below:

| TREG0 | : | 000022H |
| --- | --- | --- |
| TREG1 | : | 000023H |
| TREG2 | : | 000026H |
| TREG3 | : | 000027H |

Each register is write only and cannot be read.  Because it is not possible to initialize values of the registers that are not constants, please write in data before using 8 bit timers.

## 4.  COMPARATOR

The comparator compares the up counter value and timer register value.  If it is in unison (the same), the up counter is 0 cleared and an interrupt (INTT0~3) is generated.  Also, if timer flip flop reversal is enabled, the timer flip flop value is flipped.

## 5.  TIMER FLIP FLOP (TIMER F/F)

At each interval timer, flip flop according to the unison (matching comparison) check signal (comparator output).  The resulting values may be outputted to timer output terminal T01 and T03*3.

This timer F/F is called TFF1 and TFF3, each for the timer pair timer 0,1 and timer 2,3 respectively.  TFF1 outputs to T01 terminal and TFF3 outputs to T03 terminal.

*3  T03 is used as an interrupt to the Z80 CPU.

# CONTROL REGISTERS

### T01MOD (Timer 0, 1 Mode Register)

|  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bit Symbol T01MOD (0024H) | T01M1 | T01M0 | PWM01 | PWM00 | T1CLK1 | T1CLK0 | T0CLK1 |  | T0CLK0 |
| Read/Write | R/W |  | R/W |  | R/W |  | R/W |  |  |
| After reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| Function | 00 : 8bit Timer 01 : 16bit Timer 10 : 8bit PPG 11 : 8bit PWM |  | 00 : - 01 : 26 - 1 PWM 10 : 27 - 1 cycle 11 : 28 - 1 |  | 00 : TO0TRG 01 : T1 10 : T16 11 : T256 |  | 00 : TI0 01 : T1 10 : T4 11 : T16 |  |  |

Input clock for timer 0

| 00 | External clock TI0 |
| --- | --- |
| 01 | Internal clock T1 |
| 10 | Internal clock T4 |
| 11 | Internal clock T16 |

Input clock for timer 1

|  | T01D7,6  01 | T01D7,6 = 01 |
| --- | --- | --- |
| 00 | Comparator output for timer 0 | Timer 0 overflow output (16 bit timer mode) |
| 01 | Internal clock T1 |  |
| 10 | Internal clock T16 |  |
| 11 | Internal clock T256 |  |

Select PWM0 cycle

(Other than PWM mode Don’t Care)

| 00 | -- |
| --- | --- |
| 01 | 26- 1 |
| 10 | 27- 1 |
| 11 | 28- 1 |

Timer 0, 1 operation mode setting

| 00 | Two 8 bit timer (timer 0, 1) |
| --- | --- |
| 01 | 16 bit timer |
| 10 | 8 bit programmable square wave output |
| 11 | 8 bit PWM output (timer 0) + 8 bit timer (timer 1) |

### T23MOD (Timer 2, 3 Mode Register)

|  | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bit Symbol T23MOD (0028H) | T23M1 | T23M0 | PWM21 | PWM20 | T3CLK1 | T3CLK0 | T2CLK1 | T2CLK0 |
| Read/Write | R/W |  | R/W |  | R/W |  | R/W |  |
| After reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Function | 00 : 8bit Timer 01 : 16bit Timer 10 : 8bit PPG 11 : 8bit PWM |  | 00 : - 01 : 26 - 1 PWM 10 : 27 - 1 cycle 11 : 28 - 1 |  | 00 : TO2TRG 01 : T1 10 : T16 11 : T256 |  | 00 : - 01 : T1 10 : T4 11 : T16 |  |

Input clock for timer 2

| 00 | -- |
| --- | --- |
| 01 | Internal clock T1 |
| 10 | Internal clock T4 |
| 11 | Internal clock T16 |

Input clock for timer 3

|  | T23MOD7,6  01 | T23MOD7,6 = 01 |
| --- | --- | --- |
| 00 | Comparator output for timer 2 | Overflow output for timer 2 (16 bit timer mode) |
| 01 | Internal clock T1 |  |
| 10 | Internal clock T16 |  |
| 11 | Internal clock T256 |  |

Select cycle for PWM2

(Other than PWM mode, Don’t Care)

| 00 | -- |
| --- | --- |
| 01 | 26 - 1 |
| 10 | 27 - 1 |
| 11 | 28 - 1 |

Timer 2, 3 operation mode setting

| 00 | Two 8 bit timer (timer 2, 3) |
| --- | --- |
| 01 | 16 bit timer |
| 10 | 8 bit programmable square wave output |
| 11 | 8 bit PWM output (timer 2) + 8 bit timer (timer 3) |

### TFFCFR (8 BIT TIMER FLIP FLOP CONTROL REGISTER

TFF3			            TFF1

TFFCR

(0025H)

|  | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bit Symbol | FF3C1 | FF3C0 | FF3IE | FF3IS | FF1C1 | FF1C0 | FF1IE | FF1IS |
| Read/Write | W |  | R/W |  | W |  | R/W |  |
| After reset | - |  | 0 | 0 | - |  | 0 | 0 |
| function | 00 : Invert TFF3 01 : Set TFF3 10 : Clear TFF3 11 : Don’t Care |  | 1 : TFF3 Invert Enable | 0 : flipped by 8 bit timer 2 1 : flipped by timer 3 | 00 : Invert TFF1 01 : Set TFF1 10 : Clear TFF1 11 : Don’t Care |  | 1 : TFF1 Invert Enable | 0 : flipped by 8 bit timer 0 1 : flipped by timer 1 |

Timer flip flop TFF1 flip signal

selection (Not 8 bit timer, Don’t Care)

| 0 | Flip by timer 0 |
| --- | --- |
| 1 | Flip by timer 1 |

Timer flip flop TFF1 flip

| 0 | Disable |
| --- | --- |
| 1 | Enable |

Timer flip flop TFF1 control

| 00 | Flip the value of TFF3. (Software flip) |
| --- | --- |
| 01 | Sets TFF1 to”1”. |
| 10 | Sets TFF1 to”0”. |
| 11 | Don’t Care (The value will always be “11” when read) |

Timer flip flop TFF3 flip signal

selection (Not 8 bit timer, Don’t Care)

| 0 | Flip by timer 2 |
| --- | --- |
| 1 | Flip by timer 3 |

Timer flip flop TFF3 flip

| 0 | Disable |
| --- | --- |
| 1 | Enable |

Timer flip flop TFF3 control

| 00 | Flip the value of TFF3. (Software flip) |
| --- | --- |
| 01 | Sets TFF3 to”1”. |
| 10 | Sets TFF3 to”0”. |
| 11 | Don’t Care (The value will always be “11” when read) |

### TRUN (TIMER OPERATION CONTROL REGISTER)

8 bit timer

TRUN

(0020H)

|  | 7 | 6 |  | 5 |  | 4 |  | 3 |  | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bit Symbol | PRRUN |  |  |  |  |  |  | T3RUN |  | T2RUN | T1RUN | T0RUN |
| Read/Write | R/W |  |  |  |  |  |  |  | R/W |  |  |  |
| After reset | 0 |  |  |  |  |  |  | 0 |  | 0 | 0 | 0 |
| Function | Prescaler & Timer Run / Stop Control 0 : Stop & Clear 1 : Run (Count up) |  |  |  |  |  |  |  |  |  |  |  |

Timer 0 operation

| 0 | Stop & Clear |
| --- | --- |
| 1 | Count |

Timer 1 operation

| 0 | Stop & Clear |
| --- | --- |
| 1 | Count |

Timer 2 operation

| 0 | Stop & Clear |
| --- | --- |
| 1 | Count |

Timer 3 operation

| 0 | Stop & Clear |
| --- | --- |
| 1 | Count |

Prescaler operation

| 0 | Stop & Clear |
| --- | --- |
| 1 | Count |

NOTE:  To avoid interruption to serial communication, it is PROHIBITED to stop the prescaler.

### TRDC (TIMER REGISTER DOUBLE BUFFER CONTROL REGISTER)

|  | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TRDC (0029H) bit Symbol |  |  |  |  |  |  | TR2DE | TR0DE |  |
| Read/Write |  |  |  |  |  |  | R/W |  |  |
| After reset |  |  |  |  |  |  | 0 |  | 0 |
| Function |  |  |  |  |  |  | 0: Double Buffer Disable 1: Double Buffer Enable |  |  |

Timer register 0 double

buffer control

| 0 | Disable |
| --- | --- |
| 1 | Enable |

Timer register 2 double

buffer control

| 0 | Disable |
| --- | --- |
| 1 | Enable |

# 8 BIT TIMER SAMPLE SETTING

### H-int Setting

The 8 bit timer must be utilized in order to use the H-int with the NEOGEO POCKET.  An example is shown below.  H-int is set by using timer 0 in 8 bit timer mode.  The example below generates H-int every line.  If it is to be generated every 4 lines, please set the interval time TREG0 to 0x04.

andb	(TRUN),0y10001110	;stop count for timer 0

ldb	(T01MOD),0x00		;timer 0,1 operation mode <8 bit timer mode>

;timer 0 input clock<external clock TI0>

ldb	(TREG0),0x01		;interval tine is 0x01

ldb	rw3,VECT_INTLVSET	;system call used

ldb	rb3,0x03		;(please refer to SYSTEM CALL REF. MAN.)

ldb	rc3,0x02			;

swi	1			;setting 8 bit timer 0 interrupt level

orb	(TRUN),0y00000001	;start count for timer 0

# REVISION HISTORY

| rel 0.1 | Initial release | 1998/07/15 |
| --- | --- | --- |
| rel 0.2 | 8 bit timer outline, prescaler cycle table corrected | 1998/10/26 |