|  | K1 SOUND SIMULATION BOARD HARDWARE REFERENCE MANUAL |  |
| --- | --- | --- |

1998 07 03

Ver1.00

| Text No. |  |  |  |
| --- | --- | --- | --- |
| Issued | SNK Corporation Hardware Dept. Hardware Division |  |  |
|  |  |  | In Charge |

Name : K1 Sound Simulation Board

Model : K1-SOUND_A

Revision History

| Date | Ver | Revision | Requested | In Charge |
| --- | --- | --- | --- | --- |
| 1998/04/21 1998/0７/03 | Ver 0.1 Ver 1.00 | Provisional Issue |  | Sakai Honda |

TABLE OF CONTENTS

1.	SPECIFICATION	4

2.	BLOCK DIAGRAM	5

3.	ISA-BUS INTERFACE	6

3.1.	MEMORY SPACE	6

3.2.	MEMORY MAP (I/O MAP)	7

3.3.	MEMORY	8

3.3.1.	Z80 RAM	8

3.3.2.	Memory for Sound D/A	8

3.4.	I/O	8

3.4.1.	DAC (L/R) Control Register	8

3.4.2.	C061Compatible Registers	9

3.4.2.1	TRUN Register	10

3.4.2.2	TTFFCR Register	10

3.4.2.3	T23MOD Register	11

3.4.2.4	Clock Gear Control Register	11

3.4.3.	Z80 Sound Source Control CPU	12

3.4.3.1.	Z80 Reset, Sound Source Reset Control Register	13

3.4.3.2.	Z80 NMI Control Register	13

3.4.3.3.	PC – Z80 Communication Register	13

3.4.4.	IRQ Setting Register	14

3.4.5.	IRQ Status Register	14

4.	SOUND SOURCE	15

4.1.	SOUND SOURCE SPECIFICATION	16

4.2.	OPERATION	17

4.2.1.	Tone Generator	17

4.2.2.	Noise Generator	17

4.2.3.	Interface with PC and Z80	18

4.2.3.1.	Control Register	18

4.2.3.2.	Data Format	19

4.2.4.	Output Frequency Calculation	19

5.	Z80 CPU	22

5.1.	Z80 SOUND SOURCE CONTROL CPU ADDRESS MAP	22

5.2.	I/O	22

5.2.1.	Z80 Timer Interrupt	22

5.2.2.	PC INT Control Register	23

5.2.3.	PC – Z80 Communication Register	23

5.2.4.	Z80 Interrupt Acknowledge Register	23

6.	ADDITIONAL MATERIAL	24

6.1.	SOUND ELEMENTS AND T6W28 RELATIONSHIP	24

6.1.1.	Sound Elements	24

6.1.2.	Sound Element and T6W28 Relationship	24

6.2.	D/A CONVERTER SPECIFICATION	25

6.2.1.	Memory for D/A	25

BOARD FIGURE	26

# SPECIFICATION

This board has the same functionality as the K1 Sound.  It is possible to insert the board into an ISA bus slot to simulate sounds produced by the K1 Sound hardware.

BUS-I/F

ISA-Bus

Sound CPU

Z80 Equivalent (3.072 MHz)

Common Memory

4 KB

DAC Conversion Table Memory

512 Byte (SRAM)

Sound IC

SN76489A Equivalent (FPGA)

12 bit D/A converter

Timer Counter

900H CPU CTC timer compatible

Interrupt

Port selection and prohibition by register

Panel Terminal

Output

Stereo earphone jack

Optional terminal

Optional power supply terminal (+5V)

Power Supply

Voltage:	+5V±5%. +12V±5%

Current:	1A Max.

# BLOCK DIAGRAM

# ISA-BUS INTERFACE

## MEMORY SPACE

DIP SWITCH SETUP

The 8 KB of memory used as on board internal memory and register is an 8 KB block designated from the 128 KB of the personal computer memory.  Please switch the BASE address with the DIP switch according to the development environment.  DIP switch settings are shown in the table below.  (Please refer to 3.4.4 IRQ Designation Register for IRQ settings.)

| DIPSW |  |  |  |  |  |  | PC Address |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 3 |  | 2 |  | 1 |  |
| Off | Off |  | Off |  | Off |  | C0000h ~ C1FFFh (Initial setting) |
| Off | Off |  | Off |  | ON |  | C2000h ~ C3FFFh |
| Off | Off |  | ON |  | Off |  | C4000h ~ C5FFFh |
| Off | Off |  | ON |  | ON |  | C6000h ~ C7FFFh |
| Off | ON |  | Off |  | Off |  | C8000h ~ C9FFFh |
| Off | ON |  | Off |  | ON |  | CA000h ~ CBFFFh |
| Off | ON |  | ON |  | Off |  | CC000h ~ CDFFFh |
| Off | ON |  | ON |  | ON |  | CE000h ~ CFFFFh |
| ON | Off |  | Off |  | Off |  | D0000h ~ D1FFFh |
| ON | Off |  | Off |  | ON |  | D2000h ~ D3FFFh |
| ON | Off |  | ON |  | Off |  | D4000h ~ D5FFFh |
| ON | Off |  | ON |  | ON |  | D6000h ~ D7FFFh |
| ON | ON |  | Off |  | Off |  | D8000h ~ D9FFFh |
| ON | ON |  | Off |  | ON |  | DA000h ~ DBFFFh |
| ON | ON |  | ON |  | Off |  | DC000h ~ DDFFFh |
| ON | ON |  | ON |  | ON |  | DE000h ~ DFFFFh |

* DIP switches are ON if the key lever is down.  The numbering of the DIP switches are the same as the numbers on the board.

## MEMORY MAP (I/O MAP)

The addresses shown below are based on the BASE address set earlier.

BASE address + 000h

1000h

1200h

1800h

1880h

1890h

18A0h

18A1h

18A2h

18A3h

18A4h

18B8h

18BAh

18BCh

18BDh

18BEh

18C1h

1FFFh

18C0h

Clock Gear Control Register

(Do Not Access)

Sound Source (Right) Control Register

Sound Source (Left) Control Register

DAC(Right) Control Register

DAC(Left) Control Register

(Do Not Access)

Z80, Sound Reset Control Register

Z80 NMI Control Register

PC – Z80 Communication Register

Z80 RAM (4KB)

D/A Memory (512 Byte)

(Don’t care)

C061 Compatible Registers

(Don’t care)

IRQ Setting Register

(Don’t care)

IRQ Status Register

## MEMORY

### Z80 RAM

Z80 RAM may be accessed from both the PC and the Z80 CPU.  4 KB of memory is accessed by the Z80 CPU as regular Z80 memory.  When the Z80 RAM area is accessed by the PC, a bus request is generated to the Z80 CPU and the Z80 CPU returns the bus acknowledge signal.  The PC is in WAIT mode until this bus acknowledge signal is returned.  If the PC generates a reset to the Z80 CPU, the Z80 CPU is reset and the Z80 RAM is released to the PC.  When the Z80 CPU is in reset mode, the bus request cannot be relied on as a valid signal so care must be taken.

### Memory for Sound D/A

The Sound D/A memory allows the PC to access the 12 Bit D/A converter.  The memory has been modified for the K1 data and SRAM is used.  It is necessary to initialize the data in the memory since data after power ON is not reliable.  The data format follows.

| Address |  | Bit |  | 15 | 14 | 13 | 12 | 11 |  | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BASE + 1000 h ~ 11FFh |  |  | Initial Value > | - | - | - | - | - |  | - | - | - | - | - | - | - | - | - | - | - |  |
| Access = Read/Write 8/16 Bit |  |  |  | Invalid |  |  |  | MSB | 12 Bit Accuracy DAC data |  |  |  |  |  |  |  |  |  |  |  | LSB |

## I/O

### DAC (L/R) Control Register

DAC (L/R) control register is used to directly access the DAC from the PC in order to specify Sound D/A memory address.  This register is used after data (to be converted) has been written to the (Sound D/A) memory.  The memory is accessed by writing in the following format to the DAC control register.  Approximately 15 s is taken for the write to this register until the DAC outputs the correct analog value.

DAC write cycle starts after a write in the DAC (L/R) Control Register.

If word accessed, write is initiated on both sides of L/R DAC.

Format is as follows.

| Address |  | Bit |  | 15 | 14 | 13 | 12 | 11 | 10 | 9 |  | 8 | 7 |  | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BASE + 18A2h ~ 18A3h |  |  | Initial Value > | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Access = Read/Write 8/16 Bit |  |  |  | MSB | DAC-Left |  |  |  |  |  | LSB |  | MSB | DAC-Right |  |  |  |  |  |  | LSB |
| Address to SRAM |  |  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 |  | 1 | 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
|  |  |  |  | Adr |  |  |  |  |  |  |  |  | Adr |  |  |  |  |  |  |  |  |

### C061Compatible Registers

C061 compatible registers are compatible to the 900H CPU CTC timer, and include two 8 bit timers.  The Sound Simulation board only supports 8 bit timer mode and does not support the following output mode.

- 16 bit timer mode (cascading 8 bit timers)

- 16 bit interval timer mode (cascading 8 bit timers)

- 8 bit programmable square wave (PPG: changeable cycle and duty)

- 8 bit PWM (Pulse wavelength modification:  fixed cycle and changeable duty)

The circuit is composed of prescalar, upcounter, timer register, comparator, and timer flip flop for the timer clock.  These circuits are controlled by three control registers (TRUN, TFFCR, T23MOD).  Explanations follow.

- Prescalar

The prescalar is 9 bits, and takes the clock (fc) from the clock generator and divides by 4, and further divides the clock (fc/4) to create input clocks (T1, T4, T16, T256) for the upcounter.  The count begins/stops with the timer control register TRUN’s <PRRUN> bit being set.  After reset, it is zero cleared.

|  | (If fc = 6.144 MHz) |
| --- | --- |
| T1(8/fc) | 1.302 s |
| T4(32/fc) | 5.208 s |
| T16(128/fc) | 20.833 s |
| T256(2048/fc) | 333.333 s |

- Upcounter

This upcounter consists of an 8 bit binary counter and counts up with the input specified by the timer 2,3 mode register T23MOD.  Timer 2 input clock may be chosen from T1, T4, T16 and external input TIO (384 kHz).  Timer 3 input may be chosen from, T1, T16, T256 and output from timer 2 comparator output (equivalence check).  Control of these timers are done through timer operation control register TRUN.  Timer count start/stop and clear may be controlled every interval, and at reset, the counter is cleared and stopped.

- Timer Register

Timer register TREG2,3 is an 8 bit register to set interval time.  If the value set in this register and the upcounter match, comparator equality signal becomes active.  If value 00H is set, the signal is active when the upcounter overflows.  The registers are write only, and because the initial value is not reliable, please write in initial values.

TREG2:  BASE+1826h  TREG3:  BASE+1827h

- Comparator

Comparator compares the values from the upcounter and the timer register, and if the values are equivalent, the upcounter is zero cleared and generates interrupts (INT2,3).  If the flip/flop bit of timer 3 TFFCR register is enabled, the timer flip/flop is flipped.

- Timer Flip Flop (Timer F/F)

Timer flip-flop is a flip/flop function activated when equality is detected (comparator output) at each interval.  The signal is outputted to timer output terminal TO3.

#### TRUN Register

| Address | Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BASE+1820h | Name | PRRUN | ---- | ---- | ---- | T3RUN | T2RUN | ---- | ---- |
| Read/Write |  | R/W |  |  |  | R/W | R/W |  |  |
| Initial Value |  | 0 |  |  |  | 0 | 0 |  |  |
| Prescalar Operation 0: Stop and Clear 1: Count Start Timer 3 Operation 0: Stop and Clear 1: Count Start Timer 2 Operation 0: Stop and Clear 1: Count Start |  | Prescaler & Timer Run/Stop control 0:Stop & Clear 1:Run(Count Up) |  |  |  |  |  |  |  |

#### TTFFCR Register

| Address | Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BASE+1825h | Name | FF3C1 | FF3C0 | FF3IE | FF3IS | ----- | ----- | ---- | ---- |
| Read/Write |  | W |  | R/W |  |  |  |  |  |
| Initial Value |  | ----- |  | 0 | 0 |  |  |  |  |
|  |  | 00:Invert TFF3 01:Set TFF3 10:Clear TFF3 11:don’t care |  | 1: TFF3 Invert Enable | 0: flip by timer 2 1: flip by timer 3 |  |  |  |  |

Timer Flip Flop TFF3 Flip/Flop

0: Disable

1: Enable

Timer Flip Flop TFF3 Flip/Flop Signal Selection

0: Flip/Flop by Timer 2

1: Flip/Flop by Timer 3

Timer Flip Flop TFF3 Control

00: Flip the value of TFF3 (Software flip)

01: Set TFF3 to “1”

10: Set TFF3 to “0”

11:don’t care (when read, “11”)

#### T23MOD Register

| Address | Bit | 7 | 6 | 5 | 4 | 3 |  | 2 | 1 |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BASE+1828h | Name | ----- | ----- | ---- | ---- | T3CLK |  | T3CLK0 | T2CLK1 |  | T2CLK0 |
| Read/Write |  | ---- |  |  |  | R/W |  |  | R/W |  |  |
| Initial Vlaue |  | ----- |  |  |  | 0 | 0 |  | 0 | 0 |  |
| Timer 2 Input Clock 00: External Input Clock TIO = 384 kHz 01: Internal Clock T1 10: Internal Clock T4 11: Internal Clock T16 Timer 3 Input Clock 00: Timer 2 Comparator Output 01: Internal Clock T1 10: Internal Clock T16 11: Internal Clock T256 |  |  |  |  |  | 00: TO2TRG 01: T1 10: T16 11: T256 |  |  | 00: TIO 01: T1 10: T4 11: T16 |  |  |

#### Clock Gear Control Register

The clock gear function is compatible to the SYSCR1 register of TMP93CS42A.

Interface speed with the Z80, and prescalar source frequency (fc) are selected.

| Address | Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BASE+1880h | Name | ----- |  |  |  |  | GEAR |  |  |
| Read/Write |  | ---- |  |  |  |  | R/W |  |  |
| Initial Value Selection of Clock (fc) Gear 000: fc (6.144MHz) 001: fc/2 010: fc/4 011: fc/8 100: fc/16 101: (reserved) 110: (reserved) 111: (reserved) |  | ----- |  |  |  |  | 1 | 0 | 0 |

### Z80 Sound Source Control CPU

Z80 sound source controller CPU is equivalent to the sound source T6W28 circuit controller sub CPU.  1 Byte dual port register is used to communicate with the PC.  Related registers and reset information is shown below.

Related Register List (PC ISA bus):

Z80 reset, sound source reset control register (BASE+18B8h, BASE+18B9h),

Z80 NMI control register (BASE+18BAh, BASE+18BBh),

PC-Z80 communication register (BASE+18BCh)

Related Register List (Z80-CPU):

PC-Z80 communication register (8000h)

PC INTx control register (C000h)

| System Reset: | ‘L’ = System Reset State |
| --- | --- |
| Oscillation Enabled 0: | ‘L’ = Oscillation Enable “0” |
| Command Reset from PC: | Reset state is reached with the PC sending the reset command to the sound source reset control register. Only Z80-PC is reset. |

Each of the registers are explained below.

#### Z80 Reset, Sound Source Reset Control Register

| Address | Bit | 15 | 14 | 13 | 12 | 11 |  | 10 | 9 |  | 8 | 7 |  | 6 | 5 | 4 | 3 | 2 | 1 |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BASE+18B8 h | Initial Value > | 1 | 0 | 1 | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 | 0 | 1 | 0 | 1 |  | 0 |
| Access = Write (Read possible) 8/16 Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Z80 Reset Register, T6W28 Reset Register Write Data |  |  |  |  | Z80 State |  |  | T6W28 State |  |  | PC Direct Access |  |  |  |  |  |  | Initial State |  |
|  |  | 5555 h |  |  |  |  | RUN |  |  | RUN |  |  | No Use |  |  |  |  |  |  |  |  |
|  |  | 55AA h |  |  |  |  | RUN |  |  | Reset |  |  | DAC Use Possible |  |  |  |  |  |  |  |  |
|  |  | AA55 h |  |  |  |  | Reset |  |  | RUN |  |  | Sound Source Use Possible |  |  |  |  |  |  |  |  |
|  |  | AAAA h |  |  |  |  | Reset |  |  | Reset |  |  | DAC Use Possible |  |  |  |  |  |  | O |  |

#### Z80 NMI Control Register

| Address | Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BASE+18BA h | Initial Value > | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| Access = Write only 8/16 Bit |  | NMI request generated once to the Z80 with write access (data invalid). (Pulse width larger than 100ns) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### PC – Z80 Communication Register

| Address | Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 |  | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BASE+18BCh | Initial Value > | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Access = Read/Write 8/16 Bit |  | Fixed “0” |  |  |  |  |  |  |  |  | PC – Z80 Communication Register (Dual Port) |  |  |  |  |  |  |  |

### IRQ Setting Register

IRQ setting register designates which IRQ of the ISA bus receives the interrupt output from the board.  After reset, the value is “0000” and the output is invalid.

| Address | Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BASE+18BEh | Name | ----- |  |  |  | IRQ |  |  |  |
| Read/Write |  | ---- |  |  |  | R/W |  |  |  |
| Initial Value IRQ Port Select 0,1,2,8,Dh: IRQ don't care 3h: IRQ3 4h: IRQ4 5h: IRQ5 6h: IRQ6 7h: IRQ7 9h: IRQ9 Ah: IRQ10 Bh: IRQ11 Ch: IRQ12 Eh: IRQ14 Fh: IRQ15 |  | ----- |  |  |  | 0 | 0 | 0 | 0 |

### IRQ Status Register

The PC needs to confirm the point from which the IRQ was produced by reading the status register when interrupts (IRQ) are generated by the board.  If the register is not read, the next request for an interrupt occurring will not output IRQ.  If another request for an interrupt occurs, during the time after the IRQ has been outputted and the PC reads the register, it is necessary to note this in the status register.  Care must be taken if the interrupt request is produced for the same reason because it will be ignored.

| Address |  | Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BASE+18C0h |  | Name | ----- | ----- | Z80INT |  | CTCINT3 |  | CTCINT2 |  |
| Read/Write |  |  | ---- |  | R/W | R | R/W | R | R/W | R |
| Initial Value |  |  | ----- |  | 1 | 0 | 1 | 0 | 1 | 0 |
|  | 1:Mask 1:int out |  |  |  |  |  |  |  |  |  |

INT Status

0: no int

1: interrupt request

INT Mask

0: interrupt output enable

1: interrupt output disable

# SOUND SOURCE

The Sound Source Module uses T6W28 (L, R 2 CH) equivalent circuitry for the sound source.  The D/A module is a 12 bit D/A converter.  There is a conversion table so that accessing the D/A from the PC will result in 12 bit  1 CH equivalent data.

It is possible for Z80-CPU or PC to access the D/A converter.  The PC can generate a command to the Z80 Reset, Sound Source Reset Control Register.  Resetting Z80-CPU will allow access to the Sound Source, and resetting the Sound Source will allow access to the D/A converter.

Related Registers:

L CH Sound Source Control Register (BASE+18A1h)

R CH Sound Source Control Register (BASE+18A0h)

L CH DAC Control Register (BASE+18A3h)

R CH DAC Control Register (BASE+18A2h)

Z80 Reset, Sound Source Reset Control Register (BASE+18B8h, BASE+18B9h)

## SOUND SOURCE SPECIFICATION

FEATURES

Built-in 4 tone counter programmable through 8 bit parallel data bus.

Built-in noise counter programmable through 8 bit parallel data bus.

Built-in 8 attenuator programmable through 8 bit parallel data bus.

Uses 3.072 MHz reference clock.

Possible to output sound at the same time.  (4 chords)

BLOCK DIAGRAM

## OPERATION

### Tone Generator

Each Tone Generator consists of frequency setting (Programmable Tone Counter) and level setting (Programmable Attenuator).  Frequency setting consists of 10 bits F9 (MSB) ~ F0 (LSB).  The base clock is divided to 1/16 with the division circuit, and provides up to ten levels of the Tone Counter.  Because a Flip/Flop is connected, twice the frequency value  16 must be set to the Tone Counter.  If the Tone Counter value is changed, the setting becomes valid from the edge of the square wave outputted.

Frequency Setting Calculation Method:

F = N/32n [Hz]  (N = base clock frequency [Hz], n = 10 bits binary)

The output of the Tone Counter is sent to the attenuator.  The attenuator value is set with 4 bits data as shown below in 16 steps of volume level, 0 [dB] to 0FF [dB].

| Bit Position |  |  |  |  |  |  | Decrease [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A3 | A2 | A1 | A0 |  |  |  | T6W28 |
| 0 | 0 | 0 | 0 |  |  |  | 0 |
| 0 | 0 | 0 | 1 |  |  |  | 2 |
| 0 | 0 | 1 | 0 |  |  |  | 4 |
| 0 | 0 | 1 | 1 |  |  |  | 6 |
| 0 | 1 | 0 | 0 |  |  |  | 8 |
| 0 | 1 | 0 | 1 |  |  |  | 10 |
| 0 | 1 | 1 | 0 |  |  |  | 12 |
| A3 | A2 | A1 | A0 |  |  |  | Decrease [dB] |
| 0 | 1 | 1 | 1 |  |  |  | 14 |
| 1 | 0 | 0 | 0 |  |  |  | 16 |
| 1 | 0 | 0 | 1 |  |  |  | 18 |
| 1 | 0 | 1 | 0 |  |  |  | 20 |
| 1 | 0 | 1 | 1 |  |  |  | 22 |
| 1 | 1 | 0 | 0 |  |  |  | 24 |
| 1 | 1 | 0 | 1 |  |  |  | 26 |
| 1 | 1 | 1 | 0 |  |  |  | 28 |
| 1 | 1 | 1 | 1 |  |  |  | OFF |

Table 1. Attenuation Control

### Noise Generator

The Noise Generator consists of the Noise Source and Attenuator.  The Noise Source is a shift register with a XOR feedback.  The feedback exists to avoid lock when the shift register becomes 0.  And when the Control Register is changed, the shift register is cleared.

| FB | NOISE MODE |
| --- | --- |
| 0 | Same Period Noise |
| 1 | White Noise |

Table 2. Noise Feedback Control

When the Noise Control Register is switched, the shift register is cleared.  4 types of clocks may be set for the operation clock of the shift register with 2 NF bits.  The shift rate is based on the base clock.

| NF1 | NF0 | SHIFT RATE |
| --- | --- | --- |
| 0 | 0 | N/512 |
| 0 | 1 | N/1024 |
| 1 | 0 | N/2048 |
| 1 | 1 | Tone Generator #3 Output |

Table 3. Noise generator Frequency Control

With the modes of NF0:1, NF1:1; Tone of the Noise may be continuously changed.  The setting relationship is shown below.

### Interface with PC and Z80

The interface of PC and Sound Controller is the Sound Source Control Register (Left: 18A1h, Right: 18A0h).  The interface of Z80 and Sound Controller is the Sound Source Control Register (Left: 4001h, Right: 4000h).  Each Tone generator requires 10 bits for frequency setting and 4 bits for attenuation setting.  Updating frequency requires 2 bytes of transfer, and attenuation requires 1 byte of transfer.  Frequency and address field data in the 1st byte, frequency data in the 2nd byte composes the input to the Tone generator (Assumption:  No other control registers are accessed.).  Care must be taken, since updating the 1st byte alone will make the input valid.  This register address is latched on chip and the data is also inputted.  Attenuator is similar except for the fact that 1 byte alone inputs value to the register and for attenuation.

#### Control Register

The Sound Source circuit has 8 registers to control 3 Tone generators and Noise source for each right and left channel.  When data is sent to the controller, the 3 bits of the first byte is used to determine the control register.  The register address is shown in Table 4.

| R2 | R1 | R0 | CONTROL REGISTER ASSIGNMENT |
| --- | --- | --- | --- |
| 0 | 0 | 0 | TONE1 frequency |
| 0 | 0 | 1 | TONE1 attenuation |
| 0 | 1 | 0 | TONE2 frequency |
| 0 | 1 | 1 | TONE2 attenuation |
| 1 | 0 | 0 | TONE3 frequency |
| 1 | 0 | 1 | TONE3 attenuation |
| 1 | 1 | 0 | Noise control |
| 1 | 1 | 1 | Noise attenuation |

Table 4. Register Address Field

#### Data Format

Transfer data format is shown below.

Frequency (2 bytes transfer)

| 1st byte | ADDR |  |  | DATA |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | R2 | R1 | R0 | F3 | F2 | F1 | F0 |
| Bit 7 | Bit 0 |  |  |  |  |  |  |

| 2nd byte | ADDR |  |  | DATA |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | X | F9 | F8 | F7 | F6 | F5 | F4 |
| Bit 7 | Bit 0 |  |  |  |  |  |  |

Noise Source (1 byte transfer)

| 1st byte | ADDR |  |  | DATA |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | R2 | R1 | R0 | X | FB | NF1 | NF0 |
| Bit 7 | Bit 0 |  |  |  |  |  |  |

Attenuator (1 byte transfer)

| 1st byte | ADDR |  |  | DATA |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | R2 | R1 | R0 | A3 | A2 | A1 | A0 |
| Bit 7 | Bit 0 |  |  |  |  |  |  |

### Output Frequency Calculation

Obtain the division ratio with the following equation to output a specific frequency.  It is necessary to set this frequency by transferring 2 bytes data to the control register.

n = Base clock/(32  Tone) … (Tone generator 1, 2, 3)

n = Base clock/(32  15  Tone) … (Same period noise mode)

n:  division ratio		Base clock:  3.072 MHz		Tone:  frequency wanted

Drop the values after the decimal place and change the division ratio to a 10 bit binary data format (data format in 4.2.3.2) and transfer the 1st and 2nd byte to set the value in the control register.  The following is an example for output frequency of 440 Hz.

(Tone generator 1, 2, 3 frequency)

n = 3072000/(32  440)

= 218.18…  218 (The n value with the decimals omitted shall be the division ratio value)

For reference purpose, the relationship between Tone and Division Ratio is shown.

Tone and Division Ratio Relationship for Each Tone Generator 1 ~ 3 (Clock:  3.072 MHz)

| Music Interval (Key) | Division Ratio | HEX |  | SGC | Actual | Music Interval (Key) | Division Ratio | HEX |  | SGC | Actual |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1st | 2nd | Output (Hz) | Frequency (Hz) |  |  | 1st | 2nd | Output (Hz) | Frequency (Hz) |
| A | 872 | X8 | 36 | 110.09 | 110.00 | C#5 | 173 | XD | 0A | 554.91 | 554.37 |
| A#2 | 823 | X7 | 33 | 116.65 | 116.54 | D5 | 163 | X3 | 0A | 588.96 | 587.33 |
| B2 | 777 | X9 | 30 | 123.55 | 123.47 | D#5 | 154 | XA | 09 | 623.38 | 622.25 |
| C3 | 733 | XD | 2D | 130.97 | 130.81 | E5 | 145 | X1 | 09 | 662.07 | 659.26 |
| C#3 | 692 | X4 | 2B | 138.73 | 138.59 | F5 | 137 | X9 | 08 | 700.73 | 698.46 |
| D3 | 653 | XD | 28 | 147.01 | 146.83 | F#5 | 129 | X1 | 08 | 744.19 | 739.99 |
| D#3 | 617 | X9 | 26 | 155.59 | 155.56 | G5 | 122 | XA | 07 | 786.89 | 783.99 |
| E3 | 582 | X6 | 24 | 164.95 | 164.81 | G#5 | 115 | X3 | 07 | 834.78 | 830.61 |
| F3 | 549 | X5 | 22 | 174.86 | 174.61 | A5 | 109 | XD | 06 | 880.73 | 880.00 |
| F#3 | 518 | X6 | 20 | 185.33 | 185.00 | A#5 | 102 | X6 | 06 | 941.18 | 932.33 |
| G3 | 489 | X9 | 1E | 196.32 | 196.00 | B5 | 97 | X1 | 06 | 989.69 | 987.77 |
| G#3 | 462 | XE | 1C | 207.79 | 207.65 | C6 | 91 | XB | 05 | 1054.9 | 1046.5 |
| A3 | 436 | X4 | 1B | 220.18 | 220.00 | C#6 | 86 | X6 | 05 | 1116.3 | 1108.7 |
| A#3 | 411 | XB | 19 | 233.58 | 233.08 | D6 | 81 | X1 | 05 | 1185.2 | 1174.7 |
| B3 | 388 | X4 | 18 | 247.42 | 246.94 | D#6 | 77 | XD | 04 | 1246.8 | 1244.5 |
| C4 | 366 | XE | 16 | 262.30 | 261.63 | E6 | 72 | X8 | 04 | 1333.3 | 1318.5 |
| C#4 | 345 | X9 | 15 | 278.26 | 277.57 | F6 | 68 | X4 | 04 | 1411.8 | 1396.5 |
| D4 | 326 | X6 | 14 | 294.48 | 293.66 | F#6 | 64 | X0 | 04 | 1500.0 | 1480.0 |
| D#4 | 308 | X4 | 13 | 311.69 | 311.13 | G6 | 61 | XD | 03 | 1573.8 | 1568.0 |
| E4 | 291 | X3 | 12 | 329.90 | 329.63 | G#6 | 57 | X9 | 03 | 1684.2 | 1661.2 |
| F4 | 274 | X2 | 11 | 350.36 | 349.23 | A6 | 54 | X6 | 03 | 1777.8 | 1760.0 |
| F#4 | 259 | X3 | 10 | 370.66 | 369.99 | A#6 | 51 | X3 | 03 | 1882.4 | 1864.7 |
| G4 | 244 | X4 | 0F | 393.44 | 392.00 | B6 | 48 | X0 | 03 | 2000.0 | 1975.7 |
| G#4 | 231 | X7 | 0E | 415.58 | 415.30 |  |  |  |  |  |  |
| A4 | 218 | XA | 0D | 440.37 | 440.00 |  |  |  |  |  |  |
| A#4 | 205 | XD | 0C | 468.29 | 466.16 |  |  |  |  |  |  |
| B4 | 194 | X2 | 0C | 494.85 | 493.88 |  |  |  |  |  |  |
| C5 | 183 | X7 | 0B | 524.59 | 523.25 |  |  |  |  |  |  |

Same Period Noise Mode Output Frequency (Clock:  3.072 MHz)

| Music Interval (Key) | Actual Frequency | Division Ratio | SGC Output | HEX TONE 3 |  |
| --- | --- | --- | --- | --- | --- |
|  | (Hz) |  | (Hz) | 1st | 2nd |
| C0 | 16.351 | 391 | 16.37 | C7 | 18 |
| C#0 | 17.324 | 369 | 17.34 | C1 | 17 |
| D0 | 18.354 | 348 | 18.39 | CC | 15 |
| D#0 | 19.445 | 329 | 19.45 | C9 | 14 |
| E0 | 20.601 | 310 | 20.65 | C6 | 13 |
| F0 | 21.827 | 293 | 21.84 | C5 | 12 |
| F#0 | 23.124 | 276 | 23.19 | C4 | 11 |
| G0 | 24.499 | 261 | 24.52 | C5 | 10 |
| G#0 | 25.959 | 246 | 26.02 | C6 | 0F |
| A0 | 27.500 | 232 | 27.59 | C8 | 0E |
| A#0 | 29.135 | 219 | 29.22 | CB | 0D |
| B0 | 30.868 | 207 | 30.92 | CF | 0C |
| C1 | 32.703 | 195 | 32.82 | C3 | C0 |
| C#1 | 34.648 | 184 | 34.78 | C8 | 0B |
| D1 | 36.708 | 174 | 36.78 | CE | 0A |
| D#1 | 38.891 | 164 | 39.02 | C4 | 0A |
| E1 | 41.203 | 155 | 41.29 | CB | 09 |
| F1 | 43.654 | 146 | 43.84 | C2 | 09 |
| F#1 | 46.249 | 138 | 46.38 | CA | 08 |
| G1 | 48.999 | 130 | 49.23 | C2 | 08 |
| G#1 | 51.913 | 123 | 52.03 | CB | 07 |
| A1 | 55.000 | 116 | 55.17 | C4 | 07 |
| A#1 | 58.270 | 109 | 58.72 | CD | 06 |
| B1 | 61.735 | 103 | 62.14 | C7 | 06 |
| C2 | 65.406 | 97 | 65.98 | C1 | 06 |
| C#2 | 69.296 | 92 | 69.57 | CC | 05 |
| D2 | 73.416 | 87 | 73.56 | C7 | 05 |
| D#2 | 77.782 | 82 | 78.05 | C2 | 05 |
| E2 | 82.407 | 77 | 83.12 | CD | 04 |
| F2 | 87.307 | 73 | 87.67 | C9 | 04 |
| F#2 | 92.499 | 69 | 92.75 | C5 | 04 |
| G2 | 97.999 | 65 | 98.46 | C1 | 04 |
| G#2 | 103.826 | 61 | 104.92 | CD | 03 |

# Z80 CPU

## Z80 SOUND SOURCE CONTROL CPU ADDRESS MAP

Z80 Sound Source Control CPU address map is shown below.

| 0000 H 0FFF H | Z80 RAM (4 Kbyte) |  |
| --- | --- | --- |
| 1000 H 3FFF H | Reserved |  |
| 4000 H 4001 H | Sound Source | RIGHT |
|  |  | LEFT |
| 4002 H 7FFF H | Reserved |  |
| 8000 H | PC-Z80 communication register (1 Byte) |  |
| 8001 H BFFF H | Reserved |  |
| C000 H | PC INTx control register (1 Byte) |  |
| C001 H FFFF H | Reserved |  |

* All area 0 WAIT operation.

* No I/O mapped devices

## I/O

### Z80 Timer Interrupt

There is no timer to set timing for the Sound Source T6W28 ( 2) inside the Z80 Sound Source control CPU.  Thus the Sound Source is connected through interrupt circuitry to the PC memory map timer register output TO3 (Timer 3 external output).  Timer settings are done by the PC, and the Z80 Sound Source Control CPU requests settings through the register.  Interrupt circuit connections are shown below.

### PC INT Control Register

| Address | Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C000 h | Initial Value > | - | - | - | - | - | - | - | - |  |
| Access = Write only 8 Bit |  | INTx request to the PC generated with wrtie access (data invalid) |  |  |  |  |  |  |  |  |

### PC – Z80 Communication Register

| Address | Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8000 h | Initial Value > | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| Access = Read/Write 8 Bit |  | PC – Z80 Communication Register (Dual Port) |  |  |  |  |  |  |  |  |

### Z80 Interrupt Acknowledge Register

| Address | Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00～FF h | Initial Value > | - | - | - | - | - | - | - | - |  |
| Access = Write only 8 Bit |  | Releases INT request to the Z80 with write access (invalid data). |  |  |  |  |  |  |  |  |

Please refer to 3.3.1 Z80 RAM for further information on Z80 RAM.  Please refer to 4.1 Sound Source Specification for further information on Sound Source.

# ADDITIONAL MATERIAL

## SOUND ELEMENTS AND T6W28 RELATIONSHIP

### Sound Elements

Currently, sound occurring in nature is separated into pitch, volume, tone, and the output method.

Sound Pitch (frequency)

Tone (harmonic overtone structure)

Sound Strength (sound pressure value)

Sound Output Method (envelope)

- Sound Pitch

Although there are some differences from person to person, 20 ~ 30 Hz to 16kHz ~ 20 kHz is the range of human hearing; but structure of sound is more dependent on the other two factors.

- Tone

People refer to “light (dark) sounds,” and “soft (hard) sounds” but these are description of tone differences and are resultant of the wave form.

- Sound Strength

Normally this refers to the continuous output level, but is very important when two or more frequencies are mixed.

- Sound Output Method

This is closely related to shape of the wave to differentiate sound, but also entails volume change (Sound output start to end gradients).

Depending on the Sound Output Method, the harmonic overtone structure and its frequency may be changed.  There is a strong connection with the amplitude.

### Sound Element and T6W28 Relationship

| Sound Element | Physical Attribute | Relationship to T6W28 |
| --- | --- | --- |
| Sound Pitch | Frequency | Tone generator and noise generator frequency control |
| Tone | Sound Harmonic Structure | Mainly wave shape related, but the T6W28 has 50% duty pulse wave shape. 3 frequency output is possible and with the ATT control Sound Harmonic Structure is controllable. |
| Sound Strength | Amplitude | The 3 tone and noise generators have 4 bit attenuation control for each. |
| Sound Output Method | Envelope | It is possible to control each attenuation with external data. ATT control is possible in the following method: 4 bit DATA envelope sequence control on each step. * Frequency control is possible in the following area: f = N/32n, noise, frequency element control. |

## D/A CONVERTER SPECIFICATION

The board uses 2 (left and right) D/A converter with the following specification.

LINEAR-TECHNOLOGY PRODUCT

| SYMBOL | PARAMETER | CONDITIONS | MIN | TYP | MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| Operation Power Source |  |  |  |  |  |  |
| Vcc | Operation Voltage |  | 4.5 |  | 5.5 | V |
| Icc | Current Use | 4.5V ≤ Vcc ≤ 5.5V (*1) | 300 | 400 | 620 | A |
| D/A Specification |  |  |  |  |  |  |
|  | Division |  | 12 | - | - | Bit |
| DNL | Derivative Linear Error | (*2) |  |  | 0.5 | LSB |
| INL | Non-Linear Error | T = 25C (*2) |  |  | 4.0 | LSB |
| Vos | Offset Error | T = 25C |  |  | 18 | mV |
| Vos TC | Offset Error Temp. Coefficient |  |  | ±15 |  | V/C |
| Vfs | Full Scale Output Voltage |  | 2.006 | 2.048 | 2.088 | V |
| Vfs TC | Full Scale Output Voltage Temp. Coeff. |  |  | ±0.02 |  | LSB/C |
| AC Specification |  |  |  |  |  |  |
|  | Voltage Output Through Rate | (*3) | 0.5 | 1.0 |  | V/s |
|  | Voltage Output Settling Time | (*3, 4) to ±0.5LSB |  | 14 |  | s |

*1 Digital input signal 0 V ~ Vcc.

*2 Non-linearity is declared with the largest code from maximum offset value with full scale value.

*3 Burden 5 k, 100 pF (Parallel connection with output terminal.

*4 Input code as “18”.

### Memory for D/A

The board has an access mode to allow a PC to directly access the D/A converter.  It is necessary to set a value in the DAC control register to use this mode.  The value must be modified taking K1 chip output specifications to become input value for the D/A converter.  The relationship for register values to output voltage is shown below.  K1 chip output simulation table uses the preceding section information and the table below.  SRAM data and output relationship is listed below.

| SRAM |  | Output | SRAM |  | Output | SRAM |  | Output | SRAM |  | Output |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data (HEX) |  | Voltage (V) | Data (HEX) |  | Voltage (V) | Data (HEX) |  | Voltage (V) | Data (HEX) |  | Voltage (V) |
| 000 |  | 0.060 | 39D |  | 0.463 | 73A |  | 0.925 | AC8 |  | 1.380 |
| 028 |  | 0.065 | 3B6 |  | 0.490 | 771 |  | 0.953 | AEB |  | 1.413 |
| 064 |  | 0.080 | 40B |  | 0.518 | 7A3 |  | 0.978 | B40 |  | 1.440 |
| 08C |  | 0.085 | 44C |  | 0.550 | 7DF |  | 1.008 | B81 |  | 1.473 |
| 0C8 |  | 0.115 | 47E |  | 0.575 | 81B |  | 1.038 | BB3 |  | 1.498 |
| 0D2 |  | 0.145 | 4BA |  | 0.605 | 852 |  | 1.065 | BEA |  | 1.525 |
| 13B |  | 0.173 | 4D3 |  | 0.633 | 88E |  | 1.095 | C26 |  | 1.555 |
| 195 |  | 0.203 | 532 |  | 0.665 | 8CA |  | 1.125 | C6C |  | 1.590 |
| 1AE |  | 0.230 | 54B |  | 0.693 | 901 |  | 1.153 | C94 |  | 1.610 |
| 208 |  | 0.260 | 5A0 |  | 0.720 | 938 |  | 1.180 | CDF |  | 1.648 |
| 244 |  | 0.290 | 5D7 |  | 0.748 | 974 |  | 1.210 | D39 |  | 1.693 |
| 258 |  | 0.315 | 618 |  | 0.780 | 9B0 |  | 1.240 | D48 |  | 1.700 |
| 294 |  | 0.345 | 64F |  | 0.808 | 9E7 |  | 1.268 | D61 |  | 1.713 |
| 2EE |  | 0.375 | 668 |  | 0.835 | A1E |  | 1.295 | DAC |  | 1.750 |
| 2FD |  | 0.398 | 6C2 |  | 0.865 | A5F |  | 1.328 | E06 |  | 1.795 |
| 361 |  | 0.433 | 6F9 |  | 0.893 | A96 |  | 1.355 | E2E |  | 1.815 |

## BOARD FIGURE

DIP switches to set BASE address

Stereo Sound Volume

(Clock wise will increase volume)

Optional Terminal

Stereo Earphone Jack

Optional Power Supply Terminal (+5V)