K2GE

TECHNICAL REFERENCE

MANUAL

CONFIDENTIAL:  DEVELOPERS ONLY

FOR GENERAL DEVELOPMENT

Revision History

| Ver | Date | Revision | In Charge | Requested |
| --- | --- | --- | --- | --- |
| 0.02 |  | Initial Release | Takeda | - |
| 1.00 | 1998.11.18 | Official Release | Takeda | - |
|  |  |  |  | - |
|  |  |  |  | - |

Table of Contents

1.   OUTLINE	5

2.   FEATURES	5

3.   DISPLAY SPECIFICATION	5

3-1.  COORDINATES AND DISPLAY AREA	5

4.   GRAPHIC ENGINE	6

4-1.  MEMORY LAYOUT	6

4-2. MEMORY MAP	7

4-3.	SPRITE CONTROL	8

4-3-1.	Specification for Sprite Control	8

4-3-2.	Display Function for Sprite Control	8

4-3-3.	Data Format for Sprite VRAM	8

4-3-4.	Sprite Position Offset Function	12

4-3-5.  Data Format for Sprite Characters	13

4-4.	SCROLL CONTROL	14

4-4-1.	Specification for Scroll Control	14

4-4-2.	Display Function for Scroll Control	14

4-4-3.	Relationship of VRAM Address and Scroll Plane	14

4-4-4.	Data Format for Scroll Plane VRAM	15

4-4-5.	VRAM Address and the Relationship to the Position on Screen	16

4-4-6.	Priority Levels of Scroll Planes	16

4-4-7.	Register to Change Priority Level of Scroll Planes	17

4-4-8.	Scroll Offset Register	18

4-4-9.	Scroll Character Data Format	19

4-5.	WINDOW REGISTER	20

4-5-1.	Relationship of Vint and Screen Setup	21

4-5-2.	Relationship of Hint and Screen Setup	21

4-6.	BACKGROUND COLOR REGISTER	21

4-7.	FRAME RATE REGISTER	21

4-8.	Raster Position Register	22

4-9.  MODE SELECTION REGISTER	22

4-10.  2D STATUS REGISTER	23

4-11.	2D CONTROL REGISTER	24

4-12.	PALETTE LUT FOR SPRITES	25

4-12-1.	Color Change and Bit Weight	26

4-13.	Palette LUT for Scroll Plane 1	27

4-14.	Palette LUT for Scroll Plane 2	28

4-15.  COLOR PALETTE RAM	29

5.   K1GEK2GE EXPANDED FUNCTION SUMMARY	31

5-1.	SUMMARY	31

5-2.	K2GE MODE COLOR PALETTE RAM ADDRESS CALCULATION	32

5-3.	K1GE UPPER PALETTE COMPATIBLE MODE COLOR PALETTE RAM ADDRESS CALCULATION	33

5-4.	BACKGROUND COLOR PALETTE RAM ADRESS CALCULATION	34

5-5.	WINDOW COLOR PALETTE RAM ADDRESS CALCULATION	34

6.   OPERATION TIMING FIGURE	35

7.   COLOR PALETTE TABLE	36

8.   REGISTER TABLE	37

# 1.   OUTLINE

This manual is for the “K2 Graphics Engine” (noted as K2GE from this point on), a modified K1 Graphic Engine with color capability.  (Functions enhanced or added is noted with .)

# 2.   FEATURES

1.  Double line buffer method

2.  Character RAM method

- K1GE upper palette compatible mode

-  Color LCD compatible

-  Maximum output color

In K1GE upper palette compatible mode = 20 out of 4096 colors simultaneous output

(18 colors + 1 background color + 1 window color)

In K2GE mode = 146 out of 4096 colors simultaneous output

(144 colors + 1 background color + 1 window color)

# 3.   DISPLAY SPECIFICATION

| SCREEN SIZE | 160[dot]x152[dot] |
| --- | --- |
| COLOR LEVEL | K1GE upper palette compatible mode: maximum 20 out of 4096 colors (simultaneous display) K2GE mode: maximum 146 out of 4096 colors (simultaneous display) |
| FRAME RATE | 60[F/S] |
| INTERFACE | Color LCD interface |
| SPRITE CONTROLLER | 64 8[dot]x8[dot] character displayable |
| SCROLL CONTROLLER | 2 dot scrollable background displayable |

## 3-1.  COORDINATES AND DISPLAY AREA

| VIRTUAL DISPLAY AREA | 256[dot]x256[dot] cyclical structure |
| --- | --- |
| DISPLAY AREA | 160[dot]x152[dot] |

FIGURE 1. WORLD COORDINATES AND DISPLAY AREA

# 4.   GRAPHIC ENGINE

## 4-1.  MEMORY LAYOUT

The memory of K2GE is divided into character RAM, VRAM, color palette RAM, and line buffer.  They are included in the K2GE block.  Character RAM, VRAM and color palette RAM are unified memory all accessible from the 900/H CPU.  The layout and amount of memory is as follows:

| Character RAM |  |  |
| --- | --- | --- |
| Size | : | 8KB (actual on board memory [2048x8]x4 sets) |
| Layout | : | 512x{(8x8)x2} = 65.536Kb = 8.192KB characters x {(dots vertically x dots horizontally) x bits necessary for gradation display} |
| VRAM |  |  |
| Size | : | 4384 Bytes (actual on board memory = {[1024x8]x4sets}+{[64x8]x4sets} |
| Layout | : | 1024x16x2 = 32.768Kb = 4.096KB (scroll control only) maximum characters(1 screen) definable in RAM x parameters necessary to describe 1 character x scrollable plane numbers  64x32 = 2.304Kb = 288Bytes (sprite control only) maximum characters (1 screen) definable in RAM x parameters necessary to describe 1 character |
| Color Palette RAM |  |  |
| Size | : | 312 Bytes (actual on board memory = [256x12]x1 set) |
| Layout | : | (144+48+8+8)x(4+4+4) = 2496b = 312Bytes number of palettes x (R[4bits]+G[4bits]+B[4bits]) |
| Line Buffer |  |  |
| Size | : |  360Bytes (actual on board memory = {[16x24]x6}x2 sets) |
| Layout | : |  {20x(9x8)}x2 = 360Bytes {number of horizontal display characters x (number of bits necessary to describe 1 dot x number of dots horizontally for a character)} x 2 |

## 4-2. MEMORY MAP

| 0X8000 | Control Registers |  | There areas are not used currently, but PLEASE DO NOT ACCESS these areas. (Future expansion) |
| --- | --- | --- | --- |
| 0X81FF |  |  |  |
| 0X8200 0X83FF | Color Palette RAM |  |  |
| 0X8400 0X87FF | Control Registers |  | There areas are not used currently, but PLEASE DO NOT ACCESS these areas. (Future expansion) |
| 0X8800 | Sprite VRAM |  | There areas are not used currently, but PLEASE DO NOT ACCESS these areas. (Future expansion) |
| 0X88FF |  |  |  |
| 0X8900 | Vacant |  | PLEASE DO NOT ACCESS. (Future expansion). |
| 0X8BFF |  |  |  |
| 0X8C00 0X8C3F | Sprite VRAM (color palette code) |  |  |
| 0X8C40 0X8FFF | Vacant |  | PLEASE DO NOT ACCESS. (Future expansion). |
| 0X9000 | Scroll VRAM |  |  |
| 0X9FFF |  |  |  |
| 0XA000 | Character RAM |  |  |
| 0XBFFF |  |  |  |

FIGURE 2. K2GE MEMORY MAP

## SPRITE CONTROL

### Specification for Sprite Control

| # of sprites displayable in one frame |  | 64 |
| --- | --- | --- |
| # of sprites displayable in one line |  | 64 |
| maximum characters definable in VRAM |  | 64 |
| maximum character definable |  | 512 characters |
| # of color usable |  | K1 upper palette compatible mode: 6 colors out of 4096 + clear color Each character contains 4 colors of which one is clear K2GE mode: 48 colors out of 4096 + clear color Each character contains 4 colors of which one is clear |
| character size |  | 8[dot]×8[dot] fixed |

### Display Function for Sprite Control

| function available to whole screen |  | Position correction function |
| --- | --- | --- |
| function available to each sprite |  | Flip function |
|  |  | Defining priority with scroll screen |
|  |  | Character position chain function |

### Data Format for Sprite VRAM

VRAM data format is as follows.

| ADDRESS | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0X8800 | C.C(8+1Bit) |  |  |  |  |  |  |  |
| 0X8801 | H.F | V.F | P.C | PR.C |  | H.ch | V.ch | C.C |
| 0X8802 | H.P(8Bit) |  |  |  |  |  |  |  |
| 0X8803 | V.P(8Bit) |  |  |  |  |  |  |  |

+

| ADDRESS | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x8C00 | D.C (Does not exist as RAM. During read the value is fixed as “0” in K2GE mode) |  |  |  | CP.C (Only exists in K2GE mode) |  |  |  |

*Start address for VRAM is 0X8800.  The above example shows the first set of parameters, 4 bytes each.  Since there are memory space for 64 sprites, there are 256 bytes available.

The K2GE has an additional byte per character starting from 0X8C00 for the color palette code.  This color palette is only valid in K2GE mode and does not exist in K1GE upper palette compatible mode.

(Example:  First sprite = 0X8800 ~ 0X8803 + 0X8C00, Second sprite = 0X8804 ~ 0X8807 + 0X8C01)

Table 1. Parameters and Definitions

| V.P |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V position | Y coordinate for the character. |  |  |  |  |  |  |  |
| H.P |  |  |  |  |  |  |  |  |
| H position | X coordinate for the character. |  |  |  |  |  |  |  |
| C.C |  |  |  |  |  |  |  |  |
| Character code | Setting character code. |  |  |  |  |  |  |  |
| H.F |  |  |  |  |  |  |  |  |
| H Flip | Character display status. |  |  |  |  |  |  |  |
|  |  |  | H.F Logic | Display status |  |  |  |  |
|  |  |  | 0 | Normal |  |  |  |  |
|  |  |  | 1 | Horizontally mirrored |  |  |  |  |
| V.F |  |  |  |  |  |  |  |  |
| V Flip | Character display status. |  |  |  |  |  |  |  |
|  |  |  | V.F Logic | Display status |  |  |  |  |
|  |  |  | 0 | Normal |  |  |  |  |
|  |  |  | 1 | Vertically Mirrored |  |  |  |  |
| P.C |  |  |  |  |  |  |  |  |
| Palette Code | Specifies palette code. (Valid for K1GE upper palette compatible mode.) |  |  |  |  |  |  |  |
| PR.C |  |  |  |  |  |  |  |  |
| Priority Code | Priority level with respect to scroll screen. |  |  |  |  |  |  |  |
|  |  | Value of PR.C |  |  | Priority |  |  |  |
|  |  | 00 |  |  | Character not shown |  |  |  |
|  |  | 01 |  |  | Furthest |  |  |  |
|  |  | 10 |  |  | Middle |  |  |  |
|  |  | 11 |  |  | Front |  |  |  |
| H.ch |  |  |  |  |  |  |  |  |
| H Position Chain | Value defined becomes the offset value with respect to the previous character. |  |  |  |  |  |  |  |
|  |  |  | H.ch Logic | Display status |  |  |  |  |
|  |  |  | 0 | Normal Coordinates |  |  |  |  |
|  |  |  | 1 | Offset Coordinates |  |  |  |  |
| V.ch |  |  |  |  |  |  |  |  |
| V Position Chain | Value defined becomes the offset value with respect to the previous character. |  |  |  |  |  |  |  |
|  |  |  | V.ch Logic | Display status |  |  |  |  |
|  |  |  | 0 | Normal Coordinates |  |  |  |  |
|  |  |  | 1 | Offset Coordinates |  |  |  |  |

(Continued)

| (Continued) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CP.C (exists only in K2GE mode) |  |  |  |  |  |
| Color Palette Code | Value defined becomes the color palette code for the sprite |  |  |  |  |
|  |  |  | Value of CP.C | Color Palette Code |  |
|  |  |  | 0 ~ 15 | Selection of Sprite Palette 0 ~ 15 |  |
| D.C |  |  |  |  |  |
| Do not care |  | This register value has no effect. |  |  |  |

#### VRAM Address and Character Sprite Priority

Priority for sprites on screen is dependent on the VRAM address.  The hardware reads the values from the VRAM 0 address and writes to the line buffer.  During the write to the line buffer, the hardware checks the priority as follows to avoid writing over previously written data.

Figure 3. VRAM ADDRESS and Priority Relation

#### Scroll Screen and Sprite Priority

Figure 4 shows the PR.C value and the priority.

Figure 4. Scroll Screen and Sprite Relationship

### Sprite Position Offset Function

The value inside this register is added as an offset to the sprite position.  The relationship between sprite position and offset is as follows:

H'=H.P+PO.H

V'=V.P+PO.V(over flow ignored)

Because a value over 256 is ignored, over flow values are ignored and a value is set to 256.  Since this value is always added to the sprite position, please initialize with 0X00 if offset is not required.

| ADDRESS | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0X8020 | PO.H(8Bit) |  |  |  |  |  |  |  |
| 0X8021 | PO.V(8Bit) |  |  |  |  |  |  |  |

Caution: When the value in this register is set, it is reflected in the following line drawn on screen.

Table 2. Parameters and Definitions

| PO.H |  |
| --- | --- |
| H Position offset | Value to be added to the X value of the sprite. After reset, the initial value is 0X00. |
| PO.V |  |
| V Position offset | Value to be added to the Y value of the sprite. After reset, the initial value is 0X00. |

### 4-3-5.  Data Format for Sprite Characters

Each dot of the sprite character has 4 color codes available (2Bits).  Bits are positioned as in Figure 5.  Address bus above A4 corresponds with the character code.

Figure 5. Sprite Character RaM Data Format

#### Relationship Between Sprite Character Data and Address

The relationship between character dot data and its address is as follows.  The table below shows data and address for one character.  1 row of data represents 1 line on screen.

| ADDRESS | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0XA000 | Dot 4 |  | Dot 5 |  | Dot 6 |  | Dot 7 |  |
| 0XA001 | Dot 0 |  | Dot 1 |  | Dot 2 |  | Dot 3 |  |
| 0XA002 | Dot 4 |  | Dot 5 |  | Dot 6 |  | Dot 7 |  |
| 0XA003 | Dot 0 |  | Dot 1 |  | Dot 2 |  | Dot 3 |  |
| 0XBFFC | Dot 4 |  | Dot 5 |  | Dot 6 |  | Dot 7 |  |
| 0XBFFD | Dot 0 |  | Dot 1 |  | Dot 2 |  | Dot 3 |  |
| 0XBFFE | Dot 4 |  | Dot 5 |  | Dot 6 |  | Dot 7 |  |
| 0XBFFF | Dot 0 |  | Dot 1 |  | Dot 2 |  | Dot 3 |  |

## SCROLL CONTROL

### Specification for Scroll Control

| Scrollable planes |  | 2 planes |
| --- | --- | --- |
| Virtual screen size |  | 256[dot]x256[dot] |
| Maximum VRAM definable characters |  | 2048characters |
| Maximum definable characters |  | 512characters |
| Usable colors |  | K1 upper palette compatible mode: (6 colors out of 4096 + clear color) x 2 planes Each character contains 4 colors of which one is clear K2GE mode: (48 colors out of 4096 + clear color) x 2 planes Each character contains 4 colors of which one is clear |
| Character size |  | 8[dot]x8[dot]fixed |

### Display Function for Scroll Control

| Function applicable to screen |  | Changing priority of scroll planes |
| --- | --- | --- |
|  |  | Function to read raster position |
|  |  | Screen control such as forced blanking |
| Function applicable to each character |  | Flip(mirroring) function |
| Other functions |  | Scan line interrupt(Hint) |
|  |  | Frame interrupt(Vint) |

### Relationship of VRAM Address and Scroll Plane

Figure 6 shows VRAM address and scroll plane relationship.

| 0X9000 0X97FF 0X9800 0X9FFF | Scroll Plane 1 |
| --- | --- |
|  | Scroll Plane 2 |

Figure 6. VRAM Address and Scroll Plane Relationship

### Data Format for Scroll Plane VRAM

VRAM data format is as follows.

| ADDRESS | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0X9000 | C.C(8+1Bit) |  |  |  |  |  |  |  |
| 0X9001 | H.F | V.F | P.C | CP.C (Can be set only in K2GE mode. In K1GE upper palette compatible mode, it exists as reserved RAM.) |  |  |  | C.C |

* Start address of VRAM for scroll plane is 0X9000.  Only one parameter group is shown above.  Address increases by 2 bytes.

Table 3. Parameters and Definitions

| C.C |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Character Code | Sets character code. Character code is common with the sprites. |  |  |  |  |  |  |
| H.F |  |  |  |  |  |  |  |
| H Flip | Character display status. |  |  |  |  |  |  |
|  |  |  | H.F Logic |  | Display status |  |  |
|  |  |  | 0 |  | Normal |  |  |
|  |  |  | 1 |  | Horizontally mirrored |  |  |
| V.F |  |  |  |  |  |  |  |
| V Flip | Character display status. |  |  |  |  |  |  |
|  |  |  | V.F Logic |  | Display status |  |  |
|  |  |  | 0 |  | Normal |  |  |
|  |  |  | 1 |  | Vertically mirrored |  |  |
| P.C |  |  |  |  |  |  |  |
| Palette Code | Setting palette code. (Only valid in K1GE upper palette compatible mode.) |  |  |  |  |  |  |
| CP.C |  |  |  |  |  |  |  |
| Color Palette Code | Sets color palette code for scroll planes. |  |  |  |  |  |  |
|  |  | Value of CP.C |  | Color Palette Code |  |  |  |
|  |  | 0 ~ 15 |  | Selection of scroll palette 0 ~ 15 |  |  |  |

### VRAM Address and the Relationship to the Position on Screen

VRAM address and the positioning on screen with respect to each other are listed below.  Increase in address results in horizontal progression of characters.  The second scroll screen address starts from 0X9800 to 0X9FFE in a similar fashion.

| 0X9000 | 0X9002 | 0X9004 |  |  |  | 0X903E |
| --- | --- | --- | --- | --- | --- | --- |
| 0X9040 | 0X9042 | 0X9044 |  |  |  | 0X907E |
| 0X97C0 | 0X97C2 | 0X97C4 |  |  |  | 0X97FE |

Figure 7. VRAM Address and Screen Position Relationship

### Priority Levels of Scroll Planes

Priority of the scroll plane with respect to each other is as follows.  Changing the value in the register switches the priority level.

Figure 8. Scroll Plane Priority

### Register to Change Priority Level of Scroll Planes

This register sets the scroll plane priority level.  The relationship of register value and priority is shown in Figure 9.

| ADDRESS | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0X8030 | P.F | D.C |  |  |  |  |  |  |

Caution:

The result of the value set in this register is displayed from the next line being drawn.

Table 5. Parameters and Definitions

| P.F |  |  |  |  |
| --- | --- | --- | --- | --- |
| Scroll Plane Priority Switching | Switches the priority level of scroll planes. |  |  |  |
|  |  | P.F Logic | Display Status |  |
|  |  | 0 | Normal: Scroll Plane 1 is in front. After system reset, this is the value (initial). |  |
|  |  | 1 | Switched: Scroll Plane 2 is in front. |  |

Figure 9. P.F Logic Value and Priority RELATIONSHIP

### Scroll Offset Register

Sets the scroll plane offset value.  The relation of offset and display is shown in Figure 10.

| ADDRESS | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0X8032 | S1SO.H(8Bit) |  |  |  |  |  |  |  |
| 0X8033 | S1SO.V(8Bit) |  |  |  |  |  |  |  |
| 0X8034 | S2SO.H(8Bit) |  |  |  |  |  |  |  |
| 0X8035 | S2SO.V(8Bit) |  |  |  |  |  |  |  |

Caution:

The result of the value set in this register is displayed from the next line being drawn.

Table 6. Parameters and Definitions

| S1SO.H |  |
| --- | --- |
| Scroll Plane 1 H Scroll Offset | Scroll Plane 1 X scroll offset. After system reset, 0x00(initial value). |
| S1SO.V |  |
| Scroll Plane 1 V Scroll Offset | Scroll Plane 1 Y scroll offset. After system reset, 0x00(initial value). |
| S2SO.H |  |
| Scroll Plane 2 H Scroll Offset | Scroll Plane 2 X scroll offset. After system reset, 0x00(initial value). |
| S2SO.V |  |
| Scroll Plane 2 V Scroll Offset | Scroll Plane 2 Y scroll offset. After system reset, 0x00(initial value). |

Figure 10. Scroll Offset and Display Area

### Scroll Character Data Format

The data format is exactly the same as it is for the sprites.  The explanation will be omitted.

## WINDOW REGISTER

Determines the window display area.  WBA.n is the window origin, and WSI.n determines the window size.  Non-display area is blank.  Figure 11 shows the register, window, and screen relationship.

| ADDRESS | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0X8002 | WBA.H(8Bit) |  |  |  |  |  |  |  |
| 0X8003 | WBA.V(8Bit) |  |  |  |  |  |  |  |
| 0X8004 | WSI.H(8Bit) |  |  |  |  |  |  |  |
| 0X8005 | WSI.V(8Bit) |  |  |  |  |  |  |  |

Caution:

The window is defined by the origin and the size, and thus the maximum value is determined by the sum of WBA.n and WSI.n.  When the sum exceeds the hardware upper limit of 256, display and Vint/Hint generations are disrupted.

To avoid this disruption, please use and follow the condition given below.

Horizontal Value:	WBA.H + WSI.H ≤ 160

Vertical Value:		WBA.V + WSI.V ≤ 152

Table 6. Parameters and Definitions

| WBA.H |  |
| --- | --- |
| Window H origin | Sets the window origin X value. After system reset 0x00(initial value) |
| WBA.V |  |
| Window V origin | Sets the window origin Y value. After system reset 0x00(initial value) |
| WSI.H |  |
| Window H size | Sets the X size of window. After system reset 0xFF(initial value) |
| WSI.V |  |
| Window V size | Sets the Y size of window. After system reset 0xFF(initial value) |

Figure 11. Display Area and Window Relationship

### Relationship of Vint and Screen Setup

After the screen is drawn by the K1GE, an interrupt is generated and sent to the CPU.  The timing of the interrupt generation is determined by the value set in the window register and care must be taken.  This value is evaluated after the end of drawing in the display area, and Vint is generated after WBA.V + WSI.V line is drawn.  If the WSI.V is “0”, Vint is produced after WBA.V line.

### Relationship of Hint and Screen Setup

Hint is different from Vint and is not dependent on the value set in the Window Register.  It is constant and 152 Hint occur every time.

## BACKGROUND COLOR REGISTER

The value defines the background color.

| ADDRESS | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0X8118 | BGON |  | D.C (Exists as reserved RAM) |  |  | BGC |  |  |

Table 7. Parameters and Definitions

| BGON |  |
| --- | --- |
| Background ON | “D7=1, D6=0” sets the BGC valid. Other values set the BGC to be not valid, and the background color is set to black. After reset, the initial values are “D7=0, D6=0”. |
| BGC |  |
| Background Color | Sets the background color (color code). The value set determines the BGC color palette. After reset, the initial values are “D2=0, D1=0, D0=0”. |
| D.C (Exists as reserved RAM) |  |
| Do not Care | The values in this register has no effect |

Caution:

The value set in this register is reflected from the next line on screen.

## FRAME RATE REGISTER

This register is locked and only a priority user may be allowed to change the values.  Please do not change the initial value set.

| ADDRESS | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0X8006 | REF (8Bit) |  |  |  |  |  |  |  |

Table 8. Parameters and Definitions

| REF |  |
| --- | --- |
| Refresh rate | Sets the blanking period. After reset, the initial value is 0XC6. |

## Raster Position Register

Reading the value in this register allows access to the current raster position.

| ADDRESS | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0X8008 | RAS.H(8Bit) |  |  |  |  |  |  |  |
| 0X8009 | RAS.V(8Bit) |  |  |  |  |  |  |  |

Caution:		LCD and Scan Line

Unlike a CRT, LCD does not have a scan line.  In this case, the signal inside of the ASIC is used as the raster position.

Upper 8 bits of the 10 bit internal subtraction counter of the horizontal drawing operation time (internally 515 clock) is read in to RAS.H.  (The value decreases as the horizontal drawing operation period progresses, and this value is accessible during V blank.)

RAS.V  obtains the current line number during horizontal drawing operation.  (Accessible during V blank.)

Table 9. Parameters and Definitions

| RAS.H |  |
| --- | --- |
| Raster position H | Remaining horizontal drawing operation time |
| RAS.V |  |
| Raster position V | Horizontal drawing operation line number（Vertical Coordinate） |

## 4-9.  MODE SELECTION REGISTER

This register defines the operation mode of the K2GE.

This register is locked and only a priority user may be allowed to change the values.  Please do not change the initial value set.

| ADDRESS | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0X87E2 | MODE | D.C (Does not exist as RAM. Fixed as “0” when read. |  |  |  |  |  |  |

Table 10. Parameters and Definitions

| MODE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Selection of operation mode | Sets the operation mode of K2GE. |  |  |  |
|  |  | MODE Logic | Display Status |  |
|  |  | 0 | K2GE Color mode. (After system reset, this is the initial value.) |  |
|  |  | 1 | K1GE upper palette compatible mode. |  |
| D.C (Does not exist as RAM) |  |  |  |  |
| Do not Care | This register value has no effect on the circuit operation. |  |  |  |

## 4-10.  2D STATUS REGISTER

This register is used to determine the internal status of K2GE.

| ADDRESS | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0X8010 | C.OVR | BLNK | D.C ( Does not exist as RAM. When read, it is fixed as “0”.) |  |  |  |  |  |

Table 11. Parameter and Definitions

| C.OVR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Character Over |  | Determines the character over status. |  |  |  |  |
|  |  |  | C.OVER Logic | Operation |  |  |
|  |  |  | 0 | Character Over has not occurred |  |  |
|  |  |  | 1 | Character Over has occurred: With the end of V blanking, it is cleared |  |  |
| BLNK |  |  |  |  |  |  |
| Blanking |  | Determines if blanking or not |  |  |  |  |
|  |  |  | BLNK Logic | Operation |  |  |
|  |  |  | 0 | Displaying |  |  |
|  |  |  | 1 | V Blanking |  |  |
| D.C (Exists as reserved RAM) |  |  |  |  |  |  |
| Do not Care | The value set in this register has no effect on circuit operation. |  |  |  |  |  |

Character Over is:

K2GE sprites use the line buffer method.  Character Over is a phenomenon associated with a line buffer method sprite system.  This phenomenon results from the system determining visibility of the sprite after reading in from the VRAM and then updating the line buffer.  Due to an operation which results in this process not being completed in one scan line period, the character disappears partially or completely.  This phenomenon is called Character Over, or Sprite Line Over.  Because the K2GE writes two scroll plane and sprites to the line buffer, processing time is limited.  Even so, the system has been designed to complete the task, and normally Character Over will not occur.  But if a program is written that accesses the VRAM frequently, Character Over may occur and care must be taken.

## 2D CONTROL REGISTER

This register sets the display setting and color setting for outside the window related to the LCD display.

| ADDRESS | D7 |  | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0X8012 | NEG | D.C (Exists as reserved RAM) |  |  |  |  | OOWC |  |  |

Caution:

Setting in this register is reflected in the next line being drawn on screen.

Table 12. Parameters and Definitions

| NEG |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Negative and Positive Switched |  | Negative and positive of the screen display is switched. |  |  |  |
|  |  |  | NEG Logic | Operation |  |
|  |  |  | 0 | Normal display: After system reset, this is the value (initial). |  |
|  |  |  | 1 | Switched display |  |
|  |  | This function affects the RGB signal outputted by the K2GE. |  |  |  |
| OOWC |  |  |  |  |  |
| Outside window color |  | Color setting outside the window. After reset 0X0 (initial). This setting colors through the use of the background color palette. This is used both for K1GE upper palette compatible and K2GE mode. |  |  |  |
| D.C (Exists as reserved RAM) |  |  |  |  |  |
| Do not Care | Value set in this register has no effect on the operation of circuits. |  |  |  |  |

## PALETTE LUT FOR SPRITES

Palette code (P.C) defined in the sprite VRAM is changed to the value set in this register.  Register format is as follows:

The value set is only valid in K1GE upper palette compatible mode.

| ADDRESS | D7 | D6 | D5 | D4 | D3 |  | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0X8100 | Access not allowed. (Does not exist as RAM. No specific values when read.) |  |  |  |  |  |  |  |  |
| 0X8101 | D.C (Does not exist as RAM. “0” when read.) |  |  |  |  | SPPLT.01 |  |  |  |
| 0X8102 | D.C (Does not exist as RAM. “0” when read.) |  |  |  |  | SPPLT.02 |  |  |  |
| 0X8103 | D.C (Does not exist as RAM. “0” when read.) |  |  |  |  | SPPLT.03 |  |  |  |
| 0X8104 | Access not allowed. (Does not exist as RAM. No specific values when read.) |  |  |  |  |  |  |  |  |
| 0X8105 | D.C (Does not exist as RAM. “0” when read.) |  |  |  |  | SPPLT.11 |  |  |  |
| 0X8106 | D.C (Does not exist as RAM. “0” when read.) |  |  |  |  | SPPLT.12 |  |  |  |
| 0X8107 | D.C (Does not exist as RAM. “0” when read.) |  |  |  |  | SPPLT.13 |  |  |  |

Caution 1:		Clear code

Character color 0X0 is treated differently inside the ASIC.  The ASIC does not change the line memory if the code read in from the character RAM is 0x0 (treated as clear color).  Thus no character code can be assigned to character color 0X0.  Character color and character code relationship is shown below.

Caution 2:

The value set in this register takes effect immediately.

Table 13. Color Code and Character Color Relation

| Character Color | Color Code |
| --- | --- |
| 0X0 | - |
| 0X1 | SPPLT.01 |
| 0X2 | SPPLT.02 |
| 0X3 | SPPLT.03 |
| 0X0 | - |
| 0X1 | SPPLT.11 |
| 0X2 | SPPLT.12 |
| 0X3 | SPPLT.13 |

Table 14. Parameters and Definitions

| SPPLT.0n (Read/Write possible in K2GE mode. However, no effect on screen.) |  |
| --- | --- |
| Palette code 0, Color code n | Color code set for palette 0. |
| SPPLT.1n (Read/Write possible in K2GE mode. However, no effect on screen.) |  |
| Palette code 1, Color code n | Color code set for palette 1. |
| D.C (Does not exist as RAM) |  |
| Do not care | This register value has no effect. |

The value set in this sprite palette associated with the “K1GE upper palette compatible mode sprite color palette” is the actual colors displayed.

### Color Change and Bit Weight

LSB of the data will always be in D0 and its bit weight is the lowest.  Screen output is similar, with the smallest contrast change being the LSB and the largest the MSB.

The value set in this sprite palette associated with the “K1GE upper palette compatible mode sprite color palette” is the actual colors displayed.

Table 15. MSB and LSB Definition

| MSB | LSB |
| --- | --- |
| D2 | D0 |

## Palette LUT for Scroll Plane 1

Palette code (P.C) defined in the scroll plane 1 VRAM (0X9000～0X97FF) is changed to the value set in this register.  Register format is as follows.

| ADDRESS | D7 | D6 | D5 | D4 | D3 |  | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0X8108 | Access not allowed. (Does not exist as RAM. No specific values when read.) |  |  |  |  |  |  |  |  |
| 0X8109 | D.C (Does not exist as RAM. “0” when read.) |  |  |  |  | SPPLT.01 |  |  |  |
| 0X810A | D.C (Does not exist as RAM. “0” when read.) |  |  |  |  | SPPLT.02 |  |  |  |
| 0X810B | D.C (Does not exist as RAM. “0” when read.) |  |  |  |  | SPPLT.03 |  |  |  |
| 0X810C | Access not allowed. (Does not exist as RAM. No specific values when read.) |  |  |  |  |  |  |  |  |
| 0X810D | D.C (Does not exist as RAM. “0” when read.) |  |  |  |  | SPPLT.11 |  |  |  |
| 0X810E | D.C (Does not exist as RAM. “0” when read.) |  |  |  |  | SPPLT.12 |  |  |  |
| 0X810F | D.C (Does not exist as RAM. “0” when read.) |  |  |  |  | SPPLT.13 |  |  |  |

Caution:

The value set in this register takes effect immediately.

Table 16. Parameters and Definitions

| SC1PLT.0n (Read/Write possible in K2GE mode. However, no effect on screen.) |  |
| --- | --- |
| Palette code 0, Color code n | Color code set for palette 0. |
| SC1PLT.1n (Read/Write possible in K2GE mode. However, no effect on screen.) |  |
| Palette code 1, Color code n | Color code set for palette 1. |
| D.C (Does not exist as RAM) |  |
| Do not care | This register value has no effect. |

The value set in this scroll palette associated with the “K1GE upper palette compatible mode scroll 1 color palette” is the actual colors displayed.

## Palette LUT for Scroll Plane 2

Palette code (P.C) defined in the scroll plane 2 VRAM (0X9800～0X9FFF) is changed to the value set in this register.  Register format is as follows.

| ADDRESS | D7 | D6 | D5 | D4 | D3 |  | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0X8110 | Access not allowed. (Does not exist as RAM. No specific values when read.) |  |  |  |  |  |  |  |  |
| 0X8111 | D.C (Does not exist as RAM. “0” when read.) |  |  |  |  | SPPLT.01 |  |  |  |
| 0X8112 | D.C (Does not exist as RAM. “0” when read.) |  |  |  |  | SPPLT.02 |  |  |  |
| 0X8113 | D.C (Does not exist as RAM. “0” when read.) |  |  |  |  | SPPLT.03 |  |  |  |
| 0X8114 | Access not allowed. (Does not exist as RAM. No specific values when read.) |  |  |  |  |  |  |  |  |
| 0X8115 | D.C (Does not exist as RAM. “0” when read.) |  |  |  |  | SPPLT.11 |  |  |  |
| 0X8116 | D.C (Does not exist as RAM. “0” when read.) |  |  |  |  | SPPLT.12 |  |  |  |
| 0X8117 | D.C (Does not exist as RAM. “0” when read.) |  |  |  |  | SPPLT.13 |  |  |  |

Caution:

The value set in this register takes effect immediately.

Table 17. Parameters and Definitions

| SC2PLT.0n (Read/Write possible in K2GE mode. However, no effect on screen.) |  |
| --- | --- |
| Palette code 0, Color code n | Color code set for palette 0. |
| SC2PLT.1n (Read/Write possible in K2GE mode. However, no effect on screen.) |  |
| Palette code 1, Color code n | Color code set for palette 1. |
| D.C (Does not exist as RAM) |  |
| Do not care | This register value has no effect. |

The value set in this scroll palette associated with the “K1GE upper palette compatible mode scroll 2 color palette” is the actual colors displayed.

## 4-15.  COLOR PALETTE RAM

Color palette RAM is composed of  “1 palette = 16 Bits (4 of which are D.C), and each of the color palette RAM addresses are separated by 16 Bits for functionality.

Color palette RAM data format is listed below.

| ADDRESS | D15 | D14 | D13 | D12 | D11 | D10 | D9 | D8 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0X8200 ~ 0X83FF | D.C (Does not exist as RAM. When read, the value is “0”.) |  |  |  | B_PLT |  |  |  | G_PLT |  |  |  | R_PLT |  |  |  |

Caution 1:

This RAM area is 16 bit access only.  Using 8 bit access will produce unreliable values.

Caution 2:

Values set are immediately reflected.

Caution 3:

The clear color palette code address in “0X8200 ~ 0X837E” (i.e. 0X8200, 0X8208, 0X8210, etc.) is treated as D.C for “D15 ~ D0”, but in actuality, “D11 ~ D0” is treated as reserved RAM.

Caution 4:

The values inside the color palette RAM after reset are all undefined.  It is necessary to initialize these values appropriately.

Caution 5:

Access to color palettes are always 0 WAIT operation possible.  (Even during hardware display write it is 0 WAIT operation possible.)

Table 18. Parameters and Definitions

| R_PLT |  |
| --- | --- |
| RED palette | Defines 4Bit red palette code. (D3 = MSB, D0 = LSB) |
| G_PLT |  |
| GREEN palette | Defines 4Bit red palette code. (D7 = MSB, D4 = LSB) |
| B_PLT |  |
| BLUE palette | Defines 4Bit red palette code. (D11 = MSB, D8 = LSB) |
| D.C (Does not exist as RAM) |  |
| Do not Care | Values set does not have any effect on circuit operation. |

Color palette RAM functions are listed below in address groups.

Table 19. Color Palette RAM Functions in Address Groups

| No. | Address | Valid Mode | Affected Display | Function |
| --- | --- | --- | --- | --- |
| 0 ~63 | 0X8200 ~ 0X827F | K2GE mode | Sprite | K2GE mode sprite color palette |
| 64 ~ 127 | 0X8280 ~ 0X827F | K2GE mode | Scroll 1 | K2GE mode scroll 1 color palette |
| 128 ~ 191 | 0X8300 ~ 0X837F | K2GE mode | Scroll 2 | K2GE mode scroll 2 color palette |
| 192 ~ 207 | 0X8380 ~ 0X839F | K1GE upper palette compatible mode | Sprite | K1GE upper palette compatible mode sprite color palette |
| 208 ~ 223 | 0X83A0 ~ 0X83BF | K1GE upper palette compatible mode | Scroll 1 | K1GE upper palette compatible mode scroll 1 color palette |
| 224 ~ 239 | 0X83C0 ~ 0X83DF | K1GE upper palette compatible mode | Scroll 2 | K1GE upper palette compatible mode scroll 2 color palette |
| 240 ~ 247 | 0X83E0 ~ 0X83EF | K2GE/K1GE upper palette compatible mode | Background | Background color palette |
| 248 ~255 | 0X83F0 ~ 0X827F | K2GE/K1GE upper palette compatible mode | Window | Window color palette |

For further information, please refer to “K2GE Color Palette Table”.

# 5.   K1GEK2GE EXPANDED FUNCTION SUMMARY

The similarities and differences between K1GE and K2GE are summarized in the following sections.

## SUMMARY

K2GE inherits the functionality of K1GE with the adoption of color.  The following functions have been expanded or changed.

- K2GE mode color palette code (4Bits) added to sprite VRAM.  (X8C00 ~ 0X8C3F added.)

- K2GE mode color palette code (4Bits) added to scroll VRAM.  (D4 ~ D1 of odd address which were formally specified as D.C added.)

- Addition of color palette RAM.  (0X8200 ~ 0X83FF added.)

- For operation mode, “K1GE upper palette compatible mode” for K1GE compatibility and “K2GE mode” for maximum color output on board.  (Mode selection register added.)

K2 system software sets the operation mode from the header information of the game cassette.

The following functions have not been changed to maintain compatibility.

- Character RAM data format.  (“00 b” is always treated as clear color.)

- Sprite display function.  (Excluding color palette addition.)

- Scroll display function.  (Excluding color palette addition.)

- Background display function.  (Excluding color palette addition.)

- Window display function.  (Excluding color palette addition.)

- Vint, Hint generation timing.

- All operation timing.  (H time left, V blank time, etc.)

## K2GE MODE COLOR PALETTE RAM ADDRESS CALCULATION

In K2GE mode, the final display color (color palette RAM address) is determined as follows:

## K1GE UPPER PALETTE COMPATIBLE MODE COLOR PALETTE RAM ADDRESS CALCULATION

In K1GE upper palette compatible mode, the final display color (color palette RAM address) is determined as follows:

## BACKGROUND COLOR PALETTE RAM ADRESS CALCULATION

The background final display color (color palette RAM address) is determined as follows:

## WINDOW COLOR PALETTE RAM ADDRESS CALCULATION

The Window boarder final display color (color palette RAM address) is determined as follows:

# 6.   OPERATION TIMING FIGURE

K2GE operation timing is shown below.  (It is the same as K1GE.)

*Access to each RAM area is possible during the Hardware Drawing Period.  Please be aware of the adjustment circuitry delaying the Hardware Drawing Period if software accesses the RAM during this period.  If the total time surpasses “H Time Remaining (approx. 5 S),” Character Over occurs.  (Accessing the registers does not have an effect.  Sprite VRAM, scroll VRAM, and character RAM read/write access invokes the adjustment circuitry.)

H = H_INT generation timing

*The signal generation begins 1 H before the Hardware Drawing Period starts.  (Please be aware H_INT signal is not generated at line 151 and signal generation for the 0th line occurs at the beginning of line 198.)

V = V_INT generation timing

# 7.   COLOR PALETTE TABLE

K2GE color palette table is listed in a separate file.

# 8.   REGISTER TABLE

K2GE register table is listed in a separate file.

DOT ASPECT IS ALWAYS 1:1

Except, there is a possibility of “character over.”  Explanation on page 23.

Common with scroll character.

32character×32character×2planes.

Common with sprite characters.

Blank area is set in 4-11. 2D control register 00WC.

LUT → Look.Up.Table