K1GE Resister Table

1998.11.18 ver. 1.0 K. Takeda

| OS Development | Common Use | Expansion | Register Name | Abbreviation | Summary | R/W | Address | Control | Lock | Display Timing H: Horizontal Blanking | Effective Area | Initial Value | Function |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | X |  | Window H Origin | WBA.H | WD Origin H | W (R) | 8002 | User |  | H |  | “00” |  |
| X | X |  | Window V Origin | WBA.V | WD Origin V | W (R) | 8003 | User |  | H |  | “00” |  |
| X | X |  | Window H Size | WSI.H | WD Size H | W (R) | 8004 | User |  | H |  | “FF” |  |
| X | X |  | Window V Size | WSI.V | WD Size V | W (R) | 8005 | User |  | H |  | “FF” |  |
| X | X |  | Raster Position H | RAS.H | Raster Address H | R | 8008 | User |  | - |  |  | Negation Counter |
| X | X |  | Raster Position V | RAS.V | Raster Address V | R | 8009 | User |  | - |  | - |  |
| X | X |  | 2D Status | C.OVR BLNK | 2D Status | R | 8010 | User |  | - |  | - | D7: Character Over 0 = No, 1= Yes D6: 0 = Display Period, 1 = V Blanking |
| X | X |  | 2D Control | NEG OOWC | 2D Control | W (R) | 8012 | User |  | H |  | “00” | D7: 0 = POS display, 1 = NEG display D2 ~ D0: Outside window frame color CODE |
| X | X |  | H Position Offset | PO.H | Sprite Offset H | W (R) | 8020 | User |  | H |  | “00” |  |
| X | X |  | V Position Offset | PO.V | Sprite Offset V | W (R) | 8021 | User |  | H |  | “00” |  |
| X | X |  | Switch Scroll Plane Priority | P.F | Scroll Priority | W (R) | 8030 |  |  |  |  | “00” | D7: 0 = Scroll 1high priority, 1 = Scroll 2 high priority |
| X | X |  | Scroll Plane 1 H Scroll Offset | S1SO.H | Scroll 1 Offset H | W (R) | 8032 | User |  | H |  | “00” |  |
| X | X |  | Scroll Plane 1 V Scroll Offset | S1SO.V | Scroll 1 Offset V | W (R) | 8033 | User |  | H |  | “00” |  |
| X | X |  | Scroll Plane 2 H Scroll Offset | S2SO.H | Scroll 2 Offset H | W (R) | 8034 | User |  | H |  | “00” |  |
| X | X |  | Scroll Plane 2 V Scroll Offset | S2SO.V | Scroll 2 Offset V | W (R) | 8035 | User |  | H |  | “00” |  |
| X | X |  | Palette LUT01 for Sprites | SPPLT.01 | Sprite Palette 01 | W (R) | 8101 | User |  | Immediate |  | “07” |  |
| X | X |  | Palette LUT02 for Sprites | SPPLT.02 | Sprite Palette 02 | W (R) | 8102 | User |  | Immediate |  | “07” |  |
| X | X |  | Palette LUT03 for Sprites | SPPLT.03 | Sprite Palette 03 | W (R) | 8103 | User |  | Immediate |  | “07” |  |

(continued)

| OS Development | Common Use | Expansion | Register Name | Abbreviation | Summary | R/W | Address | Control | Lock | Display Timing | Effective Area | Initial Value | Function |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | X |  | Palette LUT11 for Sprites | SPPLT.11 | Sprite Palette 11 | W (R) | 8105 | User |  | Immediate |  | “07” |  |
| X | X |  | Palette LUT12 for Sprites | SPPLT.12 | Sprite Palette 12 | W (R) | 8106 | User |  | Immediate |  | “07” |  |
| X | X |  | Palette LUT13 for Sprites | SPPLT.13 | Sprite Palette 13 | W (R) | 8107 | User |  | Immediate |  | “07” |  |
| X | X |  | Palette LUT01 for Scroll Plane 1 | SC1PLT.01 | Scroll 1 Palette 01 | W (R) | 8109 | User |  | Immediate |  | “07” |  |
| X | X |  | Palette LUT02 for Scroll Plane 1 | SC1PLT.02 | Scroll 1 Palette 02 | W (R) | 810A | User |  | Immediate |  | “07” |  |
| X | X |  | Palette LUT03 for Scroll Plane 1 | SC1PLT.03 | Scroll 1 Palette 03 | W (R) | 810B | User |  | Immediate |  | “07” |  |
| X | X |  | Palette LUT11 for Scroll Plane 1 | SC1PLT.11 | Scroll 1 Palette 11 | W (R) | 810D | User |  | Immediate |  | “07” |  |
| X | X |  | Palette LUT12 for Scroll Plane 1 | SC1PLT.12 | Scroll 1 Palette 12 | W (R) | 810E | User |  | Immediate |  | “07” |  |
| X | X |  | Palette LUT13 for Scroll Plane 1 | SC1PLT.13 | Scroll 1 Palette 13 | W (R) | 810F | User |  | Immediate |  | “07” |  |
| X | X |  | Palette LUT01 for Scroll Plane 2 | SC2PLT.01 | Scroll 2 Palette 01 | W (R) | 8111 | User |  | Immediate |  | “07” |  |
| X | X |  | Palette LUT02 for Scroll Plane 2 | SC2PLT.02 | Scroll 2 Palette 02 | W (R) | 8112 | User |  | Immediate |  | “07” |  |
| X | X |  | Palette LUT03 for Scroll Plane 2 | SC2PLT.03 | Scroll 2 Palette 03 | W (R) | 8113 | User |  | Immediate |  | “07” |  |
| X | X |  | Palette LUT11 for Scroll Plane 2 | SC2PLT.11 | Scroll 2 Palette 11 | W (R) | 8115 | User |  | Immediate |  | “07” |  |
| X | X |  | Palette LUT12 for Scroll Plane 2 | SC2PLT.12 | Scroll 2 Palette 12 | W (R) | 8116 | User |  | Immediate |  | “07” |  |
| X | X |  | Palette LUT13 for Scroll Plane 2 | SC2PLT.13 | Scroll 2 Palette 13 | W (R) | 8117 | User |  | Immediate |  | “07” |  |
| X | X |  | Background Color | BG | Background Color | W (R) | 8118 | User |  | H |  | “07” | Using Background Color, the values are fixed as follows: D7 = “1”, D6 =”0” |