NEOGEO    OCKET

##### FLASH WRITER REFERENCE MANUAL

- (rel. 0.2)

# REVISION HISTORY

rel. 0.1		Initial release						1998/07/28

rel. 0.2		Page 5 typing correction, Page 6 data load method correction	1998/07/28

- TABLE OF CONTENTS

REVISION HISTORY	2

PREFACE	4

FLASH WRITER OUTLINE	5

FLASH WRITER OPERATION INSTRUCTIONS	6

# PREFACE

The materials covered in this reference manual is provisional.  There is the possibility of future updates.

# FLASH WRITER OUTLINE

- This software uses the game development board to write to the flash cassette.

Currently, the devices supported are the 4M, 8M and 16M devices.  Due to the game development board specification, data may be written to 15M of the 16M device.  Please be aware that data may not be written to the latter 1M of the 16M.

# FLASH WRITER OPERATION INSTRUCTIONS

Controller Keys

Cursor key:	Used to choose one of the selections.

A button:	Used to confirm selection.

B button:	Used to cancel selection.

Menu Selections

SOURCE ADDR:	Defines the source start address.  Range:  0x210000~0x3effff.

FLASH ADDR:	Defines the write address of the flash.  Range:  0x800000~0x9effff.

DATA SIZE:	Defines the data size.  Range:  0x000000~0x200000.  Because the 16th M of memory may not be written, in reality the last address is 0x1e0000.

ERS & WRT:	Erases the requested DATA SIZE, address specified by FLASH ADDR, and then writes the data with data size DATA SIZE starting from SOURCE ADDR.

WRITE START:	Writes the data with data size DATA SIZE to the address FLASH ADDR from the source with start address SOURCE ADDR.

ERASE START:	Erases DATA SIZE worth of data from the starting address FLASH ADDR.

Data Load Method

Please create the data to be written in Motorola S s24 format.  Give an offset of 0x010000 when using the EMUSE to load.  Then please load and run the FLASH WRITER software as with other software.