NEOGEO    OCKET

## NEOGEO POCKET COLOR SUMMARY

OUTLINE SPECIFICATION

- March 19th 1999 release as a color version of NEOGEO POCKET.

- Compact body with “HAND” held system in mind.

- Uses AA and lithium batteries.  Approximately 40 hours of continued use possible.

- Uses 16 bit CPU for higher performance.

- 2D circuit to allow variety of graphic displays (146 colors out of 4096 colors).

- 20 colors out of 4096 colors display mode (NEOGEO POCKET compatible mode).

- 146 colors out of 4096 colors display mode (NEOGEO POCKET COLOR mode).

- Reflective TFT LCD used.

- To allow ease of control even for fighting games, joystick-like “click” feel controller used.

- To increase possibility of games, calendar IC on board.  (Alarm statup possible.)

- Communication function available.  (2 player)

- With the use of flash memory, it is possible to create RPG type games that require memory.

- NEOGEO POCKET compatible.  (Upper, lower compatible.)

HARDWARE

### CPU

- TOSHIBA 16 bit CPU (TLCS 900H) core.

- 12 KB main RAM.

- 64 KB boot ROM.

- Maximum 6.144 MHz main clock gear.

- With the clock gear control, it is possible to manipulate power usage for power conservative games.

- SIO 1 channel maximum 19200 bps.

- Interrupt controller

Interrupt cause:  timer, horizontal blanking, vertical blanking, serial communication, sound CPU, alarm startup, and micro DMA end.

- Timer controller 8 bit 4 channels

- RTC (Real Time Clock)

1991 to 2090.  Year, month, day, hour, minute and seconds.

GRAPHICS

- Resolution 160 x 152 dot.

- Virtual screen 256 x 256 dot.

- Character 8 x 8 dot 4 (1 clear) color out of 4096 colors.

- 16 palettes per plane with total of 48 palettes.

- Window display function area changeable.

- Negative-positive flip function possible for full screen.

- NEOGEO POCKET compatible mode (20 colors out of 4096) *Using Monochrome software.

### SPRITES

- H, V flip possible per sprite.

- Display priority level set.

Possible to set sprite display priority with respect to scroll plane for each sprite.

- Position Chain.

Able to inherit position of previous sprite.

- Sprite position correction.

Can scroll all of sprites on sprite plane.

- Maximum 64 sprites displayable (in 1 frame).

### SCROLL

- 2 scroll planes.

- Priority change possible for the 2 planes

- H, V flip possible on character basis

SOUND

- 8 bit CPU (Z80) core.

- 3.072 MHz main clock.

- 4KB main RAM (common with main CPU).

- 3 square wave tones, 1 noise modified tone, 4 tone simultaneous output.

- 6 bit DAC x 2 on board.

- Stereo sound (with headphones).

### I/O

- 8 direction lever.

- A, B and Option buttons.

- Audio volume.

- Headphone jack.

- DC jack.

- 5 pin serial port.

- External cassette port.

### GAME CASSETTE

- Maximum 2 MB (16 Mbit) linear access possible.

- 4 M, 8 M, 16 Mbit flash memory (data save can change from game to game).

### EXPANSION HARDWARE

- Wireless control adapter (provisional)

“NEOGEO POCKET WIRELESS COMMUNICATION UNIT”

Communication distance approximately 10 m.

Maximum communication units:  64 (non-synchronized), 12 (synchronized).

Synchronized and non-synchronized mix possible.

One AAA battery used.

SOFTWARE

### SYSTEM SOFTWARE

- System call to support hardware control on board in 64 KB boot ROM.

### INTERNAL APPLICATION  *Usable when game cassette is not inserted.

- Calendar:  1991 to 2090 possible.

- 12 country world clock (summer time function).

- Daily horoscope (money, health, and love).

- Daily alarm.

- Japanese and English compatible.

### SYSTEM LIBRARY

- Library software distributed with routines to be used in games.

### DREAMCAST

- Using 5 pin serial connector, it is possible to communicate with the SEGA Dreamcast.  (Special cable to be sold.)

DEVELOPMENT EQUIPMENT

### PROGRAM DEVELOPMENT

- Internal ROM emulator program development target available.

- Midoriya Electronics ROM emulator EMUSE used.

- Windows 95 and Windows NT 4 compatible.

- Development language:  C, assembler (Z80 like mnemonic).

### SOUND DEVELOPMENT

- PC-AT compatible sound board available (ISA bus).

- Standard MIDI file format 1 compatible.

### GRAPHIC DEVELOPMENT

- Simple character format.  (No plans to supply graphic tools.)

NEOGEO POCKET (MONOCHROME)

### COMPATIBLITY

- 8 level monochrome LCD.

- Character 8 x 8 dot, 4 levels out of 8 levels.

- Each plane has 2 palettes.  (Total of 6 palettes.)

- Contrast volume on board.

- 2 AAA batteries used.  (Continuous play of approximately 20 hours.)