NEOGEO    OCKET

SERIAL COMMUNICATION REFERENCE MANUAL

- (Rel. 1.0)

# REVISION HISTORY

| rel 0.1 | Initial release | 1998/05/19 |
| --- | --- | --- |
| rel 0.2 | Change of variables and addition of sample program | 1998/05/21 |
| rel 0.3 | Addition of caution notes, fixed variables, of subroutines, and sample program corrections | 1998/06/11 |
| rel 0.8 | Manual and sample program corrections | 1998/08/21 |
| rel 1.0 | Official release | 1998/12/09 |

- TABLE OF CONTENTS

SERIAL COMMUNICATION REFERENCE MANUAL	1

REVISION HISTORY	2

PREFACE	4

SERIAL COMMUNICATION OUTLINE	5

Serial Communication Specification	5

Serial Communication BIOS	5

SERIAL COMMUNICATION BIOS VECTOR TABLE	6

CAUTION NOTES REGARDING SERIAL COMMUNICATION BIOS USE	6

VECT_COMINIT  Serial Communication Initialization	7

VECT_COMSENDSTART  Commence Transmission	8

VECT_COMRECIVESTART  Commence Reception	9

VECT_COMCREATEDATA  Creating Communication Data	10

VECT_COMGETDATA  Obtain Data Recieved	11

VECT_COMONRTS  RTS Signal Transmission Permission	12

VECT_COMOFFRTS RTS Signal Transmission Prohibition	13

VECT_COMSENDSTATUS  Obtain Transmission Status	14

VECT_COMRECIVESTATUS  Obtain Reception Status	15

VECT_COMCREATEBUFDATA  Transmission Data Buffer Create	16

VECT_COMGETBUFDATA  Reception Data Buffer Write	17

SAMPLE PROGRAM	18

# PREFACE

This reference is written with the understanding that the developer will use assembly level language.  If C language is to be used for development, please refer to the manual for the Toshiba development language software package.

# SERIAL COMMUNICATION OUTLINE

Listed below are the BIOS subroutines which support serial communication for the NEOGEO POCKET.  Vector numbers are set in the header file “COM.INC.”

## Serial Communication Specification

| Communication speed | : | 19200bps |
| --- | --- | --- |
| Bits | : | 8 bit |
| Parity | : | None |
| Stop bit | : | 1 bit |
| Handshake flow | : | handshake with CTS, RTS signals |

## Serial Communication BIOS

Serial communication BIOS calls are made with the same method as system calls.  Please refer to “SYSTEM CALL REFERENCE MANUAL.”

# SERIAL COMMUNICATION BIOS VECTOR TABLE

| VECT_COMINIT | : | Serial Communication Initialization BIOS |
| --- | --- | --- |
| VECT_COMSENDSTART | : | Commence Transmission BIOS |
| VECT_COMRECIVESTART | : | Commence Reception BIOS |
| VECT_COMCREATEDATA | : | Create Communication Data BIOS |
| VECT_COMGETDATA | : | Obtain Communication Data BIOS |
| VECT_COMONRTS | : | RTS Signal Transmission Permission BIOS |
| VECT_COMOFFRTS | : | RTS Signal Transmission Prohibition BIOS |
| VECT_COMSENDSTATUS | : | Obtain Transmission Status BIOS |
| VECT_COMRECIVESTATUS | : | Obtain Reception Status BIOS |
| VECT_COMCREATEBUFDATA | : | Transmission Data Buffer Create BIOS |
| VECT_COMGETBUFDATA | : | Reception Data Buffer Write BIOS |

*Detailed information of each BIOS is stated in the following pages.

## CAUTION NOTES REGARDING SERIAL COMMUNICATION BIOS USE

*Serial data communication transmission and reception are done as interrupts.  Any operation which requires a long operation period and suffers from multiple interrupts such as V-BLANK should have VECT_COMOFFRTS before the operation and VECT_COMONRTS before reti command.  Other operations which require minimal time (ex. H-BLANK) should have similar coding as above.

*During VECT_COMGETDATA and VECT_COMGETBUFDATA operations, transmission and reception interrupts are not allowed.  Thus if long operation interrupts (ex. V-BLANK) occur during these operational BIOS calls, there is a possibility of a buffer overrun error and care must be taken to prevent such situations from arising.  There is no restriction concerning where these commands must be placed, but it is recommended that these commands be placed after V-BLANK or at the beginning of the MAIN.  If these commands are used during interrupts, please push the value in register bank 3 to stack.

*When using the serial communication BIOS (all BIOS vector calls), please do not use any vector call with software interrupt 1 (swi 1).  Please use the system library SYSTEM_CALL subroutine.

*The serial communication speed is dependent on the clock gear.

# VECT_COMINIT  Serial Communication Initialization

DEFINITION:

Defines ports necessary for serial communication.  Please use this call when serial communication is needed.

INPUT VARIABLES:

None

OUTPUT VARIABLES:

None

RETURN VALUES:

None

CONSTANTS:

None

# VECT_COMSENDSTART  Commence Transmission

DEFINITION:

This is the BIOS to commence transmission of data created in the internal buffer.  Please use this call once after the data is created in the buffer.

INPUT VARIABLES:

None

OUTPUT VARIABLES:

None

RETURN VALUES:

None

CONSTANTS:

None

# VECT_COMRECIVESTART  Commence Reception

DEFINITION:

This is the BIOS to obtain permission to commence reception.  Please use this call once to allow reception.

INPUT VARIABLES:

None

OUTPUT VARIABLES:

None

RETURN VALUES:

None

CONTSTANTS:

None

# VECT_COMCREATEDATA  Creating Communication Data

DEFINITION:

Creates transmission data in the internal (system) buffer.  The buffer is a 64 byte loop buffer.  Please use this BIOS before VECT_COMSENDSTART is called.

INPUT VARIABLES:

| register rb3 | : | 1 byte transmission data |
| --- | --- | --- |

OUTPUT VARIABLES:

None

RETURN VALUES:

| register ra3 | : | Transmission buffer status flag |
| --- | --- | --- |

CONSTANTS:

| COM_BUF_OVER | : | Transmission buffer over |
| --- | --- | --- |
| COM_BUF_OK | : | Transmission buffer normal |

# VECT_COMGETDATA  Obtain Data Recieved

DEFINITION:

1 byte of data received in the internal (system) buffer is obtained.

INPUT VARIABLES:

None

OUTPUT VARIABLES:

| register rb3 | : | 1 byte data received |
| --- | --- | --- |

RETURN VALUES:

| register ra3 | : | Reception buffer status flag |
| --- | --- | --- |

CONSTANTS:

| COM_BUF_ EMPTY | : | Reception buffer empty |
| --- | --- | --- |
| COM_BUF_OK | : | Reception buffer normal |

# VECT_COMONRTS  RTS Signal Transmission Permission

DEFINITION:

RTS signal is set to low to allow transmission from others units.

INPUT VARIABLES:

None

OUTPUT VARIABLES:

None

RETURN VALUES:

None

CONSTANTS:

None

# VECT_COMOFFRTS RTS Signal Transmission Prohibition

DEFINITION:

RTS signal is set to high to prohibit transmission from other units.

INPUT VARIABLES:

None

OUTPUT VARIABLES:

None

RETURN VALUES:

None

CONSTANTS:

None

# VECT_COMSENDSTATUS  Obtain Transmission Status

DEFINITION:

Buffer over flag and data count in the internal buffer is obtained.  Reserved ERROR status bits do not have set values.

INPUT VARIABLES:

None

OUTPUT VARIABLES:

None

RETURN VALUES:

| register rwa3 | : | ERROR status bits & transmission buffer count |
| --- | --- | --- |

CONSTANTS:

| COM_BUFOVERERROR | : | Buffer over flag |
| --- | --- | --- |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Transmission buffer count

Transmission buffer over

reserved

reserved

reserved

reserved

reserved

reserved

reserved

# VECT_COMRECIVESTATUS  Obtain Reception Status

DEFINITION:

Transmission errors which have occurred up till now and the data count in the buffer is obtained.  After this BIOS is called, communication error flag is cleared.

Reserved area ERROR status bits are 0.

INPUT VARIABLES:

None

OUTPUT VARIABLES:

None

RETURN VALUES:

| register rwa3 | : | ERROR status bits & reception buffer count |
| --- | --- | --- |

CONSTANTS:

| COM_BUFOVERERROR | : | Buffer over flag |
| --- | --- | --- |
| COM_FLAMEERROR | : | Framing error flag |
| COM_PARITYERROR | : | Parity error flag |
| COM_OVERRUNERROR | : | Over run error flag |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Reception buffer count

Reception buffer over

reserved

Framing error

reserved

reserved

Parity error

Over run error

reserved

# VECT_COMCREATEBUFDATA  Transmission Data Buffer Create

DEFINITION:

Address and size of the transmission data stored in user buffer is defined and stored in the system buffer.  This is a “block transfer” version of VECT_COMCREATEDATA.  It is quicker than using a loop with VECT_COMCREATEDATA.

INPUT VARIABLES:

| register xhl3 | : | Transmission data buffer address |
| --- | --- | --- |
| register rb3 | : | Transmission data buffer size |

OUTPUT VARIABLES:

| register xhl3 | :: | When the buffer works normally, points to the next data address. If data has been left from previous transmission, points to the data. |
| --- | --- | --- |
| register rb3 | : | Number of data left |

RETURN VALUES:

None

CONSTANTS:

| COM_BUF_OK | : | Buffer transfer normal (no transfer left over) |
| --- | --- | --- |

# VECT_COMGETBUFDATA  Reception Data Buffer Write

DEFINITION:

Address and size of the reception data wanted in the user buffer is defined and obtained from the system buffer. This is a “block transfer” version of VECT_COMGETDATA. It is quicker than using a loop with VECT_COMGETDATA.

INPUT VARIABLES:

| register xhl3 | : | Reception data buffer address |
| --- | --- | --- |
| register rb3 | : | Reception data buffer size |

OUTPUT VARIABLES:

| register xhl3 | : | Points to the address where the next data will be stored. |
| --- | --- | --- |
| register rb3 | : | Number of data left |

RETURN VALUES:

None

CONSTANTS:

| COM_BUF_OK | : | Buffer obtained normally (no reception left over) |
| --- | --- | --- |

# SAMPLE PROGRAM

;*************************************************************************

;*                                           							*

;*	SERIAL COMMUNICATION SAMPLE PROGRAM			*

;*									*

;*	com_sample.asm							*

;*									*

;*	First Edition	1998/05/21	NEOGEO POCKET PROJECTS	*

;*	Second Edition	1998/06/11	NEOGEO POCKET PROJECTS	*

;*		Sample program and vector calls modified			*

;*	Third Edition	1998/08/21	NEOGEO POCKET PROJECTS	*

;*		Bug fixes							*

;*									*

;*************************************************************************

;****************************************

$MAXIMUM				;

module  COM_SAMPLE		;

;****************************************

;  ---------------------------------

$include	"sub_ext.inc"

$include	"data_equ.inc"

$include	"k1head.inc"

$include	"glbwork.inc"

$include	"com.inc"

PROG	section code large			;

public		SYSTEM_CALL

;*************************************************************************

;*									*

;*	MAIN_Init							*

;*									*

;*	[FUNCTION]        INITIALIZATION PROGRAM			*

;*									*

;*	First Edition	1998/05/21	NEOGEO POCKET PROJECTS	*

;*	Second Edition	1998/06/11	NEOGEO POCKET PROJECTS	*

;*      		Vector calls modified					*

;*************************************************************************

MAIN_Init:

;~ OTHER INITIALIZATION ~

ldb		rw3,VECT_COMINIT	;serial communication initialization

calr		SYSTEM_CALL		;vector call

;~ OTHER INITIALIZATION ~

ret

;*************************************************************************

;*									*

;*	MAIN_main							*

;*									*

;*	[FUNCTION]        MAIN PROGRAM					*

;*									*

;*	First Edition	1998/05/21          NEOGEO POCKET PROJECTS   	*

;*	Second Edition	1998/06/11          NEOGEO POCKET PROJECTS   	*

;*		Vector calls modified					*

;*									*

;*************************************************************************

MAIN_main:

;~ OTHER MAIN OPERATIONS ~

ldb	rw3,VECT_COMRECIVESTART	;set reception BIOS permission

calr	SYSTEM_CALL			;vector call

;*****************************************************************

;* CAUTION							*

;*  Please make sure both units status for “power on” may be assessable.	*

;*  If permission is given with one unit off, garbage data will be received	*

;*  and undesired multiple interrupts will result.				*

;*  THIS MAY BE CHANGED AT A LATER DATE.			*

;*****************************************************************

;~ OTHER MAIN OPERATIONS ~

;*  CREATING TRANSMISSION DATA  *

calr	MAIN_CreateData

;~ OTHER MAIN OPERATIONS ~

;*  OBTAINING RECEPTION DATA  *

calr	MAIN_GetData

;~ OTHER MAIN OPERATIONS ~

;* WAITING FOR V-Blank INTERRUPT *

ldb	(vwait),0xff

vwait_loop:

cpb	(vwait),0

jr		nz,vwait_loop

jr		MAIN_main

;*************************************************************************

;*									*

;*	MAIN_CreateData							*

;*									*

;*	[FUNCTION]	SUBROUTINE TO CREATE TRANSMISSION BUFFER	*

;*									*

;*	First Edition	1998/05/21	NEOGEO POCKET PROJECTS	*

;*	Second Edition	1998/06/11	NEOGEO POCKET PROJECTS	*

;*		Vector calls modified					*

;*	Third Edition	1998/08/21	NEOGEO POCKET PROJECTS	*

;*		Bug fixes							*

;*									*

;*************************************************************************

MAIN_CreateData:

lda	xix,data_send_addr			;obtaining data address

ldb	b,(data_send_size)			;obtaining data size

data_create:

ldb	a,(xix+)

ldb	rb3,a				;transferring 1 byte data

dec	1,b				;decrementing data size

cpb	b,0				;data end comparison

jr	eq,data_create_end			;end buffer creation if data end

data_create_over:

ldb	rw3,VECT_COMCREATEDATA	;set transmission data creation BIOS

calr	SYSTEM_CALL			;vector call

cpb	ra3,COM_BUF_OVER		;buffer over?

jr	eq,data_create_over			;if buffer over, wait until over

jr	data_create			;if normal operation, create next data

data_create_end:

ldb	rw3,VECT_COMSENDSTART	;set commence transmission BIOS

calr	SYSTEM_CALL			;vector call

ret

;*************************************************************************

;*CAUTION								*

;*	If transmission data exceeds 64 bytes in one transfer, please include the	*

;*	the commence transmission BIOS inside the loop.  Because commence	*

;*	transmission calls are ignored while data exists in the buffer, the call may	*

;*	be made numerous times without problems.				*

;*************************************************************************

;*************************************************************************

;*									*

;*	MAIN_GetData							*

;*									*

;*	[FUNCTION]	DATA RECEPTION SUBROUTINES			*

;*									*

;*	First Edition	1998/05/21	NEOGEO POCKET PROJECTS	*

;*	Second Edition	1998/06/11	NEOGEO POCKET PROJECTS	*

;*		Vector calls modified					*

;*	Third Edition	1998/08/21	NEOGEO POCKET PROJECTS	*

;*		Bug fixes							*

;*									*

;*************************************************************************

MAIN_GetData:

ldb	rw3,VECT_COMRECIVESTUTAS	;set obtain reception status BIOS

calr	SYSTEM_CALL			;vector call

ldw	wa,rwa3				;move return value to local

;mask error bit

andw	wa,COM_FLAMEERROR | COM_PARITYERROR | COM_OVERRUNERROR

jr	nz,data_get_error			;error operation

lda	xix,data_recive_addr		;obtain address of data to be stored

ldb	b,(data_recive_size)			;obtain data size

data_get:

dec	1,b				;decrementing data size

cpb	b,0				;data end comparison

ret	eq				;end data reception if data end

data_get_empty:

ldb	rw3,VECT_COMGETDATA		;set obtaining data recieved BIOS

calr	SYSTEM_CALL			;vector call

cpb	ra3,COM_BUF_EMPTY		;buffer empty?

jr	eq,data_get_empty			;wait for reception if buffer empty

ldb	a,rb3

ldb	(xix+),a				;obtain data received

jr	data_get				;if normal operation, obtain next data

data_get_error:

;~ ERROR OPERATION ~

ret

;*************************************************************************

;* (Example of operation during interrupt)					*

;*									*

;*	INT_V_Blank							*

;*									*

;*	[FUNCTION]	V-Blank interrupt					*

;*									*

;*	First Edition	1998/05/21	NEOGEO POCKET PROJECTS	*

;*	Second Edition	1998/06/11	NEOGEO POCKET PROJECTS	*

;*		Vector calls modified					*

;*									*

;*************************************************************************

INT_V_Blank:

ldb	rw3,VECT_COMOFFRTS	;set RTS signal trans. prohibition BIOS

calr	SYSTEM_CALL		;vector call

;~ OPERATION DURING V-blank ~

ldb	rw3,VECT_COMONRTS	;set RTS signal trans. permission BIOS

calr	SYSTEM_CALL		;vector call

reti

;*************************************************************************

;*									*

;*	MAIN_CreateData							*

;*									*

;*	[FUNCTION]	TRANSMISSION BUFFER CREATION SUBROUTINE	*

;*			USING VECT_COMCREATEBUFDATA		*

;*									*

;*	First Edition	1998/06/11	NEOGEO POCKET PROJECTS	*

;*	Second Edition	1998/08/21	NEOGEO POCKET PROJECTS	*

;*		Bug fixes							*

;*									*

;*************************************************************************

MAIN_CreateData:

lda	xhl,data_send_addr			;obtaining data address

ldl	xhl3,xhl				;obtaining data address

ldb	b,(data_send_size)			;obtaining data size

ldb	rb3,b				;obtaining data size

data_create:

;set transmission data creation storing buffer BIOS

ldb	rw3,VECT_COMCREATEBUFDATA

calr	SYSTEM_CALL			;vector call

cpb	rb3,COM_BUF_OK		;normal? (no transfer data left over)

jr	eq,data_create_end			;if normal, jump to operation end

ldb	rw3,VECT_COMSENDSTART	;set commence trans. BIOS

calr	SYSTEM_CALL			;vector call

jr	data_create			;loop until no transfer data left over

data_create_end:

ldb	rw3,VECT_COMSENDSTART	;set commence trans. BIOS

calr	SYSTEM_CALL			;vector call

ret

;*************************************************************************

;*									*

;*	MAIN_GetData							*

;*									*

;*	[FUNCTION]	RECEPTION BUFFER SUBROUTINE			*

;*			USING VECT_COMGETBUFDATA			*

;*									*

;*	First Edition	1998/06/11	NEOGEO POCKET PROJECTS	*

;*	Second Edition	1998/08/21	NEOGEO POCKET PROJECTS	*

;*		Bug fixes							*

;*									*

;*************************************************************************

MAIN_GetData:

ldb	rw3,VECT_COMRECIVESTATUS	;set reception status BIOS

calr	SYSTEM_CALL			;vector call

ldw	wa,rwa3				;move return value to local

;mask error bit only

andw	wa,COM_FLAMEERROR | COM_PARITYERROR | COM_OVERRUNERROR

jr	nz,data_get_error			;error operation

lda	xhl,data_recive_addr		;obtain address to store

lda	xhl3,xhl				;obtain address to store

ldb	b,(data_recive_size)			;obtain data size

ldb	rb3,b				;obtain data size

data_get:

;set reception data buffer writing BIOS

ldb	rw3,VECT_COMGETBUFDATA

calr	SYSTEM_CALL			;vector call

cpb	rb3,COM_BUF_OK		;normal operation? (no data left over)

ret	eq				;if normal operation, return

jr	data_get				;loop until no data left over

data_get_error:

;~ ERROR OPERATION ~

ret