NEOGEO    OCKET

- SYSTEM CALL REFERENCE MANUAL

- SNK CORPORATON

- 1998.10.21 Release 1.0

- TABLE OF CONTENTS

PREFACE	3

SYSTEM CALL OUTLINE	4

USING SYSTEM CALLS	4

USE WITH SUBROUTINE CALLS	4

USE WITH SOFTWARE INTERRUPTS	4

SYSTEM CALL VECTOR TABLE	5

EXAMPLE 1	5

EXAMPLE 2	5

SYSTEM PROGRAM VERSION USE CONDITIONS	6

VECTOR DEFINITIONS	7

VECT_SHUTDOWN	7

VECT_CLOCKGEARSET	7

VECT_INTLVSET	8

VECT_RTCGET	8

VECT_ALARMSET	9

VECT_ALARMDOWNSET	9

VECT_SYSFONTSET	10

VECT_FLASHWRITE	11

VECT_FLASHALLERS	11

VECT_FLASHERS	11

VECT_FLASHPROTECT	12

VECT_GEMODESET	13

REVISION HISTORY	14

# PREFACE

Materials covered in this manual are based on system program requirements of the NEOGEO POCKET.  Please understand that there is the possibility of updates to the materials and thus the user program needs to be corrected accordingly.

This manual assumes development will be in assembly language.  If C language is to be used for development, please refer to the manual supplied with the Toshiba Development package.

# SYSTEM CALL OUTLINE

These are subroutines prepared as part of the system to allow changes to the settings managed by the system and special operations such as accessing the flash memory.

Each operation has a specific vector number associated with it and the vector numbers are defined in the header file SYSTEM.INC.

## USING SYSTEM CALLS

There are two different methods for using the system calls.  Please choose according to the need and operation necessary.

## USE WITH SUBROUTINE CALLS

First set the vector number in register W (RW3) of register bank 3 (register bank for the system).  Next set the necessary parameters specific to the operation.  And finally, call the subroutine SYSTEM_CALL.

Because this method does not use any interrupts unlike the method below, it does not hinder other interrupts.  If system calls are to be made while using horizontal blanking interrupt (H-int), or serial communication interrupts are used, this method is recommended.

However, all interrupts are prohibited during system calls related to flash memory management.

To use the SYSTEM_CALL subroutine, it is necessary to link the system library.  Please refer to “SYSTEM LIBRARY REFERENCE MANUAL” on how to link to the library.

## USE WITH SOFTWARE INTERRUPTS

First set the vector number in register W (RW3) of register bank 3 (register bank for the system).  Next set the necessary parameters specific to the operation.  And finally, use software interrupt 1 (SWI 1).

This method prohibits other interrupts (DI).  Because of this, it is not affected by most of the other interrupts.  If it is preferable not to have interrupts (such as V-int) generated during an operation, so this method is recommended.  Specifically, please use the VECT_SHUTDOWN operation during shutdown and flash memory related operations (VECT_FLASHWRITE, VECT_FLASHERS, and VECT_FLASHALLERS).

Conversely, this method is capable of changing the interrupt generation timing of other interrupts.  Using this method during time crucial operations (such as H-int and serial communication) is not recommended.

# SYSTEM CALL VECTOR TABLE

| VECT_SHUTDOWN | ;Shutdown (power off) |
| --- | --- |
| VECT_CLOCKGEARSET | ;Changing CPU clock speed |
| VECT_INTLVSET | ;Interrupt level setting |
| VECT_RTCGET | ;Real time clock  obtain time |
| VECT_ALARMSET | ;Real time clock  alarm setting during game |
| VECT_ALARMDOWNSET | ;Real time clock  unit start up alarm |
| VECT_SYSFONTSET | ;System font setting |
| VECT_FLASHWRITE | ;Flash memory  data write |
| VECT_FLASHALLERS | ;Flash memory  erase all blocks |
| VECT_FLASHERS | ;Flash memory  erase block specified |
| VECT_FLASHPROTECT | ;Flash memory  protect block specified |
| VECT_GEMODESET | ;color LCD specific  set color mode |

*Each subroutine requires different variables.  Please refer to vector definitions for details.

## EXAMPLE 1

 Lowering CPU clock speed

ldb	rw3, VECT_CLOCKGEARSET	; change clock gear

ldb	rb3,0x04				; clock speed to the lowest 0x04

ldb	rc3,0x00				; no auto regeneration with control panel input

call	SYSTEM_CALL			; call system call subroutine

## EXAMPLE 2

Writing data to flash memory

ldb	rw3, VECT_FLASHWRITE		; writing flash memory

ldb	ra3,0x00				;selecting flash memory card (0x200000)

ldw	rbc3,0x02				; number of transfer (0x200）

ldl	xhl3,0x6600			; from address (RAM for example)

ldl	xde3,0x70000			; to address (0x270000 in this example)

swi	1				; software interrupt 1

# SYSTEM PROGRAM VERSION USE CONDITIONS

Some of the system calls are only valid for use in the color LCD system program.  The definitions following this page are noted to specify applicability in monochrome LCD system program or color LCD system program.

Please look in “OS_Version” of the system work.  If the environment is a monochrome LCD system program, please do not use system calls only valid in color LCD environment.

Please refer to SYSTEM WORK REFERENCE MANUAL (rel 0.9 or later) for further information on “OS_Version”.

# VECTOR DEFINITIONS

Please use register bank 3 (system register) for all registers specified by the parameters.

| ## VECT_SHUTDOWN | Monochrome |  | Color |  |
| --- | --- | --- | --- | --- |

Parameter	None

Return value	None

Powers off the NEOGEO POCKET unit.

The power on/off of the NEOGEO POCKET is managed by the system.  When a request from the system to power off the unit is received, the game program must initiate shutdown by calling this subroutine.  Please refer to the SYSTEM WORK REFERENCE MANUAL for details about shutdown.

Power off request from the system is generated when the following occurs:

- Main battery output level drops considerably

- When the system has been left ON for a long time (no panel switch input) and auto power off system is being used

| ## VECT_CLOCKGEARSET | Monochrome |  | Color |  |
| --- | --- | --- | --- | --- |

Parameter	RB3………CPU clock speed (0x00 ～ 0x04)

RC3………auto clock speed regeneration with panel switch input

(0: No	other than 0:  Yes)

Return value	None

Destroys	None

Changes the CPU clock speed of NEOGEO POCKET.

The CPU operation clock speed is highest with 0 and as the values increase it slows down.

If auto clock speed regeneration is on, any input from the panel switch will automatically set the clock speed to the highest value.

In order to lower the power consumption by the system, it is recommended to lower the clock speed when high clock speeds are not necessary.

| Parameters | System Clock |
| --- | --- |
| 0x00: 1 x cycle | 6.144 MHz |
| 0x01: 2 x cycle | 3.072 MHz |
| 0x02: 4 x cycle | 1.536 MHz |
| 0x03: 8 x cycle | 768 kHz |
| 0x04: 16 x cycle | 384 kHz |

| ## VECT_INTLVSET | Monochrome |  | Color |  |
| --- | --- | --- | --- | --- |

Parameter	RB3………interrupt level (0x00 ～ 0x05)

RC3………interrupt number to be set (0x00 ～ 0x09)

Return value	None

Destroys	None

Sets the interrupt level. Because some interrupts in the NEOGEO POCKET have system involvement, please use this subroutine to change interrupt levels.  Please DO NOT set interrupt levels that may not be set by this subroutine.  DO NOT create functions to do so.

The higher the level number, the higher the priority.  If the interrupt level is set to 0, the interrupt that was set to 0 is nullified.  The highest level is 0x05.

Also, setting the interrupt with this subroutine retains the interrupt request at this time.  If the interrupt request is to be destroyed, please use the system library routine INT_LV_SET (Please refer to SYSTEM LIBRARY MANUAL for further information).

“Interrupt number to be set” should be the interrupt who’s level is to be modified.  The following is the list of interrupts and their corresponding interrupt number.

| 0 : | Interrupt request from RTC alarm (Only valid when the game is running *1) |
| --- | --- |
| 1 : | Interrupt request from the Z80 CPU |
| 2 : | Interrupt request from 8 bit timer 0 |
| 3 : | Interrupt request from 8 bit timer 1 |
| 4 : | Interrupt request from 8 bit timer 2 |
| 5 : | Interrupt request from 8 bit timer 3 |
| 6 : | End of transfer interrupt request from Micro DMA channel 0 |
| 7 : | End of transfer interrupt request from Micro DMA channel 1 |
| 8 : | End of transfer interrupt request from Micro DMA channel 2 |
| 9 : | End of transfer interrupt request from Micro DMA channel 3 |

*1 When the power is off, RTC alarm interrupt is set at the system level and thus can not be set at the user level.

| ## VECT_RTCGET | Monochrome |  | Color |  |
| --- | --- | --- | --- | --- |

Parameter	XHL3……get real time clock data storage address

Return value	(XHL3+0)(1Byte)……year0~99(00~90=2000~2090, 91~99=1991~ 1999)

(XHL3+1)(1Byte)……month1~12

(XHL3+2)(1Byte)……day1~31

(XHL3+3)(1Byte)……hour0~23

(XHL3+4)(1Byte)……minute0~59

(XHL3+5)(1Byte)……second0~59

(XHL3+6)(1Byte)……upper 4 bit:  leap year0~3

(0: leap year  1: 1 year after  2: 2 years after  3: 3 years after)

lower 4 bits:  day of the week0~6

(0: sun  1: mon  2: tue  3: wed  4: thur  5: fri  6: sat)

Destroys	None

Obtains the current time set in the real time clock.

Value obtained is a BCD value.

| ## VECT_ALARMSET | Monochrome |  | Color |  |
| --- | --- | --- | --- | --- |

Parameter	QC3……dayerror value if other than 1~31 or 0xFF

RB3………hourerror value if other than 0~23 or 0xFF

RC3………minuteerror value if other than 0~59 or 0xFF

Return value	RA3………SYS_SUCCESS: no error

SYS_FAILURE: error

Destroys	RB3, RC3

Sets the game alarm.  Please set values as BCD values.

VECT_ALARMSET is only valid when the power is ON.  The interrupt is initiated on the time and date set, but if the user software which set the alarm is not inserted in the unit (NEOGEO POCKET) or if the power is OFF, the interrupt is ignored (*2).

When the power is ON, the interrupt operation set with the user vector 0x6FC8 “RTC alarm interrupt operation” is initiated (during normal operation of the unit).

There is only one alarm setting.  If multiple settings are set, the last setting is valid.

*2  Even when the user software which set the alarm is inserted in the unit, if the system program  (i.e. internal software, eye catch, etc.) is running the interrupt is ignored.

Please DO NOT create a program with high dependency on the alarm.

| ## VECT_ALARMDOWNSET | Monochrome |  | Color |  |
| --- | --- | --- | --- | --- |

Parameter	QC3………dayerror value if other than 1~31 or 0xFF

RB3………hourerror value if other than 0~23 or 0xFF

RC3………minuteerror value if other than 0~59 or 0xFF

Return value	RA3………SYS_SUCCESS: no error

SYS_FAILURE: error

Destroys	RB3, RC3

Sets the alarm (interrupt) which powers ON the NEOGEO POECKET unit when the power is OFF.  Please set values as BCD values.

VECT_ALARMDOWNSET operates only when the power is OFF.  If the power is OFF on the day and time specified, the user software is run.  User_Boot (0x6F84) in the system work has a bit ON to signify the difference between this startup and normal start up.  Please refer to SYSTEM WORK REFERENCE MANUAL for details.

The user software which set the alarm must be inserted on the time specified or if the power is ON the interrupt is ignored.  If another game software is started up even once, the alarm setting is destroyed (excluding the same software title).

There may be only one alarm.  If multiple alarms are set, the last one is the valid one.  Please be aware that VECT_ALARMSET and VECT_ALARMDOWN set may not be set at the same time.

About 0xFF Setting of the Alarm

This value in the day, time and minute parameters, allows the alarm to ignore a parameter.  For example, setting 0xFF in the day, 0x10 in the hour, and 0x30 in the minute sets the alarm to 10:30 AM on the day the alarm is set (if it is before 10:30 AM) or the next day.

*If the alarm is ignored once, the setting is invalid from that time and is destroyed.

| ## VECT_SYSFONTSET | Monochrome |  | Color |  |
| --- | --- | --- | --- | --- |

Parameter	RA3……upper 4 bits    specify palette code for transparency (0~3)

lower 4 bits    specify palette for font (0~3)

Return value	None

Destroys	None

Transfers 256 system font characters to the front half of the character RAM (0xA00~0xAFFF).

In general, please specify ‘0x03’ for RA3.

SYSTEM FONT

0123456789ABCDEF

| ## VECT_FLASHWRITE | Monochrome |  | Color |  |
| --- | --- | --- | --- | --- |

Parameter	RA3……..specify flash memory address (0: 0x200000 / 1: 0x800000)

RBC3……transfer byte number (transfers in multiples of 256)

XHL3……data transfer from address

XDE3……data transfer to address

Return value	RA3……..SYS_SUCCESS: no error

Other values: error

Destroys	RBC3, XHL3, XDE3

Writes data to flash memory.

Data writes are in 0x100 bytes.  Setting 0x01 in RBC3 register transfers 0x100 bytes.

| ## VECT_FLASHALLERS | Monochrome |  | Color |  |
| --- | --- | --- | --- | --- |

Parameter	RA3……..specify flash memory address (0: 0x200000 / 1: 0x800000)

Return value	RA3……..SYS_SUCCESS: no error

Other values: error

Destroys	None

Erases all blocks of flash memory(*3).  However, protected areas are not erased.

| ## VECT_FLASHERS | Monochrome |  | Color |  |
| --- | --- | --- | --- | --- |

Parameter	RA3……..specify flash memory address (0: 0x200000 / 1: 0x800000)

RB3…..…block number

Return value	RA3……..SYS_SUCCESS: no error

Other values: error

Destroys	None

Erases specified block of flash memory.  However, protected areas are not erased.

Caution regarding flash memory block erase system call

System call “VECT_FLASHERS” cannot operate on blocks 32, 33, 34 (F16_B32, F16_B33, F16_B34).  When these areas need to be operated on, please use the system library routine “CLR_FLASH_RAM” (Please refer to SYSTEM LIBRARY MANUAL for further information).

*3  Please refer to FLASH MEMORY REFERENCE MANUAL for flash memory address and block address information.

| ## VECT_FLASHPROTECT | Monochrome |  | Color |  |
| --- | --- | --- | --- | --- |

Parameter	RA3……..please set 0x01

RB3…..…start block number for protect

RC3…..…flash memory card type (0x00: 4Mbit, 0x01: 8Mbit, 0x02:16Mbit)

RD3…..…number of blocks to be protected

Return value	RA3……..SYS_SUCCESS: no error

Other values: error

Destroys	None

Protects the requested number of blocks from the start address in the flash memory.  As an exception, it is not possible to protect blocks after block 32 (F16_B32).

Once the blocks are protected, it is impossible to write or erase data in these blocks.

There is no operation which will remove the protection, and care must be taken when this system call is used.

Flash memory card applicable to this operation is the CS1 (the slot in the 0x800000 development tool).  If the program is run only with the use of the NEOGEO POCKET, this system call is invalid.

Also, this system call destroys the user work RAM area.  The area to be destroyed is approximately 0x400 bytes starting from 0x6000.

Calling this system call while using the debugger will cause the debugger to freeze up.

| ## VECT_GEMODESET | Monochrome | X | Color |  |
| --- | --- | --- | --- | --- |

Parameter	RA3……mode setting	0x00: K1GE upper palette compatible mode

0x10: K2GE mode

Destroys	None

Sets the color display mode to either the K1GE upper palette compatible mode or the K2GE mode.  Mode change is immediate.  Please refer to K2GE TECHNICAL REFERENCE MANUAL for more information.

# REVISION HISTORY

| rel 0.1 | Initial release | 1998/05/18 |
| --- | --- | --- |
| rel 0.2 | “VECT_FLASHWRITE” register use modified | 1998/06/01 |
| rel 0.3 | System font modified System call use method added in subroutine calls “VECT_ALARMDOWNSET” added Register to be used designated to bank 3 | 1998/07/15 |
| rel 0.4 | System font modified Header file changed from “SYS_CALL.INC” to “SYSTEM.INC” | 1998/07/27 |
| rel 0.8 | PREFACE changed “VECT_ALARMDONSET” explanation changed “VECT_ALARMSET” *2 explanation changed “VECT_FLAHPROTECT” added | 1998/08/20 |
| rel 0.9 | “VECT_INTLVSET” interrupt request caution not added Block number caution notes added to “VECT_FLASHERS” and “VECT_FLASHPROTECT” | 1998/09/25 |
| rel 1.0 | “VECT_GEMODESET” added “SYSTEM PROGRAM VERSION USE CONDITIONS” section added Monochrome/color use valid/invalid notations added | 1998/10/21 |