- NEO-GEO Pocket

- Sound Driver Module

- Specification

- Technical Document

- Ver. 1.01

- Technical Development DIV.3

- Research & Development Section

###### NEOGEO Pocket Sound Module

The Sound Module consists of a sequencer (which plays BGM and SE), Channel Voice Generator (which generates tones) and Sound Generator (which outputs sound).

The Sound Module has 4 channels (3 tone channels and 1 noise channel) of sound generation which can output in stereo.

###### Sound Sequencer

The Sound Sequencer reads in the Sound Data outputted by the Sound Tool and plays using the Channel Voice Generator.  The division of the data played is 48.

The BGM is played using a maximum of 4 voice channels, and the SE uses 1 voice channel to play the sound.  If the BGM is being played on the channel specified for SE, the SE has higher priority to have the sound played.  Once the SE has been played, playing of the BGM is resumed (other channels are not affected).  If a play SE request is generated on the same channel as the one being played, the Sound Code with the lower value (closer to 0) has the higher priority and is played.  (Please refer to the Sound Manual)

As another function, the host commands (Fade Out, Tempo Change) created by the Sound Tool is accessible.  A unit of the value specified in the Fade Out equals approximately 1 second of fade out.  The Tempo Change value set in the Sound Tool is used in the following equation, and the resulting value is the next tempo to be used.

Value Set in the Sound Tool x Initial Tempo  64

<Caution> Tempo Change is only valid for the BGM.  SE is fixed with a tempo of 120.

###### Channel Voice Generator

The Channel Voice Generator consists of an Envelope Generator, LFO (Low Frequency Oscillator) and Modulator.  It has the functionality to modulate the envelope, tremolo, vibrato, etc. of each of the channels of the Sound Generator.  It is also possible to produce complicated tones by modulating 2 LFOs (Please refer to Modulation Algorithm).

###### ADSR Envelope Generator

Envelope Generator creates the change in volume of the ADSR.  ADSR has 4 phases (Attack, Decay, Sustain, Release) and each phase is specified with rate numbers (Attack Rate [AR], Decay Rate [DR], Sustain Rate [SR], Release Rate [RR].  Please refer to Wave Step Oscillator) and Sustain Level (SL).  Envelope Generator creates the wave shape with the Wave Step Oscillator accoding to the settings in ADSR.

- Attack

Attack Rate (AR) specifies the increasing gradient of the level from play start (Key ON).  Once the attack reaches the maximum value, the phase changes to decay.  If Attack Rate is set to the maximum gradient, the attack begins at maximum, and if it is set to 0, no key is produced.

Decay

Decay Rate (DR) specifies the decreasing gradient of the level decreasing from the maximum level to the Sustain Level (SL).  If the Decay Rate is set to the maximum gradient, the phase changes to sustain phase from the Sustain Level.  If the value is set to 0, the Sustain Level is ignored and the sustain phase starts from the maximum gradient.

Sustain

Sustain Rate (SR) specifies the decreasing gradient from the Sustain Level.  Sustain phase changes to Release phase once the play stops (Key Off).  If the Sustain Level is 0, play stops.

Release

Release Rate (RR) specifies the decreasing gradient from play stop (Key Off) to the level reaching 0.  If the level is 0, play stops automatically (Key OFF).

- Low Frequency Oscillator (LFO)

LFO is a Low Frequency Oscillator to modulate the key with effects such as vibrato and tremolo.  The LFO uses Wave Type, Hold Time, Depth and Rate of the Wave Oscillator to create waves with frequency T.  Hold Time is the wait time before starting oscillation and the value is from 0 to 31 (1/16 of 0 to 512 ms).  Depth is set with the value from 0 to 127.  Rate is the gradient time for the amplitude to reach the depth.  The rate is the time 0 ~ 4000 ms divided into 0 ~ 31 steps.  If the Rate is 0, the LFO is stopped.

Wave Step Oscillator

The Wave Oscillator produces the wave form specified by the Envelope Generator, LFO, Fade Out Generator and Modulator.  There are five types of base shapes; triangle, square, saw, sweep source and destination.  32 levels of Rate Time allow frequency control.  If the Rate is 0, the LFO stops.

- LFO Modulation Algorithm

- Vibrato & Sweep

Vibrato is determined by the output obtained by the LFO (LFO Value).

- Tone Generator Output Frequency

###### Voice Program Data

Voice Data refers to data for Channel Voice Generator including tone settings.