NEOGEO POCKET

SOUND USER’S MANUAL

2026/7/28   Ver. 1.28

NEOGEO POCKET	1

SOUND USER’S MANUAL	1

1	NEOGEO POCKET SOUND STRUCTURE	4

2	SOUND TOOL	6

2.1	SOUND TOOL OUTLINE	6

2.2	PROJECT	6

2.2.1 Definition	6

2.2.2 File Structure Managed by the Project	7

2.2.3 Files created by the Sound Tool From the Project	8

3	SOUND TOOL USE	10

3.1	SOUND TOOL OPERATION ENVIRONMENT	10

3.2	SOUND TOOL STARTUP	11

3.3	PROJECT SETTINGS	12

3.3.1 Project Setting Window Structure	12

3.3.2 Creating a New Project	12

3.3.3 Project Load and Save	13

3.3.4 Project Setting Window Manipulation	13

3.3.5 Project Data Modification	19

3.3.6 Erasing Data from Project	20

3.3.7 Display Refresh	20

3.4	BOARD ADDRESS SETTING	21

3.5	HOST COMMAND MODIFICATION	21

3.6	TONE EDITING	23

3.6.1 VOICE EDIT Window Functions	23

3.6.2 VOICE EDIT Window Manipulation	23

3.6.3 Label and Comment Input	27

3.6.4 Piano Keys	27

3.7	PLAYING MIDI	29

3.7.1 MIDI PLAY Window Functions	29

3.7.2 MIDI PLAY Window Use	30

3.8	TARGET EMULATION	31

3.8.1 Target Sound Emulator Dialogue Functions	31

3.8.2 Target Play Dialogue Start Method	31

3.8.3 Target Play Dialogue Use	32

3.8.4 Sound Code Table	33

3.9	TARGET FILE	34

3.10	SOUND TOOL ERROR MESSAGES	35

3.10.1 MIDI File Conversion Error Messages	35

3.10.2 Sound Tool Error Messages	42

4	SOUND TOOL MIDI FILE FORMAT	48

4.1	MIDI FILE FROMAT	48

4.2	MIDI COMMANDS VALID FOR CONVERSION	50

4.3	META EVENTS NECESSARY FOR CONVERSION	52

4.4	MIDI FILE FORMAT DIFFERENCE BGM AND SE	52

4.4.1 MIDI Channel	52

4.4.2 MIDI Command	53

4.4.3 Meta Event	53

5	SOUND LIBRARY	54

5.1	SOUND LIBRARY OUTLINE	54

5.2	DATA TRANSFER LIBRARY	54

5.3	PLAY LIBRARY	54

5.4	TRANSFER LIBRARY USE	54

5.4.1 Creating Files Necessary for Data Transfer Using the Converter	55

5.4.2 Assembling Data File Used by the Transfer Program	55

5.4.3 Linking the Link File Created by Assembling	55

5.4.4 Calling Transfer Program as Required	55

5.5	FUNCTION LIST	56

# NEOGEO POCKET SOUND STRUCTURE

NEOGEO Pocket uses Z80 as the sound CPU.  4 Kbytes of program and data exist in RAM for operation.

NEOGEO Pocket sound is separated into the following three parts:

- SOUND DRIVER – Program which generates the music data in Z80.

- SOUND TOOL – Sound Tool consists of the following three functions:

- Uses the board created to produce NEOGEO Pocket sounds on DOS machines (sound board) to emulate the NEOGEO Pocket.

- Tone modification while listening to music generated from the sound board.

- Converts and manages MIDI files and tone data to NEOGEO Pocket data.

Sound Tool provides the functions listed above for sound creation environment.

- SOUND LIBRARY – Library to allow the host computer to manipulate sounds.  The library functions consist of controls for the sound driver to create sounds, and functions to transfer data to the Z80.

Figure 1.  NEOGEO Pocket Sound Development Environment Data Flow

The 3 programs are used to play music on the NEOGEO Pocket and sound board.  Figure 1 shows the flow of data in the sound development environment.

This manual explains the usage of the Sound Tool and Sound Library in the NEOGEO Pocket sound development environment.

Please refer to NEOGEO POCKET SOUND DRIVER MODULE SPECIFICATION TECHNICAL DOCUMENT Ver. 1.00 for information on the Sound Driver.

# SOUND TOOL

## SOUND TOOL OUTLINE

The Sound Tool is a program which provides the sound development environment to produce music used on the NEOGEO Pocket.  It allows the management and editing of data and target emulation.

The following functionality is included in the Sound Tool.

- Play sound data in the DOS environment

The Sound Tool uses the NEOGEO Pocket sound emulation board in a DOS based computer to produce the same sound quality as on the target.

- Tone editing is possible while the music data is being played

This functionality allows real time editing of tones as it would be played on the target.  It is possible to confirm the desired tone in real time with ease.

- Function to edit host commands

The Sound Tool allows editing of host commands.

- Ease of data management on project basis

The Sound Tool manages data related to each sound created through data information called projects.  Editing the project will allow management of data.  Please refer to the section related to projects for further information.

## PROJECT

### Definition

A project contains information about MIDI files and tones, Host Commands, the Sound Driver, and the Sound Library (used to make sound output possible on the NEOGEO Pocket) to allow management of sound data and sound program file.

This project is created and edited in the Sound Tool and allows management of all sound data in the Sound Tool.

The sound data managed by the project is played on the target with the host program accessing the Sound Library.  Thus the Sound Tool converts the sound data managed by the Sound Tool to a format the Sound Library can use and outputs this data.

The project structure is shown in Figure 2.

PROJECT

Tone Data

Driver

Host Data

Library

GROUP

SE MIDI

BGM MIDI

GROUPプ

SE MIDI

BGM MIDI

Multiple MIDIs allowed

Figure 2. Project Structure

Definition of Group

Sound data which may be played is limited due to the sound memory available (4096 Bytes).  The sound usable in this space is defined to be the Group.  Sound data consists of BGM and SE, and each group may have these two types.

### File Structure Managed by the Project

A Project is created in the Sound Tool and manages the following files:

MIDI file (MID) – BGM and SE score data

Tone file (PRG) – Tone data used in the score

Sound driver file (DRV) – Sound driver which makes the scores playable

Host command file (HOS) – Command request made by the host to the sound driver when playing the scores

Sound Library file (REL) – Library which bridges the sound driver and the host program.  As a main function, it controls the sound driver and sound data transfer

<Caution>

MIDI files cannot be edited in the Sound Tool.  MIDI sequencer software must be used.  Please use the format recognizable to the Sound Tool to register the files to the Project.  Conversion Error message will appear if a format not recognizable to the Sound Tool is used.  Please refer to Sound Tool MIDI File Format section of this manual for information regarding MIDI file format recognizable by the Sound Tool.

Please use the Sound Driver file and Library file prepared by SNK.  Please refer to NEOGEO POCKET SOUND DRIVER MODULE SPECIFICATION TECHNICAL DOCUMENT Ver1.00 for further information on the Sound Driver.  And please refer to the Sound Library section of this manual for further information on the Sound Library.

Tone and Host Command files are files created and editable in the Sound Tool.  There is no need to prepare these files outside of the Sound Tool.  Tone and Host Command files created previously may be reused.  (When the Project file is saved, a new version of the Tone file and Host Command files are created.)

Please refer to the sections in this manual for further information on Tone and Host Command files.

### Files created by the Sound Tool From the Project

The Sound Tool creates a Project file (PRJ) to save the changes in the Project file.  When this Project file (PRJ) is saved, the Tone and Host Command edited is also saved and updated as Tone and Host Command files.

If all of the necessary data is present in the Project, files for the target can be outputted.  The 3 files are, Sound Library file, ASM file, and INC file.

From this point on, these 3 files will be referred to as Target files.

When the target file is saved, the data is converted to the format for the target system (NEOGEO Pocket) and saved.  Explanation of the 3 files are as follows:

Sound Library file:  The file is saved with a fixed file name SLIB.REL.

ASM file:  The file is a sound data file to be used by the Sound Library based on the Project information.

INC file:  The file contains commands the host program will be using to request the generation of commands by the Sound Library.

To play scores and sounds by the host program (game target program) the target files must be linked to the host program.

The files created by the Sound Tool is listed below:

Project file (PRJ) – File containing Project information

Tone file (PRG) – File containing editing of tones

Host Command file (HOS) – File containing editing of host commands

Sound Library file (SLIB.REL) – Sound Library file with fixed name

Sound data file (ASM) – Data file to play scores

Command register file (INC) – Registration file containing commands to be requested to the Sound Library

Example)

When saving “SAMPLE.PRJ” project file to the “C:\DATA” directory:

Files created:

C:\DATA\SAMPLE.PRJ

C:\DATA\SAMPLE.PRG

C:\DATA\SAMPLE.HOS	If host commands are available.

When saving, files the Sound Library will use is saved to the “C:\DATA” directory:

Files created:

C:\DATA\SLIB.REL		If REL file is registered.

C:\DATA\SAMPLE.ASM	If the driver and score data are registered.

C:\DATA\SAMPLE.INC	If the driver and score data are registered.

# SOUND TOOL USE

## SOUND TOOL OPERATION ENVIRONMENT

Sound Tool runs in a Windows95 and Windows98 environment.

Please choose the working directory (folder) by specifying the environment variable TMP.

Please set the environment variable TMP. (If this variable is not set, the working folder is designated as the current.)

To play scores in the Sound Tool, it is necessary to have the NEOGEO Pocket sound emulation board (designated as Sound Board).  To use the Sound Board, it is necessary to set the access address (designated as Board Address) for the sound board.  The Sound Board Address space is the physical address C0000h ~ DFFFFh, and changing the dip switch setting will define the 8 Kbytes necessary.

<Caution 1>  The Sound Tool Board Address setting is automatic from NEOGEO Pocket Sound Tool Ver. 1.20.  Please refer to the BOARD ADRESS SETTING section in this manual for further information on confirmation of the Board Address.

<Caution 2>  Please make sure the address used by the Sound Board and the devices used by the Windows OS is not in conflict.  If the addresses are in conflict, there is a high possibility of not being able to play scores.

For example, Windows normally uses C0000h ~ C7FFFFh for Video BIOS, and thus setting the Board Address to C0000h ~ C1FFFFh will create a conflict.

We recommend using Board Address D0000h ~ D1FFFh.

Board Address setting table is shown below.

| DIPSW |  |  |  | Board Address |
| --- | --- | --- | --- | --- |
| 4 | 3 | 2 | 1 |  |
| Cannot be set |  |  |  | Non Board |
| Off | Off | Off | Off | C0000h ~ C1FFFh |
| Off | Off | Off | ON | C2000h ~ C3FFFh |
| Off | Off | ON | Off | C4000h ~ C5FFFh |
| Off | Off | ON | ON | C6000h ~ C7FFFh |
| Off | ON | Off | Off | C8000h ~ C9FFFh |
| Off | ON | Off | ON | CA000h ~ CBFFFh |
| Off | ON | ON | Off | CC000h ~ CDFFFh |
| Off | ON | ON | ON | CE000h ~ CFFFFh |
| ON | Off | Off | Off | D0000h ~ D1FFFh (Initial setting) |
| ON | Off | Off | ON | D2000h ~ D3FFFh |
| ON | Off | ON | Off | D4000h ~ D5FFFh |
| ON | Off | ON | ON | D6000h ~ D7FFFh |
| ON | ON | Off | Off | D8000h ~ D9FFFh |
| ON | ON | Off | ON | DA000h ~ DBFFFh |
| ON | ON | ON | Off | DC000h ~ DDFFFh |
| ON | ON | ON | ON | DE000h ~ DFFFFh |

Table 1.  Board Address Setting Table

## SOUND TOOL STARTUP

To use the Sound Tool, it is necessary to setup a password for each machine (password is different for each machine).  Information necessary to obtain the password must be forwarded in order to obtain password.

Method to obtain password is given below.

Running the Sound Tool for the first time on a computer will display the password input window as in Figure 3.  A password must be entered or the Sound Tool will not run.  Once the password is entered, it is not necessary to enter it again to run the program.  In future version updates and for other unforeseen reasons, it maybe necessary to enter it again.

Please convey the “Information necessary to obtain password” to receive the password.  After receiving the password, please enter it into the “Password” box.  This will allow startup of the Sound Tool.

Figure 3. Password Dialogue

<Note>

It is currently impossible (versions after NEOGEO Pocket Sound Tool Ver. 1.20) to run two Sound Tool tasks.

## PROJECT SETTINGS

A Project is edited in the VOICE EDIT & SOUND EMULATOR window (designated as Project Setting Window) of the Sound Tool.  Explanation of the Project Setting Window is given in the following sections.

### Project Setting Window Structure

Project Setting Window consists of the File Selector (left) and Project Editor (right) as shown in Figure 4.  File manipulation (loading and saving files) is done through the Load and Save functions in the FILE menu.  Dragging and dropping files listed in the File Selector to the Project tree in the Project Editor registers the data.  Use of these functions are given in the following sections.

Figure 4. Voice Edit & Sound Emulator Window

### Creating a New Project

A drop down menu is displayed when FILE is chosen from the Menu bar (Figure 5).  Select NEW and then PROJECT.  Selecting NEW will abandon the Project that was being modified.  If any modifications were made, a prompt to save will appear.  Please choose accordingly.

### Project Load and Save

To load a Project, select FILE from the Project Setting Window Menu bar and select LOAD from the drop down menu.  Next left mouse click the PROJECT and a file selector dialogue is displayed.  Please select the Project file with the .PRJ extension and load.

To save a Project, choose SAVE from the FILE menu.  Selecting PROJECT will display a file selector dialogue.  Please specify the file name and directory and save.

Figure 5. Drop Down Menu

Project SAVE saves the Project modification.  Loading the saved Project will allow editing of Project from the last modification.  Also Project SAVE also saves the Tone file (PRG) and Host program file (HOS).

<Caution> Project files (PRG) created by Sound Tool Ver. 1.10 and later cannot be loaded in Ver. 1.00 and Ver. 1.01 of the Sound Tool.

### Project Setting Window Manipulation

It is possible to register Library File, Driver File, Host Command File, Tone File and MIDI File to the Project.  Registration is performed through drag and drop of the files.

- Sound Library File – File with REL extension

- Sound Driver File – File with DRV extension

- Host Command File – File with HOS extension

- Tone File – File with PRG extension

- MIDI File – File with MID extension

Drag the files from the File Selector to the Project Group in the Project Editor window to register files A) ~ C).

Registration of Tone Files D)

Tone file consists of 128 tone data.  Dragging and dropping Tone File from the File Selector to the Project in Project Edit overwrites and registers all tones.

Double clicking on the Tone File in the File Selector displays the 128 tones registered.  Dropping a tone data on top of one in the Project Editor will overwrite and register one tone.

Tone File

Drag PRG

Drag Tone

Drop Tone

Drop PRG to

Tone File

Figure 6. Registering Tone

Registration of MIDI Files E)

MIDI file is converted as score data and registered when it is registered in the Project.  MIDI files must be separated into Group units and registered.  The NEOGEO Pocket has 4 Kbytes designated for sound, and thus loading all of the scores used for a game is not possible in most cases.

It is necessary to structure the scores per scene and registering the score data possible to load as a Group.  The Sound Library will transfer the Group of score data to the RAM with the request from the host.

Score data flow chart for data converted from MIDI data is shown below.

Group 1

Score Data 1

(MIDI 1)

Score Data 3

(MIDI 3)

Group 2

Score Data 1

(MIDI 1)

Score Data 2

(MIDI 2)

Group 3

Score Data 4

(MIDI 4)

Score Data 6

(MIDI 6)

MIDI Files

MIDI 1

MIDI 2

MIDI 3

MIDI 4

MIDI 5

MIDI 6

Separated to Groups and Registered

Sound Tool

Conversion

Sound Library

Sound Memory

Driver

Sound Data Group N

Play Data in Sound Memory

Transfer Data as Conditions arise

Memory Size 4 KBytes

Data Used by the Sound Library

Sound Data

Group 1

Sound Data

Group 2

Sound Data

Group 3

Figure 7. MIDI Data Flow chart

First, MIDI files are separated into Groups in the Sound Tool.  Next the Group is converted per Group into score data to be used by the Sound Library.  The host program uses the Sound Library to transfer data to RAM according to the need of the game.

MIDI File Registration in the Sound Tool

MIDI file registration must start with the creation of a Group.  In the Project Setting Window, select EDIT from the Menu, PROJECT from the drop down menu and then ADD GROUP.  A dialogue is displayed (Figure 8).  Please enter the Group name and click on OK.  The Group name just entered should be displayed in the Project Editor below the Group folder.

Figure 8. Group Input Dialogue

This operation should also create a BGM and SE folders.  Please register scores to be used as BGM in the BGM directory and SE in the SE directory.  The sound driver treats these two data formats differently.  (Please refer to BGM and SE MIDI Format Difference in this manual for

further information).

The Group created and MIDI file registered to the Group will have a “converted size” displayed (right of the Group and MIDI files) in the Project Editor.  This “converted size” is the approximate size of the data when the Sound Converter function of the Sound Tool has finished converting the data.

The value next to the MIDI is the data size of the MIDI once it has been converted to score data.

The convert size of the Group is the approximate size of data to be used on the target.  If this value is over the RAM (4096 Bytes) the Z80 in NEOGEO Pocket uses, SizeOver comment is added to the right of the convert size.  When this comment is present, it is not possible to output target files.

Group convert size is calculated from the Sound Driver size, Host Command data size, Score data size and Tone data the MIDI uses.

As with the other files A) ~ D), registration of the MIDI file is done through drag and drop of MIDI files into BGM and SE folders.

<Caution 1>

Group convert size is an approximation.

If same Tone data is used for a different position, the Sound Tool compresses data to use the same Tone data.  The Group convert size should be used as a guide to create data.

<Caution 2>

A MIDI file may only be registered in either BGM or SE.  To change a MIDI file registration from BGM to SE it is necessary to make sure it is not registered as BGM in another Group.  If it is to be changed from SE to BGM, then check SE of other Groups.  To change the MIDI file from BGM to SE, it is required to erase the registration from BGM before registering as SE.

If the MIDI file is registered in one, trying to register in the other will result in an error message and will not be registered (Please refer to Sound Tool Error Message section for further information.  Error Code [9042]).

Figure 9 shows A) ~ E) files registered.

In Figure 9, K1SND.DRV file is registered as the Sound Driver, K1SLIB.REL file as the Sound Library, SAMPLE.PRG file as Tone data and SAMPLE.HOS file as the Host Command File.

The Group is designated as tmp, BGM folder as MIDI1.mid and SE folder as MIDI2.mid.  Numerous MIDI files may be registered to Group.

Figure 9. Project Registration Sample

### Project Data Modification

It is possible to change the Group name and MIDI registration order in a Project.

Group Name Change:

First, choose the Group in the Project manager.  Click on the right mouse button and open the menu to change the Group name.

MIDI Registration Order:

Left click the MIDI file, drag and drop on top of the file to be succeeded by.  The file that you drop on to will be preceded by the file chosen.  (Registration order may only be changed in the same Group.  If the MIDI file to be registered is used in another Group as the same type (BGM or SE), it is necessary to erase the MIDI in the other Group before preceding.  A MIDI file registration is restricted to either BGM or SE and not both types.)

### Erasing Data from Project

To erase data from a project, choose the file with the mouse and press the delete key on the keyboard.  The Sound Driver and Sound Library file may not be erased, but may be re-registered.

Choosing the Group erases all of the MIDI file.  (It is not possible to erase just the BGM and SE separately.)

### Display Refresh

The display in the File Selector and Project Editor window is updated when a directory is displayed.  If display refresh is required, it is necessary to close and reopen the directory.

## BOARD ADDRESS SETTING

Board Address is the memory address in the RAM the Z80 of the Sound Board uses.  Board Address setting is not necessary for NEOGEO Pocket Sound Tool Ver. 1.20.  It is automatically set.

To confirm which address the Sound Tool has defined, please follow the steps listed.  Select HELP from the Project Setting Window menu.  Board Address set by the Sound Tool will be displayed when the BOARD ADDRESS selection is made from the drop down menu.

Please refer to SOUND TOOL OPERATION SECTION for further information about the Board Address.

## HOST COMMAND MODIFICATION

Host commands are pre-defined commands the host requests to the Sound Driver.

There are two types of host commands:  Fade Out Command (Slowly lowers the play volume) and Tempo Change Command (Changes the play tempo).

Fade Out Command has a value of 1 ~ 31, and the higher the value, the longer the time taken to fade out.

Tempo Change Command has a value 1 ~ 255, and the tempo value is determined with the equation below.

Tempo Change Value = Tempo Value to be Set  Initial Tempo Set x 64

Host command is set with both the command type and value.  The maximum command that may be set in a Project is 32 types.  The same host command type may not be set twice.

Host Command Definition:

Left click on the EDIT of the menu and choose HOST from the drop down menu.  The dialogue below appears.

Figure 10. Host Command Definition Dialogue

Please click on the ADD button to define a new Host Command.  Table Add dialogue (Figure 11) will appear.  Please choose Fade Out or Tempo Change and input the parameter value desired.  Finally clicking OK will add the new command in the Host Command List.  Selecting a command in the Host Command List and clicking on the delete button (DEL) will delete the chosen Host Command.

Dragging will also allow change of order for the host command.  Left click the command to be moved and drag it over the host command to succeed the command.

Figure 11. Host Command Add Dialogue

## TONE EDITING

Tone is edited (modified) in the VOICE EDIT window.

VOICE EDIT window functions, methods of Tone editing, and parameter definitions follow.

### VOICE EDIT Window Functions

Tone parameter editing is possible in the VOICE EDIT window.  It is also possible to have the MIDI file playing, set in the MIDI PLAY window, with all of the changes being reflected while editing.

It is also possible to edit the Tone without the MIDI file through the use of the piano key on screen.

0 ~ 127 Tones (128 types) accessible by the MIDI can be edited.

The editing involves the following parameters:  ADSR setting to define the envelope, algorithm LF01 and LF02 to set modulation, and noise type to use the tone as noise.

In order to reflect the changes in the MIDI file concurrently as the Tones are being changed, it is necessary to set the program change in the MIDI file.  Please edit the same number of program and tone.

### VOICE EDIT Window Manipulation

The VOICE EDIT window allows the tone data in Tone File (PRG file set in the Project) to be edited.  Tone editing is possible by choosing the tone to be edited (chosen by program number), and setting the parameters (ADSR, algorithm, LFO and noise type).

Changes to Tone edited in this VOICE EDIT window can be reflected in the score played in the PLAY window.  It is possible to listen to the score while editing.  When the Project is saved, the Tone File is automatically saved.

Tone Editing Parameter

Figure 12 shows the VOICE EDIT window.  The setting shown may be edited.

Figure 12. VOICE EDIT Dialogue

(1) Program Number Selection

This is the pull down menu to select the 128 types of tone.

The 128 types of tone are displayed as [PROG 0] ~ [PROG 127] and are associated with the MIDI program number.  The program number (Tone) has a label and comment (to allow easy understanding).  The label is displayed to the right of the program number, and the comment is displayed in the VOICE EDIT window to the right of the program number.  Please use the comments to manage tone data.

Please click on the downward arrow to display the pull down menu and choose the desired program number.  The tone number and label will be displayed.

(2) ADSR Settings

This box is used to set the volume change initiated with the ADSR.  ADSR wave form is determined by the 5 settings:  attack rate (AR), decay rate (DR), sustain level (SL), sustain rate (SR) and release rate (RR).  Parameters are explained below.

Attack rate (AR) – This is the gradient from volume output initial level to its maximum volume.  The parameter value is 0 ~ 15.  If the attack value is 15, the sound is produced at maximum from the start.  If the attack is 0, no tone is generated.

Decay rate (DR) – This is the decay rate from the maximum volume to the sustain level.  The parameter value is 0 ~ 15 and if 0 is specified there is no decay.

Sustain rate (SR) – This is the negative gradient from the end of decay rate (sustain level) to key off (sound output stop).  The value is 0 ~ 15.

Sustain Level (SL) – This is the end of decay rate level.  The value is 0 ~ 15 and if the sustain level is set to 0, no decay is produced.

Release rate (RR) – This is the reverberation gradient from Key off to volume level 0.  The value is 0 ~ 15.

Please use the up and down arrow buttons on the keyboard to set the values.  If entering using the keyboard, please press the ENTER key after inputting the values.

(3) Algorithm and LFO settings

The algorithm setting is used to specify the tone waveform and the volume changes.  LFO parameter settings are used to set the two LFO, LFO1 and LFO2, used by the algorithm.

- Algorithm setting

There are 8 settings, 0 ~ 7, or algorithms prepared.  The algorithms use the LFO in different combination to change the keys.  The algorithms are defined below.

Algorithm 0 – Does not change key.

Algorithm 1 – LFO1 is used for Tremolo and LFO2 is used for vibrato.

Algorithm 2 – LFO1 and LFO2 are used to create a FM change key for both tremolo and vibrato effect.

Algorithm 3 – LFO1 and LFO2 are used to create an AM change key for both tremolo and vibrato effect.

Algorithm 4 – LFO1 and LFO2 are used to create a FM change key for tremolo effect.

Algorithm 5 – LFO1 and LFO2 are used to create an AM change key for tremolo effect.

Algorithm 6 – LFO1 and LFO2 are used to create a FM change key for vibrato effect.

Algorithm 7 – LFO1 and LFO2 are used to create an AM change key for vibrato effect.

Algorithms are set using the up and down arrow buttons to the right of the algorithm button.

- LFO setting

Sets the wave form change to be added to the tone to be produced (played).

Two types of LFO may be set, and the two are referred to as LFO1 and LFO2.  The changes LFO1 and LFO2 produce are for volume and frequency of the tone.  The use of LFO1 and LFO2 are dependent on the algorithms set.

The LFO is set using 4 parameters, wave shape type, hold, depth and rate.

Wave Shape Type – This is the base shape of the wave.  They may be chosen from the following:  triangle wave, square wave, saw wave, sweep 1, sweep 2.  If the LFO set is to be used for tremolo effect, sweep 1 and sweep 2 may not be chosen.  The wave shape type is numbered from 0 to 4 from triangle, square, saw, sweep 1 and sweep2 in that order.

Hold – This is the wait time until the change takes effect.  The value is set from 0 to 512 ms divided into 32 steps from 0 to 31.  If the hold value is 0, the wait time is also 0.

Rate – This is the gradient (rate) of time of the depth reaching the value set.  0 to 4000 ms is divided into 32 steps from 0 to31.  If the rate is set to 0, the LFO does not function as a LFO.

Depth – The depth change of the wave form.  The value is from -127 to +127.  If the depth is set to 0, the LFO does not function as a LFO.

If the maximum range (127) is used, LFO changes 1 octave.  If the sweep wave shape is used, 4 octaves of change is achieved.

These parameters should be changed using the arrow buttons or direct keyboard input.  If values are inputted directly, please press ENTER.

(4) Noise Setting

This setting determines if the noise type when the sound is to be used as noise.  The noise types are PINK and WHITE.  In the NEOGEO Pocket, the noise is used for the channel 4 sequence.  For this reason, the use or non-use of this tone as noise is set in channel 4 tone setting.

Please check the button left of the White or Pink selection to switch settings for Noise.

(5) Tone Initialization

Pressing the Clear button in the box titled This Tone will initialize the parameters of the tone of the program number chosen.  However, the label and comment information are not cleared.

Initialized parameter values of tone data are listed below.

ADSR

Attack Rate = 0

Delay Rate = 0

Sustain Rate = 0

Sustain Level = 0

Release Rate = 0

Algorithm = 0

LFO1 and LFO2

Wave Shape Type = Triangle (0)

Hold = 0

Rate = 0

Depth = 0

Noise Type = Pink Noise

### Label and Comment Input

Pressing the Label button to the right of the program number results in a label input dialogue (Figure 13).  It is possible to input a label and a comment for the program number chosen.

This label is displayed following the program number and the comment is shown in the VOICE EDIT window title bar.

Figure 13. Label Input Dialogue

### Piano Keys

The piano keys on the bottom of the VOICE EDIT window will produce the tone being created with the key of that piano key.  The range available on the piano key in Figure 12 is from G2 to C9.  (In MIDI note number, it is from 43 to 120).

Choosing the piano key noise use selection to Normal will produce the normal tone, while setting the selection to Noise will produce noise.

The tone played when the piano key is pressed, is based on the fixed sound data of the Sound Driver registered to the Project.  Thus it is not possible to control key-on, key-off play time using the piano key.  This is due to the fact that the Sound Driver can only play data converted earlier, and it is not possible to control the Sound Driver in real time.

The MIDI data information when the piano key is pressed is shown below.

Tempo = 120

Note Number = 43 ~ 120 (G2 ~ C9)

Step = 384 (2 measures [bars])

Gate = 384 (2 measures [bars])

Volume = 127 (maximum)

Pan = Center

## PLAYING MIDI

MIDI files are played in the MIDI PLAY window.

The functionality and the use of the MIDI PLAY window are given below.

### MIDI PLAY Window Functions

In the MIDI PLAY window, it is possible to play a MIDI file, selected in the File Selector of the MIDI PLAY window, with the same quality as if the file was converted to NEOGEO Pocket music data.  (From this point on, playing MIDI files will be referred to as playing MIDI file converted to NEOGEO Pocket music data and playing on the Sound Board.)

The Tone information used during play is affected by the Tone set in the VOICE EDIT window.

It is possible to mute the MIDI file being played on a track basis.  Also, it is possible to set the tone per track.  (The selected Tone is displayed in the VOICE EDIT window.)

Figure 14. MIDI PLAY Dialogue

### MIDI PLAY Window Use

- The directory name and MIDI file names are displayed in the MIDI File Selector in the drive.   Double clicking on a directory name produces the directory and MIDI file names included in this directory.  Double clicking on the MIDI file and selecting the file will display the file in the MIDI PLAY window and is registered.  (The MIDI is not played at this time.)

- To play the MIDI, please click on the PLAY button.  To stop, please press the STOP button.

- The MIDI file is played using the Tone set in the VOICE EDIT window.  To change the tone, please change the Tone settings in the VOICE EDIT window.  Changing the Tone setting will affect the MIDI currently being played.

- To determine which Tone file is being used for each track, use the Tone Being Used Combo Box to display the program numbers.  Choosing the program number while it is being displayed will swap the program number to the one chosen in the VOICE EDIT window.  The Tone displayed is in order of program changes (Tone changes) per each track play sequence.

- It is possible to play the MIDI file and mute on a track basis.  Checking the track in the TRACK MUTE BOX in the MIDI PLAY window will mute the track.  If track mute is changed while the track is currently being played, the MIDI PLAY function will stop.  To start the MIDI PLAY, please press the PLAY button.

- If the MIDI file is selected in the File Selector when the MIDI file is registered, the software prompts the question weather to destroy the current registration.  If Cancel button is pressed, the new MIDI file is not registered.  Pressing the OK button requires steps to be followed from B.

<Caution>

The MIDI files selected in the MIDI PLAY window, to allow track mute function to be available for all tracks, is converted with tempo change information (usually 1 track).

And because the convert size is larger than the normal conversion, music data larger than 4096 Byte in the Z80 RAM cannot be used in the MIDI PLAY window.

## TARGET EMULATION

Target emulation is done through the TARGET PLAY dialogue.  The functions, startup, control and sound code used in the target emulation is explained below.

### Target Sound Emulator Dialogue Functions

TARGET Sound Emulator dialogue emulates the target functionality based on the Project information set in the Project Setting window.

In the Target Sound Emulator window, the request commands are generated to the Sound Library as the host would.

Figure 15 shows the Target Sound Emulator dialogue.

Selectable Code List

Figure 15. Target Sound Emulator Dialogue

### Target Play Dialogue Start Method

There are 2 methods to startup the Target Play Dialogue.  One method is to set in the Project Setting window the state to be emulated by the Target Sound Emulate dialogue and target emulate with the Project data.  The other method is to read in the data files (.ASM, .INC) outputted for use by the target and to target emulate with this data.

The first method is implemented by left clicking the Target selection from the menu in the Project Setting window to display the drop down menu.  Choosing Target from this menu will open the Target Sound Emulate dialogue as in Figure 15.

The second method is implemented by selecting FILE from the menu to display the drop down menu.  And from this drop down menu, choose LOAD and left click.  On the right, after selecting Target, File Selector to load .ASM file is displayed.  Please choose the file that is to be target emulated.  Since the .INC file and .ASM are outputted in pair, it is necessary to have the correct .INC file or the target emulation is not done.  If loading of these two files is successful, the Target Sound Emulate dialogue as in Figure 15 will appear.

### Target Play Dialogue Use

- TARGET PLAY dialogue is displayed if the method listed above is used.

- Clicking on one of the five combo box right of the dialogue will display the sound code list selectable.  Sound Code List is categorized into 5 types and are sorted by name.  The 5 types are separated on the basis of use.

- Existing Sound Code – The base sound codes.  These are normally BGM, SE Control and Sound Driver Reset.

- Sound Data Transfer Sound Code – Sound code set for each group to transfer sound data (one per group).

- BGM Play Start Sound Code – BGM start play sound code registered for each group (one per BGM type).

- SE Play Start, Stop Sound Code – SE start and stop play code registered for each group.  (Twice the number of SE exists.  This is because the SE is different from the BGM in that, more than one SE may be played and stop play sound code exists for the number of SE registered.)

The sound code is chosen from the sound codes listed above.

3.  The sound code chosen with the method above will be related to the SEND button to the left of the list.  Every time the SEND button is pressed, the sound code selected will be processed.

4.  Since each of the SEND buttons may be related to different sound codes, it is possible to play the BGM, SE, fade out, and tempo change.

### Sound Code Table

The sound code name that is registered in the .INC file outputted when the project file is saved is used.  Table 3.8-1 explains the sound code used in the target emulate function.

| ALL_EI | Will accept all sound code related to BGM and SE. |
| --- | --- |
| BGM_EI | Will accept all sound code related to BGM. |
| SE_EI | Will accept all sound code related to SE. |
| ALL_DI | Will not accept all sound code related to BGM and SE. (Exception: ALL_EI, BGM_EI and SE_EI) |
| BGM_DI | Will not accept all sound code related to BGM. (Exception: ALL_EI and BGM_EI) |
| SE_DI | Will not accept all sound code related to SE. (Exception: ALL_EI and SE_EI) |
| BGM_STOP | Stops the BGM being played currently. |
| SE_ALL_STOP | Stops all SE being played currently. |
| RESET | Initializes the Driver. |
| GROUP_XXXX | XXXX group is transferred to the sound memory. After this transfer, BGM and SE registered in this group may be played. (XXXX is the group name registered in the Project.) |
| BGM_XXXX | Starts playing the BGM of XXXX. (XXXX is the MIDI file registered in the Project.) |
| SE_XXXX | Starts playing the SE of XXXX. (XXXX is the MIDI file registered in the Project.) |
| SE_STOP_XXXX | Stops playing the SE of XXXX. (XXXX is the MIDI file registered in the Project.) |
| HOST_XXXX | Processes the host commands of XXXX. (XXXX label is the host command name registered and the value set in host command editing.) |

Table 3.8-1 Sound Code Table

## TARGET FILE

Sound data file refers to the LIB.REL, .ASM, .INC files needed to link with the target program.  (These 3 files will be noted as Target files from this point onward.)  SLIB.REL file is the Sound Library file used to link with the host program.  .ASM file is the sound data file accessed when the Sound Library is used.  .INC file is the sound code registered for the host to use the Sound Library.  These 3 files make it possible to play sounds on the target.

Follow these steps to output the Target files.  First, select FILE in the Project Editor window and display the pull down menu.  From the pull down menu, select SAVE and then choose the Target selection to the right and the File Selector will appear.  Specify the ASM file name and the output path.  When the Sound Tool outputs the .ASM file, it also outputs the .INC and SLIB.REL files.  If output (save) was successful, the Sound Driver and Sound Library version used to output is displayed.  Please do not change the .ASM and .INC file in a text editor.  If these files are edited, the sound program processing and sound data on the target are not guaranteed to be correct.

## SOUND TOOL ERROR MESSAGES

### MIDI File Conversion Error Messages

In the Sound Tool, MIDI files registered in the Project and selected in the MIDI PLAY window are converted to the NEOGEO Pocket sound data format.  If errors are encountered during conversion, error messages or warning messages are displayed.

Error messages denote a data not convertible or a state exists in which the conversion is not possible.  MIDI files with error messages cannot be managed by the Project.

Warning messages are outputted when meaningless data exists, not when conversion has failed.

The following tables list the error and warning messages when the Sound Tool converts files.  (If multiple error codes are listed, they all have the same meaning.)

Error Message Table

| Error Code | 1000 ~ 1011 |
| --- | --- |
| Error Message | Not standard MIDI format. ｽﾀﾝﾀﾞｰﾄﾞMidiﾌｫｰﾏｯﾄではありません |
| Error Defined | The MIDI file is not a standard format file. This error is not related to NEOGEO Pocket limitations. The file is not a standard MIDI format or there is the possibility of this file being corrupt (destroyed). |

| Error Code | 2000 |
| --- | --- |
| Error Message | Invalid fadeout command data[??]. ｻﾎﾟｰﾄされていないﾌｪｰﾄﾞｱｳﾄを設定しています[??] |
| Error Defined | The value of the host command fade out in the convert setting file is invalid. ?? is the invalid value. |

| Error Code | 2001, 2003, 3020, 3021 |
| --- | --- |
| Error Message | Too many host commands. ﾎｽﾄｺﾏﾝﾄﾞが最大数を超えています |
| Error Defined | The host command registered exceeds the maximum 32. There are two areas, convert setting file and MIDI file registered, where the host commands may be set. The total number of these to areas is compared to the maximum. |

| Error Code | 2002 |
| --- | --- |
| Error Message | Invalid tempo chenge command data[??]. ｻﾎﾟｰﾄされていないﾃﾝﾎﾟﾁｪﾝｼﾞを設定しています[??] |
| Error Defined | Invalid tempo change value (located in the host command of convert settings file). ?? is the invalid value. |

| Error Code | 2004, 2013, 2014 |
| --- | --- |
| Error Message | No Sectoin_End. Section_Endがありません |
| Error Defined | There is no Section_End, which defines the end, in the convert settings file. |

| Error Code | 2005 |
| --- | --- |
| Error Message | No driver file name. ﾄﾞﾗｲﾊﾞｰﾌｧｲﾙ名がありません |
| Error Defined | There is no Sound Driver file registered in the convert settings file. Please register the Sound Driver file in the settings section. |

| Error Code | 2006 |
| --- | --- |
| Error Message | No group lists. [GROUP_LIST]がありません |
| Error Defined | There is no Group list, and at least one group list is necessary. Or there is a possibility of incorrect registration in the [GROUP_LIST]. |

| Error Code | 2007 |
| --- | --- |
| Error Message | No sound lists in the ?? group list. ??番目の[GROUP_LIST]に[SOUND_LIST]がありません |
| Error Defined | No [SOUND_LIST] is registered as specified in the Group list ??. Or there is a possibility of incorrect registration in the [SOUND_LIST]. |

| Error Code | 2008 |
| --- | --- |
| Error Message | No group name in the ?? group list. ??番目の[GROUP_LIST]にｸﾞﾙｰﾌﾟ名がありません |
| Error Defined | In the group list ?? there are no group names registered. Or there is a possibility of incorrect registration of group names. |

| Error Code | 2011, 3022 |
| --- | --- |
| Error Message | Too many BGM music. BGMが最大数を超えています |
| Error Defined | Number of BGM registered exceeds the maximum 95. |

| Error Code | 2012, 3023 |
| --- | --- |
| Error Message | Too many SE music. SEが最大数を超えています |
| Error Defined | Number of SE registered exceeds the maximum 384. |

| Error Code | 2015 |
| --- | --- |
| Error Message | No music in group list. 曲の登録されていない[GROUP_LIST]があります |
| Error Defined | There is no music registered in the group list. Or there is an incorrect registration of MIDI files in the [SOUND_LIST] section. |

| Error Code | 3000 |
| --- | --- |
| Error Message | Invalid MIDI format type ??. MIDIﾌｫｰﾏｯﾄ ﾀｲﾌﾟ ?? は無効です |
| Error Defined | MIDI file with invalid MIDI format is used. The NEOGEO Pocket uses MIDI format type 1. ?? contains the incorrect format type number. |

| Error Code | 3001 |
| --- | --- |
| Error Message | Can’t use division of time code type. ﾀｲﾑｺｰﾄﾞ管理の分解能は使用できません |
| Error Defined | MIDI file used has division used in the time code management. |

| Error Code | 3003 |
| --- | --- |
| Error Message | Track=??, Channel=??, Measure=??, Step=??, Invalid note No.[??]. ﾄﾗｯｸ=??, ﾁｬﾝﾈﾙ=??, ﾒｼﾞｬｰ=??, ｽﾃｯﾌﾟ=??, ﾉｰﾄ番号[??]は無効です |
| Error Defined | Invalid note number ?? is used in Track = ??, Channel = ??, Measure = ??, and Step = ??. |

| Error Code | 3007, 3008 |
| --- | --- |
| Error Message | Track=??, Channel=??, Measure=??, Step=??, Invalid tempo change command. ﾄﾗｯｸ=??, ﾁｬﾝﾈﾙ=??, ﾒｼﾞｬｰ=??, ｽﾃｯﾌﾟ=??, 無効なﾃﾝﾎﾟﾁｪﾝｼﾞｺﾏﾝﾄﾞです |
| Error Defined | Invalid tempo change command is used in the MIDI file on Track ??, Channel ??, Measure ??, and Step ??. |

| Error Code | 3011 |
| --- | --- |
|  | Track=??, Measure=??, Step=??, Invalid fadeout command (host command) [??]. ﾄﾗｯｸ=??, ﾒｼﾞｬｰ=??, ｽﾃｯﾌﾟ=??, ﾌｪｰﾄﾞｱｳﾄ( ﾎｽﾄｺﾏﾝﾄﾞ )[??]は無効です |
| Error Defined | Track ??, Measure ??, and Step ?? of MIDI file has an invalid fade out host command. [??] is the invalid value. |

| Error Code | 3012 |
| --- | --- |
| Error Message | Track=??, Measure=??, Step=??, Invalid tempo change command( host command )[??]. ﾄﾗｯｸ=??, ﾒｼﾞｬｰ=??, ｽﾃｯﾌﾟ=??, ﾃﾝﾎﾟﾁｪﾝｼﾞ( ﾎｽﾄｺﾏﾝﾄﾞ )[??]は無効です |
| Error Defined | Track ??, Measure ??, and Step ?? of the MIDI file has invalid tempo change host command. [??] is the invalid value. |

| Error Code | 3013 |
| --- | --- |
| Error Message | Track=??, Measure=??, Step=??, Too many nests of loop command. ﾄﾗｯｸ=??, ﾒｼﾞｬｰ=??, ｽﾃｯﾌﾟ=??, ﾙｰﾌﾟのﾈｽﾄ回数が最大数を越えています |
| Error Defined | Track ??, Measure ??, and Step ?? of the MIDI file has too many nested loop commands. The number of nested loop commands exceeds the maximum allowed. |

| Error Code | 3014 |
| --- | --- |
| Error Message | Track=??, Measure=??, Step=??, Main volume command is in the loop section. ﾄﾗｯｸ=??, ﾒｼﾞｬｰ=??, ｽﾃｯﾌﾟ=??, ﾙｰﾌﾟ区間中にﾒｲﾝﾎﾞﾘｭｰﾑがあります |
| Error Defined | Main volume command may not be specified while in the loop section. |

| Error Code | 3015 |
| --- | --- |
| Error Message | No ?? loop end command in track ??. ﾄﾗｯｸ=??でﾙｰﾌﾟｴﾝﾄﾞが ??個 足りません |
| Error Defined | ?? number of loop end commands are missing in track ??. |

| Error Code | 3016 |
| --- | --- |
| Error Message | No ?? loop start command in track ??. ﾄﾗｯｸ=??でﾙｰﾌﾟｽﾀｰﾄが ??個 足りません |
| Error Defined | ?? number of loop start commands are missing in track ??. |

| Error Code | 3017 |
| --- | --- |
| Error Message | Convert size over. XXXX, Over Size = ??h ｺﾝﾊﾞｰﾄｻｲｽﾞがｻｲｽﾞｵｰﾊﾞｰです XXXX, ｵｰﾊﾞｰｻｲｽﾞ = ??h |
| Error Defined | Group name [XXXX] sound data size is over the size limit after conversion. |

| Error Code | 3025 |
| --- | --- |
| Error Message | Division was not convertible. 分解能を変換出来ませんでした |
| Error Defined | If the division of the MIDI file is smaller than the normal division (48) used by the NEOGEO Pocket, the sound converter will attempt to convert to normal division. This is the error message displayed if this attempt fails. Please refer to 5.1 MIDI FILE FORMAT 2. Division. |

| Error Code | 3028 |
| --- | --- |
| Error Message | Tempo was changed into invalid value by conversion of division. 分解能の圧縮によりﾃﾝﾎﾟがｻﾎﾟｰﾄされていない値に変更されました |
| Error Defined | If the division of the MIDI file is smaller than the normal division (48) used by the NEOGEO Pocket, the sound converter will attempt to convert to normal division. In this conversion, tempo values are also changed and the validity of the value is checked. If error is encounter during this check, this error message is displayed. |

| Error Code | 3030 |
| --- | --- |
| Error Message | Track=??, Measure=??, Step=??, Invalid step[??]. ﾄﾗｯｸ=??, ﾒｼﾞｬｰ=??, ｽﾃｯﾌﾟ=??, 無効なｽﾃｯﾌﾟ[??]です |
| Error Defined | Track ??, Measure ??, and Step ?? has an invalid step. ?? is the invalid value set. |

| Error Code | 3031 |
| --- | --- |
| Error Message | Track=??, Measure=??, Step=??, Invalid step[??]. ﾄﾗｯｸ=??, ﾒｼﾞｬｰ=??, ｽﾃｯﾌﾟ=??, 無効なｹﾞｰﾄ[??]です |
| Error Defined | Track ??, Measure ??, and Step ?? has an invalid gate command. ?? is the invalid value. |

| Error Code | 4000 ~ 4005, 4013 |
| --- | --- |
| Error Message | A description error is in tone data.(program No.??) 音色ﾃﾞｰﾀに記述ｴﾗｰがあります(ﾌﾟﾛｸﾞﾗﾑNo.??) |
| Error Defined | There is an invalid description in the convert settings file and/or MIDI file Tone settings data. ?? is the invalid Tone program number. |

| Error Code | 4006, 4011 |
| --- | --- |
| Error Message | Too many tone data in mk file. mkﾌｧｲﾙの音色ﾃﾞｰﾀが最大数を超えています |
| Error Defined | The total number of Tone data registered in the convert settings file exceeds the 128 maximum. |

| Error Code | 4007 |
| --- | --- |
| Error Message | The same program No.?? is used in mk file. 同じﾌﾟﾛｸﾞﾗﾑNo.?? がmkﾌｧｲﾙにあります |
| Error Defined | There are Tone data with same program number in the convert settings file. |

| Error Code | 4008, 4010, 4012 |
| --- | --- |
| Error Message | Too many tone data. 音色ﾃﾞｰﾀが最大数を超えています |
| Error Defined | The number of Tone data registered in 1 Group exceeds the maximum 32. |

| Error Code | 4010 |
| --- | --- |
| Error Message | Invalid XXXX data[??](program No.??). XXXX はｻﾎﾟｰﾄされていない値[??]です(ﾌﾟﾛｸﾞﾗﾑNo.??) |
| Error Defined | XXX parameter set for Tone data with program number ?? has the invalid value ?? (XXXX is the Tone data parameter. ) |

| Error Code | 4015 |
| --- | --- |
| Error Message | No tone data of program No.?? in Midi XXXX file. ﾌﾟﾛｸﾞﾗﾑNo.?? の音色ﾃﾞｰﾀがありません( XXXX ) |
| Error Defined | Program number ?? used in the MIDI file has no Tone data registration. (XXXX is the MIDI file with the error.) |

| Error Code | 4016 |
| --- | --- |
| Error Message | Too many tone data of all groups. 全ｸﾞﾙｰﾌﾟの合計の音色ﾃﾞｰﾀが最大数を越えています |
| Error Defined | The number of Tone data in all of the Group exceeds the maximum 128. |

Warning Message Table

(Caution:  Error message with multiple code number has the same definitions.)

| Code Number | 3002 |
| --- | --- |
| Message | Invalid midi channel (5~16) is used. 無効なMidiﾁｬﾝﾈﾙ( 5～16 )を使用しています |
| Definition | Channel 1 ~ 4 are the MIDI channel used. Channels 5 ~ 16 are invalid channels. If data exist for channel 5 ~ 16, they are ignored and the file is converted. |

| Code Number | 3004 |
| --- | --- |
| Message | Track=??, Channel=??, Measure=??, Step=??, Invalid noise note No.[??]. ﾄﾗｯｸ=??, ﾁｬﾝﾈﾙ=??, ﾒｼﾞｬｰ=??, ｽﾃｯﾌﾟ=??, ﾉｲｽﾞのﾉｰﾄ番号[??]無効です |
| Definition | Track ??, Channel ??, Measure ?? and Step ?? of the MIDI file has an invalid noise note. The Sound Tool will use the invalid value for conversion. This produces incorrect interval. (Specifying incorrect noise note will not result in incorrect Driver operation. ?? is the invalid noise note number.) |

| Code Number | 3005 |
| --- | --- |
| Message | Track=??, Measure=??, Step=??, Invalid tempo change command. ﾄﾗｯｸ=??, ﾒｼﾞｬｰ=??, ｽﾃｯﾌﾟ=??, 無効なﾃﾝﾎﾟﾁｪﾝｼﾞｺﾏﾝﾄﾞです |
| Definition | Track ??, Measure ?? and Step ?? of the MIDI file uses an invalid tempo change command. The invalid tempo change command is ignored and the file is converted. |

| Code Number | 3006 |
| --- | --- |
| Message | Track=??, Measure=??, Step=??, Invalid beet command. ﾄﾗｯｸ=??, ﾒｼﾞｬｰ=??, ｽﾃｯﾌﾟ=??, 無効な拍子変更ｺﾏﾝﾄﾞです |
| Definition | Track ??, Measure ?? and Step ?? of the MIDI file uses incorrect rhythm. The invalid rhythm is ignored and the file is converted. |

| Code Number | 3018, 3019 |
| --- | --- |
| Message | Treck=??, Invalid tempo track of text event. ﾄﾗｯｸ=??, TEMPOﾄﾗｯｸ指定は無効です |
| Definition | There are multiple TEMPO tracks set. The initial TEMPO track is used and the others are ignored and the file is converted. |

| Code Number | 3024 |
| --- | --- |
| Message | Since it is finer than the minimum division, division is changed. 最小分解能より細かい分解能であるため,分解能を変換します |
| Definition | The MIDI file uses finer division than the normal division (48), the division has been changed to the normal division. If it is successful, warning message [3026] is displayed. If it was not successful, error message [3025] is displayed. |

| Code Number | 3026 |
| --- | --- |
| Message | Division was convertible. 分解能を変換出来ました |
| Definition | If division change to the normal division was successful, this message is displayed after the warning message above [3024]. |

| Code Number | 3027 |
| --- | --- |
| Message | Since the step and the gate are over maximum, a step, a gate, and tempo are changed into a half. ｽﾃｯﾌﾟ,ｹﾞｰﾄが最大値を越えているためｽﾃｯﾌﾟ,ｹﾞｰﾄ,ﾃﾝﾎﾟを半分に変換します |
| Definition | A MIDI file has the step and the gate which exceed the maximum 255. The step, gate and tempo will be changed to half of the original value and converted. If conversion fails, warning message [3029] is displayed. If the conversion is successful, no messages are displayed. |

| Code Number | 3029 |
| --- | --- |
| Message | Rest was omitted and changed. 余りを切り捨てて変換しました |
| Definition | Conversion as noted in the above message [3027] required the remainder to be omitted. |

| Code Number | 3032 |
| --- | --- |
| Message | Pich Bend Too Big Pich Bend Too Small ﾋﾟｯﾁﾍﾞﾝﾄﾞの値が大きすぎます。 ﾋﾟｯﾁﾍﾞﾝﾄﾞの値が小さすぎます。 |
| Definition | The value specified with Pitch Bend was too small or too large. The Sound Tool converts using the invalid value. For this reason, the pitch bend will not be correct when the music is played. (Invalid Pitch Bend specification will not result in incorrect operation by the Driver.) Please refer to section 2.4.2 for more information for Pitch Bend. |

| Code Number | 4009 |
| --- | --- |
| Message | The same program No.?? is used. 同じﾌﾟﾛｸﾞﾗﾑNo.?? があります |
| Definition | Tone data with the same program number was detected. Because the same tone is specified, it is ignored. |

### Sound Tool Error Messages

Error messages of the Sound Tool are listed below.  (Note:  Even if multiple error codes are listed, the definitions are the same.)

| Code Number | 9001 9019 9020 9021 9022 9023 9024 9025 9026 9027 9028 9029 9030 9031 9036 9037 9038 9043 9044 9046 9047 9051 9052 9500 |
| --- | --- |
| Message | System Error |
| Definition | Sound Tool system error. In normal operation, this error should not occur. If this message is displayed, there is an abnormality in the operation. Please save the Project and exit the Sound Tool. |

| Code Number | 9002 |
| --- | --- |
| Message | File Error |
| Definition | The file name should be 8 characters, 3 characters for extension. Or the file does not exist. |

| Code Number | 9003 |
| --- | --- |
| Message | File Open Error [xxxx] |
| Definition | File XXXX could not be opened. |

| Code Number | 9004 |
| --- | --- |
| Message | Driver Read Error |
| Definition | Failed to obtain Sound Driver information. |

| Code Number | 9005 |
| --- | --- |
| Message | Library Read Error |
| Definition | Sound Library information was not obtained. |

| Code Number | 9006 |
| --- | --- |
| Message | Cnt't Make File [xxxx] |
| Definition | Could not create file XXXX. |

| Code Number | 9007 |
| --- | --- |
| Message | File Write Error |
| Definition | File write error. |

| Code Number | 9008 |
| --- | --- |
| Message | Driver File Name Write Error [xxxx] |
| Definition | Sound Driver file XXX could not be written when the Project file was saved. |

| Code Number | 9009 |
| --- | --- |
| Message | Driver Data Write Error |
| Definition | Sound Driver could not be written when the Project file was saved. |

| Code Number | 9010 |
| --- | --- |
| Message | Library File Name Write Error [xxxx] |
| Definition | Sound Library file XXX could not be written when saving the Project file. |
| Code Number | 9011 |
| Message | Library Data Write Error |
| Definition | Sound Library could not be written when saving the Project file. |

| Code Number | 9012 |
| --- | --- |
| Message | Host File Name Write Error [xxxx] |
| Definition | Host command file XXXX could not be written when saving the Project file. |

| Code Number | 9013 |
| --- | --- |
| Message | Host Data Write Error |
| Definition | Host Command file could not be written when saving the Project file. |

| Code Number | 9014 |
| --- | --- |
| Message | Tone File Name Write Error [xxxx] |
| Definition | Tone data file XXXX could not be written when saving the Project file. |

| Code Number | 9015 |
| --- | --- |
| Message | Tone Data Write Error |
| Definition | Tone data file could not be written when saving the Project file. |

| Code Number | 9016 |
| --- | --- |
| Message | Midi Data Error |
| Definition | Music data could not be written when saving the Project file. |

| Code Number | 9017 |
| --- | --- |
| Message | Group Cnt Write Error |
| Definition | Number of Groups could not be written when saving the Project file. |

| Code Number | 9018 |
| --- | --- |
| Message | Group Name Write Error [xxxx] |
| Definition | Group name XXX could not be written when saving the Project file. |

| Code Number | 9032 |
| --- | --- |
| Message | Driver No Entry. Please Entry Driver. |
| Definition | The Driver is not registered. |

| Code Number | 9033 |
| --- | --- |
| Message | Midi Entry Error. |
| Definition | Music data is not registered. |

| Code Number | 9034 |
| --- | --- |
| Message | Midi File No Found. |
| Definition | MIDI file does not exist. |

| Code Number | 9035 |
| --- | --- |
| Message | Midi Image No Found. |
| Definition | There is no convert image for MIDI file. |

| Code Number | 9039 |
| --- | --- |
| Message | Driver No Entry. |
| Definition | The driver is not registered. |

| Code Number | 9040 |
| --- | --- |
| Message | Convert Size Over |
| Definition | Total data size after conversion is too large. |

| Code Number | 9041 |
| --- | --- |
| Message | Invalid Midi File Name Error [XXXX] |
| Definition | MIDI file XXXX can not be registered to the Project. |

| Code Number | 9042 |
| --- | --- |
| Message | Invalid Midi Type Error [XXXX] |
| Definition | MIDI file XXXX can not be registered since it is already used in another Group as BGM or SE. |

| Code Number | 9045 |
| --- | --- |
| Message | Converted Size is Zero [XXXX] |
| Definition | MIDI file XXXX can not be registered since the converted size is 0. |

| Code Number | 9048 |
| --- | --- |
| Message | Asm File Read Error [XXXX] |
| Definition | ASM file XXXX could not be read in. |

| Code Number | 9049 |
| --- | --- |
| Message | Inc File Read Error [XXXX] |
| Definition | INC file XXXX could not be read in. |

| Code Number | 9050 |
| --- | --- |
| Message | Non Select Driver |
| Definition | Sound Driver is not registered. |

| Code Number | 9051 |
| --- | --- |
| Message | File Open Error |
| Definition | File open error. |

| Code Number | 9053 |
| --- | --- |
| Message | Non Group Error |
| Definition | Group is not registered. |

| Code Number | 9054 |
| --- | --- |
| Message | Invalid Driver |
| Definition | Invalid Sound Driver. |

| Code Number | 9055 |
| --- | --- |
| Message | Invalid Sound Library |
| Definition | Invalid Sound Library. |

| Code Number | 9502 |
| --- | --- |
| Message | Check Sum Error |
| Definition | Check sum error. |

| Code Number | 9503 |
| --- | --- |
| Message | Fread Error |
| Definition | File read error. |

| Code Number | 9504 |
| --- | --- |
| Message | ID Error |
| Definition | File data is corrupt. |

| Code Number | 9505 9520 |
| --- | --- |
| Message | Driver Name Read Error |
| Definition | Sound Driver name was not obtained when reading in the Project file. |

| Code Number | 9506 |
| --- | --- |
| Message | Driver Data Read Error |
| Definition | Sound Driver data read in error when the Project file was read in. |

| Code Number | 9507 9521 |
| --- | --- |
| Message | Driver Entry Error |
| Definition | Sound Driver registration failed when the Project file was read in. |

| Code Number | 9508 |
| --- | --- |
| Message | Library Name Read Error |
| Definition | Sound Library name was not obtained when reading in the Project file. |

| Code Number | 9509 |
| --- | --- |
| Message | Library Data Read Error |
| Definition | Sound Library data read in error when the Project file was read in. |

| Code Number | 9510 |
| --- | --- |
| Message | Library Entry Error |
| Definition | Sound Library registration failed when the Project file was read in. |

| Code Number | 9511 |
| --- | --- |
| Message | Host Name Read Error |
| Definition | Host Command data name was not obtained when reading in the Project file. |

| Code Number | 9512 |
| --- | --- |
| Message | Host Data Read Error |
| Definition | Host Command data read in error when the Project file was read in. |

| Code Number | 9513 |
| --- | --- |
| Message | Host Entry Error |
| Definition | Host Command data registration failed when the Project file was read in. |

| Code Number | 9514 |
| --- | --- |
| Message | Tone Name Read Error |
| Definition | Tone data name was not obtained when reading in the Project file. |

| Code Number | 9515 |
| --- | --- |
| Message | Tone Data Read Error |
| Definition | Tone data read in error when the Project file was read in. |

| Code Number | 9516 |
| --- | --- |
| Message | Tone Entry Error |
| Definition | Tone data registration failed when the Project file was read in. |

| Code Number | 9517 |
| --- | --- |
| Message | Midi Name Read Error |
| Definition | MIDI file name was not obtained when reading in the Project file. |

| Code Number | 9518 |
| --- | --- |
| Message | Midi Data Read Error |
| Definition | MIDI data read in error when the Project file was read in. |

| Code Number | 9519 |
| --- | --- |
| Message | Midi Entry Error |
| Definition | MIDI registration failed when the Project file was read in. |

| Code Number | 9522 |
| --- | --- |
| Message | Group Name Read Error |
| Definition | Group name was not obtained when reading in the Project file. |

| Code Number | 9523 |
| --- | --- |
| Message | Group Enter Error |
| Definition | Group registration failed when the Project file was read in. |

| Code Number | 9524 |
| --- | --- |
| Message | Midi File (BGM) Name Read Error |
| Definition | MIDI file (BGM) name was not obtained when reading in the Project file. |

| Code Number | 9525 |
| --- | --- |
| Message | Midi File (BGM) Entry Error |
| Definition | MIDI file (BGM) registration failed when the Project file was read in. |

| Code Number | 9526 |
| --- | --- |
| Message | Midi File (SE) Name Read Error |
| Definition | MIDI file (SE) name was not obtained when reading in the Project file. |

| Code Number | 9527 |
| --- | --- |
| Message | Midi File (SE) Entry Error |
| Definition | MIDI file (SE) registration failed when the Project file was read in. |

# SOUND TOOL MIDI FILE FORMAT

Because there are limitations in the NEOGEO Pocket Driver, it is not possible to play all of the functionality embedded in the MIDI data format on the NEOGEO Pocket.  Please follow the guidelines given below to ensure the MIDI file can be converted correctly by the Sound Tool.  MIDI format for use by the Sound Tool is explained below.

## MIDI FILE FROMAT

MIDI Format Type

Standard MIDI format type 1 only.

Division

NEOGEO Pocket uses division of 48.  If divisions smaller (finer) is specified for a MIDI file (ex: division of 480), the system will check if it is changeable to division of 48 and if so, it is converted.  If conversion fails, error message [3025] is displayed.  Please determine if the MIDI file in question has MIDI commands for division of 48 and the total step value, gate value is convertible.

The Sound Tool will also determine if the division could be smaller than 48, and if so, the value is changed to a smaller value and converted.  For example, if all of the step values and gate values can be divided by 2 in the MIDI file, the division is set to 24 and the step value and gate value is divided by 2 and data converted.

This operation is done to raise the compression ratio.  This change will not affect the music when it is played.

Delta Time

Only MIDI clock time method is allowed.  Actual time management time code method may not be used.

MIDI Channel and Track

MIDI channels 1 ~ 4 may be used.  The MIDI channels 1 ~ 4 correspond to the NEOGEO Pocket track 1 ~ 4.  In the NEOGEO Pocket system, channel 4 is the noise channel.  There are no restrictions on use of the number of tracks in the MIDI data (It is possible to use different tracks with the same MIDI channels).  MIDI tempo track is restricted to track 1.  Please refer to the next section for tempo track information.

Tempo Track

Tempo track is used to set the tempo and the rhythm (Done in meta-event.  Setting tempo and rhythm is explained later.).  The Sound Tool uses the tempo and rhythm from the tempo track to create music.  Tempo and rhythm information in other tracks is ignored.  And a warning message will be displayed.  The tempo track for the NEOGEO Pocket is the first track of the MIDI.

Data Size

There are no restrictions on the MIDI file size before conversion.  Because the converted MIDI file music data must reside with the Sound Driver, Tone data, Host Command data, and other music in the same Group in the Z80 RAM; the total must not exceed 4 KB.  The songs must be composed accordingly.

No Chord and Last Arrival Priority

It is not possible to produce a chord with the NEOGEO Pocket using 1 channel.  Slur in MIDI (chord type tone generation) is changed to the previous sound “key off” and the next sound “key on.”  This is because in the NEOGEO Pocket, the last request arrived will have the priority to generate its tone.

Step Value Until the Music is played

In the NEOGEO Pocket Sound Tool, the first note command is designated as the starting point for each of the music (BGM, SE) play position.  The first note command is defined as the first note command operation of the MIDI channel 1 ~ 4 time wise.

For this reason, the Sound Tool sets the step value to 0 for all songs until the note command is operated and the music data is converted.  If the play start is to be delayed on purpose, please use the note OFF command to create “rest” notes.

Mac Binary MIDI File Format

It is possible to use the Mac Binary MIDI file format.

MIDI File Name

The following names may not be used for MIDI file names in the Sound Tool.

STOP.MID

EI.MID

DI.MID

ALL_STOP.MID

If MIDI files with these names are to be used, please change the file name before use.

MIDI Sequencer Software

There are possibility that some MIDI sequencer software may not meet the above requirements.  The software recommend for use to create MIDI data for the NEOGEO Sound Tool is listed below.

Recomposer	Windows version

Cake Walk	Windows version

## MIDI COMMANDS VALID FOR CONVERSION

A. Note ON, Note OFF

Note numbers usable are 43 ~ 120 (Same for noise note number.).  Velocity has 128 levels (0 ~ 127) and is converted with the following equation to 16 levels (0 ~ 15).

Velocity = MIDI Velocity  16

B. Pitch Bend

16384 levels (0 ~ 16383) is converted to 126 levels (1~ 126) with the following equation.

Pitch Bend = MIDI Pitch Bend  128

If the Pitch Bend value is 0 or 127, the Pitch Bend value can not be used.  Pitch Bend value of 127 would raise the music interval 1 octave, and a value of 0 will lower the music interval 1 octave.  But because these values can not be used, values 126 (raise approximately 1 octave) and 1 (lower approximately 1 octave) are used.

Also it is not possible to lower 1 octave with the lowest value or to raise 1 octave with the highest value using Pitch Bend.  Because Pitch Bend rises or lowers approximately 1 octave when set to maximum value, it is necessary to keep the Pitch Bend value to not exceed the lowest and highest interval.

C. Pan Pot (Potentiometer)

128 levels (0 ~ 127) is converted to 16 levels (0 ~ 15) with the following equation.  It is not used for SE.

Pan Pot = MIDI Pan Pot  8

D. Expression

128 levels (0 ~ 127) is converted to 16 levels (0 ~ 15) with the following equation.  It is not used for SE.

Expression = MIDI Expression  8

E. Program Change

Can use all of the numbers from 0 to 127.  The restriction is that maximum of 32 may be used in 1 Group.

G. Main Volume

Can use all of the values from 0 to 127.  It is not used for SE.  Also, Main Volume command may not be used during loops.

H. NRPN and Data Entry

To supplement functions not covered in the standard MIDI format, the NEOGEO Pocket uses NRPN command and Data Entry command.  The commands currently accessible are the loop start and loop end commands.  Loop start and loop end commands are used in pairs.  The area specified by the loop start and loop end commands are looped for the designated times and the music is played.

First, specify the command type (loop start, loop end) with the NRPN command.  Next, set the data entry command for this NRPN command.  At the point when the data entry command is set, the command type set with the NRPN command becomes valid.

As an exception, NRPN commands which do not require setting data, become valid soon as NRPN command is set.

Data value which may be specified for use for data entry and values of NRPN for each commands are shown below.

|  | NRPN | Data Entry Value |
| --- | --- | --- |
| Loop Start | 20 | 0 is an infinite loop, 1 ~ 31 is the number of loops |
| Loop End | 30 | None |

<Caution 1>

During MIDI file conversion, the Sound Tool takes each of the songs (BGM, SE) and searches all track sequences for MIDI command groups which may be looped.  If these areas are found the Sound Tool inserts loop start and end commands for compression reasons.  Please be careful not to set NRPN and data entry commands in areas where a loop may be generated.

<Caution 2>

There are MIDI sequencer software which interchange commands with the same clock.  If this is the case, the NRPN and data entry may be switched in position to create problems during conversions.  To ensure conversion, please use MIDI commands with “timing like” attributes.

During MIDI file conversion, the Sound Tool sets the total steps to 0 before the start play of music.  This may be used to set commands in the correct order.

Performer has been found to interchange commands of same clock.  Please be careful setting loop start command if Performer is to be used.

I. MIDI Command Priority for Same Clock Timing

The commands explained above have the following priority if they occur with the same clock timing.  If MIDI commands with the same priority exist, then they are handled as a standard MIDI file command order.

MIDI Commands With Higher Priority

- NRPN 20 (Loop Start), Data Entry

- Program Change

- Expression

- Pan Pot

- Pitch Change

- Note Command

- NRPN 30 (Loop End)

- Main Volume

MIDI Commands With Lower Priority

## META EVENTS NECESSARY FOR CONVERSION

1. Tempo

For BGM, values from 10 to 255 are usable.  For SE, it is not possible to change the tempo during play.  The SE uses the tempo set by the Driver (120).  When the SE is converted, the tempo value set in the sequencer software is ignored.

2. Rhythm

All rhythms are usable.  If the step value exceeds 255 because of the rhythm used (ex. 6/4 rhythm, total number of notes over 1 measure, the step value is 288), the step value, gate value and tempo value will all be halved and converted.  If the value is odd, the remainder is dropped and converted.  A warning message will appear.

3. End of Track

End of track is a MIDI command which designates end for each of the tracks.  Please enter end of track command at the end of track.  Most of the MIDI sequencer software automatically create this command at the end of tracks.

## MIDI FILE FORMAT DIFFERENCE BGM AND SE

### MIDI Channel

BGM and SE both can use 4 channels.  Because the NEOGEO Pocket can only produce 1 sound per channel, channel used for SE is interrupted and the SE is played.  For example, BGM which uses channels 1 through 4 is interrupted with the SE which uses channel 3, the BGM playing in channel 3 is interrupted and the SE is played.  After the SE is played, the BGM channel interrupted begin playing the BGM again.

If two SE are played at the same time, the SE registered first in the Group (closest to the top on screen) has the higher priority.

### MIDI Command

BGM may use all of the MIDI commands listed above.  The SE may only use Main Volume, Expression and Pan Pot.

### Meta Event

BGM may use all of the meta event listed above.  Because the SE uses the fixed tempo (120), it can not use commands to set the tempo.  Other meta commands are accessible by SE.

# SOUND LIBRARY

## SOUND LIBRARY OUTLINE

The Sound Library is a library to provide two functions (data transfer and play function).

## DATA TRANSFER LIBRARY

As noted in 1. NEOGEO Pocket Sound Structure, the NEOGEO Pocket Sound is comprised of data and program (Driver) which operate in 4 KB of memory.

Because of this limited memory, data transfer library allows transfer of data as needed to play more than few music (scores).

## PLAY LIBRARY

This is the program which hands the sound code (commands to determine which music, play, step, etc.) from the main program (900H) to the Driver.

## TRANSFER LIBRARY USE

Figure 16. Transfer Library Data Flow

Figure 16 shows the data flow on the NEOGEO Pocket.  If data is created on a situation basis in Group units, it is possible to transfer the Group necessary to the sound memory with the host program.  Following steps may be taken to transfer data.

### Creating Files Necessary for Data Transfer Using the Converter

The following files are necessary for data transfer.  Please refer to 3.9 Target File section for the method to output the files.

.ASM file – Transfer data file created for the transfer program.

.INC file – Sound code registration file to specify music and group by the game program.

### Assembling Data File Used by the Transfer Program

To assemble the Target file (.ASM) outputted by the Sound Tool, ASM900.EXE file is required.  Please execute the file under DOS prompt.  Please refer to Toshiba 900H Assembler Manual for use of ASM900.EXE.

>ASM900 -$ -l -J -g -e File_Name.ERR File_Name.ASM

This command assembles the code and creates the link file (.REL).  The link file (.REL) created becomes the sound data portion of the transfer program.

### Linking the Link File Created by Assembling

Please follow the method below to link the link file (.REL) and Sound Library (SLIB.REL) and the game program.

<Caution>

The relocate address of the Sound Data and Sound Library (.REL and SLIB.REL) are assembled with the section name sndprog (for the program) and sndwork (for the work meory).  Please set the address to sndprog section and sndwork section for game programming.

Please refer to Toshiba 900H Assembler manual for further information on linking.

Please refer to \HOST\SAMPLE\SND_TEST.CMD of the Sound Program disk for an example of linking.

### Calling Transfer Program as Required

Call the Sound_List_Dt_Send function, listed in the Function List below, to transfer data.  Please refer to \HOST\SAMPLE\SND_TEST.ASM of the Sound Program disk for examples.

## FUNCTION LIST

Sound Library functions are explained below.

Sound_Init

<Definition> Sound Library initialization function.

<Usage> Please call the function once at Power ON.

<Registers Destroyed>  xde, xhl, bc, xix

[Caution] This function does not transfer Sound Data.

Sound_Buf_Init

<Definition> Sound Code Buffer initialization function.

<Usage> This function clears the Sound Code if Sound Code is in the buffer.

<Registers Destroyed>  a

[Caution] Because memory for hand shake with the Sound Driver is also initialized, please do not use this function when the reset command for the Sound Code is used (This function is called inside the reset command).  Usage of this function before the reset command is allowed.

Put_Sound_Buf

<Definition> This is the Sound Code transmission function.  This function generates music play start, play stop, host commands, etc. to the Sound Driver.

<Usage> Sound Code specifications are all set in the .INC file after conversion.  The game program should specify the Sound Codes in this .INC file to transmit Sound Codes.

<Registers Destroyed> None

[in]	wa : Sound Code

[out]	CY : OK if the value is 1

[Caution] Please call the Sound_List_Dt_Send function before calling this function.

Sound_List_Dt_Send

<Definition> Sound Data transfer function.  Transfers Sound Data in Group unit.

<Usage> The Sound Data is specified with the number of the Sound Data List Table and the data is transferred.  List Table numbers are set in the .INC file after conversion.  Please specify the List Table number in this .INC file to transfer Sound Data in the game program.

<Registers Destroyed> xwa

[in]	xwa : List Table number

[out]	CY : OK if the value is 0, error if the value is 1

[Caution] Sound_Init function does not call this function (when there are no Sound Data).  Please call this function once before playing the music.

Sound_Periodic_Func

<Definition> Function to be called periodically.

<Usage> This function checks to see if Sound Code is present in the Sound Code buffer.  Please call this function periodically.

<Registers Destroyed> None

[out]	CY : OK if the value is 1

[Caution] Please avoid multiple calls of this function (interrupt inside an interrupt).